3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102604

# Elbonia, April 12 – 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2673 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3B, Temporal Validity Procedure including Temporal Validity Condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-17, Time Synchornization Service Exposure is not fully described in TS23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Validity Condition to Time Synchronization parameters - Add Time Synchornization Service Activation/Deactivation/Modification Procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Time Synchornization Service is not clear in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.11, F.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

#### 4.15.6.11	Time Synchronization parameters

As described in TS 23.501 [2] clause 5.27.1.z, an AF may learn 5GS capabilities to support time synchronization based on parameters exposed by the NEF (Table 4.15.6.11-1) and AF may request to perform time synchronization distribution configuration by providing input parameters towards NEF.

Table 4.15.6.11-1: Description of Time Synchronization capabilities exposed NEF -> AF

| Time Synchronization Parameter | Description |
| --- | --- |
| Time synchronization distribution method | Identifies the time synchronization distribution methods supported by 5GS. Allowed values: IEEE Std 1588 [76] operation (i.e. as a Boundary Clock, peer-to-peer Transparent Clock, or end-to-end Transparent Clock) and transport protocol (Ethernet, UDP over IPv4, or UDP over IPv6), IEEE Std 802.1AS [75] operation, or Access Stratum-based 5G clock sync. |
| (g)PTP grandmaster capable | Indicates separately whether NW-TT supports acting as a gPTP or PTP PTP grandmaster. |
| 5G AS Clock quality | Indicates the clock quality supported in case of Access Stratum-based 5G clock sync |

Table 4.15.6.11-2: Description of Time Synchronization parameters input from AF -> NEF

| Time Synchronization Parameter | Description |
| --- | --- |
| Time synchronization distribution method | Identifies the time synchronization distribution method requested by AF. Allowed values: IEEE Std 1588 [76] operation (i.e. as a Boundary Clock, peer-to-peer Transparent Clock, or end-to-end Transparent Clock) and transport protocol (Ethernet, UDP over IPv4, or UDP over IPv6), IEEE Std 802.1AS [75] operation, or Access Stratum-based 5G clock sync. |
| Grandmaster enabled | Indicates whether AF requests 5GS to act as a grandmaster for PTP or gPTP (depending on the requested Time synchronization distribution method). This is applicable for IEEE Std 1588 [76] or IEEE Std 802.1AS [75] operation. |
| Gandmaster priority | Indicates a priority used as defaultDS.priority1 when generating Announce message when 5GS acts as (g)PTP GM. |
| Time Domain | As defined in IEEE Std 1588 [76] |
| Spatial Validity Condition | Indicateds the UE(s) location |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the TimeSync service is active. |

The AF may use Nnef_ParameterProvision_Get operation to learn 5GS capabilities as in Table 4.15.6.11.1 to support time synchronization service.

Editor's note:	5G AS Clock quality and Access Stratum-based 5G clock sync is FFS.

Editor's note:	How a clock accuracy parameter as well as other parameters for time synchronization can be supported is FFS.

***** END OF CHANGES *****