3GPP TSG-SA WG2 Meeting #144-e	S2-2102605r01

Elbonia, April 12 – 16, 2021

# Title:	[Draft] Reply LS on NAS Non delivery for RRC_INACTIVE state

# Response to:	Reply LS on NAS Non delivery for RRC Inactive state (S2-2102083/ R3-207170)

# Release:	3GPP Rel-15/Rel-16

# Work Item:	NR_newRAT-Core

Source:	SA2

To:	RAN3

Cc:	CT4

Contact Person:	Yunjing Hou

E-mail Address:	houyunjing (at) catt (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks RAN3 about the LS on NAS Non delivery for RRC Inactive state (R3-207170). SA2 discussed the questions and agreed the following answers.

Question 1: For a UE in RRC_INACTIVE state, is there any use case for AMF to piggyback a non-PDU session related NAS PDU in PDU SESSION RESOURE SETUP REQUEST?

Answer 1: The scenario described in TS 23.502 related to Question 1 is as follows:

“If the Service Request procedure is triggered by the Network (as described in clause 4.2.3.3) while the UE is in CM-CONNECTED state, only N2 SM information received from SMF is included in the N2 Request.”

Based on the description above, AMF will not piggyback a non-PDU session related NAS PDU in PDU SESSION RESOURCE SETUP REQUEST.

Question 2: Which solution is preferred to inform AMF the non-delivery of the non-PDU session related NAS-PDU in the "Initial Context Setup Request"?

- Solution 1: Use NAS NON DELIVERY INDICATION message to indicate the failure of the NAS delivery.

- Solution 2: Use the "Initial Context Setup failure” to implicitly indicate the failure of the NAS delivery.

Answer 2: Normally SA2 does not specify error handling, but exceptions can be made if there are system wide impact reasons to do so. In this case, there is a requirement for the NG-RAN to inform the AMF of the NAS message delivery failure in clause 5.3.3.2.5 of TS23.501 as,

“If the RAN paging procedure, as defined in TS 38.300 [27], is not successful in establishing contact with the UE the procedure shall be handled by the network as follows:

-	If NG-RAN has at least one pending NAS PDU for transmission, the RAN node shall initiate the AN Release procedure (see TS 23.502 [3], clause 4.2.6,) to move the UE CM state in the AMF to CM-IDLE state and indicate to the AMF the NAS non-delivery.”

SA2 would like to point out that the expression "indicate to the AMF the NAS non-delivery" does not imply that the AMF shall be informed by means of "NAS Non Delivery Indication" procedure.

2. Actions:

To SA2 group:

ACTION: SA2 requests RAN3 and CT4 to take the above information into account.

3. Date of Next SA2 Meetings:

SA2#145-e	17 – 28 May 2021	Electronic