SA WG2 Meeting #144E e-meeting	S2-2102680r05

Elbonia, 12 – 16 April 2021	(was S2-210xxxx)

Source:	Qualcomm Incorporated

Title:	Issues with DNS based EAS (re-)discovery: solution

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract: During SA2#143E, S2-2100745 proposed considerations on the User/Application/ HLOS overwriting the DNS settings provided by the MNO to the UE. This issue, if not solved, can potentially invalidate several optimizations introduced in the Rel-17 eEDGE_5GC work. This paper proposes a way forward.

# 1.	Discussion

As discussed in S2-2102679, it is essential to address the issue of application/user interface/HLOS overriding the DNS settings provided by the MNO to the UE.

# 2. 	Text proposal

It is proposed to agree the following changes vs. TS 23.548:

>>>>BEGINNING OF CHANGES<<<<

## 6.2	EAS Discovery and Re-discovery

### 6.2.1	General

Editor's note:	This clause describes general parts including e.g. privacy considerations, which DNS properties that are enabling DNS based Edge AS Discovery, recommendations/limitations for cases that OS/user overrides DNS setting.

In order for a UE to use the enhancements for EAS discovery and re-discovery it is important that the DNS Query originated by the UE is sent to the DNS server/resolver or EASDF indicated by the SMF.

Based on the type of deployment used by the operator:

1.	Edge Computing service is provided over a PDU sessions used solely for Edge Computing. All the DNS queries sent over such PDU sessions should be directed to the DNS server/resolver or EASDF provided by the SMF.

2.	Edge Computing service is provided over  PDU sessions that are shared with other services. In this case it is assumed that applications agreed with the HPLMN that the DNS queries are sent to the  DNS server/resolver or EASDF indicated by the SMF.

When indicating to the UE the address of the DNS server/resolver or of the EASDF, the SMF also indicates that such address is associated to Edge supporting features. Once received the UE shall forward such indication to the application layer and HLOS.

>>>>END OF CHANGES<<<<