SA WG2 Meeting #146e (e-meeting)	S2-2106196r07-Qualcomm

August 16 - August 27, 2021, Electronic, Elbonia 	(revision of S2-2103940, 4662)

- Source:	Tencent, Oracle, Spreadtrum, NEC

- Title:	New SID on 5G System Enhancement on High Data Rate and Low Latency Services

- Document for:	Discussion and information

Agenda Item:

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study on 5G System Enhancement on High Data Rate and Low Latency Services

## Acronym: FS_5G_HDRLL

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Release 18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No |  |  |  |  |  |
| Don't know | X | X |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 840030 | Network Controlled Interactive Service (NCIS) | Release 17 Normative work to define the high data rate and low latency service and its QoS KPI. |
| 870013 | Study on Typical Traffic characteristics for XR Services and other Media (FS_XRTraffic) | Release 17 Study the AR/VR service characteristics and QoS Requirements. |
| 840019 | 5G System Enhancement for Advanced Interactive Services (5G_AIS) | Release 17 Normative work on defining new 5QI and providing TSC Assistance Information from AF to the 5GC. |
| 910059 | Support of Enhanced Industrial IIOT (IIoT) | Release 17 Time synchronization and QoS enhancement for the IIOT |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

## 3	Justification

HDRLL (High Data Rate and Low Latency) services are defined in SA1 specification (TS 22.261 and TS 22.263) and some QoS enhancements on HDRLL in Release 17 have been defined in Stage 2 specifications, e.g., TS 23.501/2/3.

HDRLL services will introduce a lot of new QoS challenges for the 5GS. Based on the observation of several types of HDRLL services, e.g., for the cloud gaming, cloud AR (Augmented Reality)/VR (Virtual Reality), tele-operated driving, real-time digital twin, etc., the users still experience some problems like frozen pictures and voice loss while the UE quickly moves, lag when radio connection is weak and congestion when the number of users or HDRLL applications increase within one cell.

The GBR QoS Flow and Guaranteed Bit Rate (which are used to guarantee the QoS requirements), QoS Notification Control and Alternative QoS Profile were introduced in Rel-15/16. New 5QI(s) and service pattern information, e.g., periodicity information conveyed to RAN reusing TSCAI as per IIoT work item, were defined in Rel-17 within the context of the 5G_AIS work item in order to satisfy QoS requirements of HDRLL services. However, these mechanisms are still not sufficient for HDRLL services to handle the very high data rate with low latency under network dynamics related to the observed user experience problems.

More specifically, the assumption in SA4 (see S4-210614, “FS_XRTraffic: Permanent document, v0.6.0” clauses 8.3.2 and 9.1) is that XR applications impose requirements in terms of Application Data Units (ADUs), rather than in terms of single packets/PDUs. An ADU represents a typically large sequence of PDUs that includes, e.g., all the necessary information to reconstruct a video frame. An ADU is useful at the receiver only if all its packets have been received by the receiver (or a continuous sequence of packets up to the first missing packet can be used).

While, as explained above, XR applications benefit from a ADU handling, on the other hand 5GS still handles QoS in terms of packets (see, e.g., PER = Packet Error Rate and PDB = Packet Delay Budget). Consequently, mechanisms need to be designed to determine which Packet Data Units (PDUs) belong to the same ADU and therefore apply accordingly the same QoS treatment and, if RAN is not able to deliver one or few of the PDUs belonging to the same ADU, to be able to move to try to deliver PDUs from the next ADU and so on. In addition, as shown in the tdoc "[FS_XRTraffic] SA4 and RAN1 Collaboration Proposal(S4-210612)", additional information associated to ADUs may be beneficial in optimizing content delivery. In order to do that, SA2 needs to consult with SA4 about possible mechanisms that 5GS can use to identify the PDUs belonging to an ADU, the size of the ADU, etc.

HDRLL services, e.g., cloud gaming, are characterized by heavy burst high peak-to-average ratio in terms of data rate. AF may have the capability to adjust its data rate in a big range which brings a good chance for adaptation. However, The AF may have limited capability or information to detect and predict the quick change of the data rate in the 5G network. Thus, the 5GS needs to provide mechanisms to help the AF to quickly adapt to the dynamic data rate fluctuation.

If there is HDRLL media data congestion in the RAN, the 5GS can provide new network information to help AF to do more application/transport layer adaptation so that the 5GS can quickly recover to normal state from congestion state for the HDRLL service.

If there is HDRLL media data congestion in the RAN and the radio bit rate is degrade, the RAN may decide to drop packet(s). For the HDRLL media streaming, if any media packet is dropped or any of its IP packet is dropped, the subsequent packets become useless. The 5GS needs to prevent and reduce the resource wasting to transport these useless packets. And it is also important for 5GS to help the AF to reduce the following packets as useless as much as possible. It is beneficial for 5GS to be able to perform eligible packet dropping for the less important packets when packet dropping is deemed.

Another potential issue is that large bursts of traffic may be sent towards the network by different application clients at approximately the same time. This may lead to an excessive amount of resources requested for a specific point in time within the PDB time frame at the RAN node. In order to address this problem, mechanisms that allow to redistribute the downlink traffic load within the PDB time frame should be studied.

In the SA1 specification (TS 22.261), the max allowed end-to-end latency has been defined for different HDRLL services, and the end-to-end latency is the total PDB of  UL and DL direction. Though new 5QIs have been defined for the HDRLL in Release 17, it is still needed to study how to enhance the QoS to support the low round trip time (UL+DL) for the HDRLL traffic, e.g., new 5QI with a low and different UL and DL PDB.

## 4	Objective

The study item aims at identify the system architecture aspects related to better support advanced media services, e.g., High Data Rate Low Latency (HDRLL) services, AR/VR/XR services, and tactile/multi-modality communication services.

Objective1: Enhancements for supporting multi-modality service:

-	Study whether and how to enable delivery of related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) to the user at a similar time.

Objective 2: Enhancements of network exposure to support interaction between 5GS and application：

-	Study whether and how interaction between AF and 5GS is needed  for application synchronization and QoS control coordination between multiple QoS flows per UE or among multiple UEs.

NOTE: The above objectives related with tactile and multi-modality service may be updated based on SA1 outputs on TACMM.

-	Study exposure of 5GS QoS information, e.g., QoS capabilities and network conditions to the Application to enable quick codec/rate adaptation.

-	Study whether and how QoS Notification enhancements are needed to help the 5GS to quickly recover to normal state from congestion state for the HDRLL service, e.g., via network information provided to AF.

-	Study whether and how to enable DL traffic redistribution within the PDB time frame to improve RAN resource scheduling.

NOTE: Parameters for exposure may coordinate with RAN and SA4.

Objective 3: Study whether and how the following Enhancements for XR service and media service transmission are needed:

-	Study the traffic characteristics of media service enabling improved network resources usage and QoE.

-	Enhance QoS framework to take media units (e.g., video/audio frame, Application Data Unit, control information), where media units consist of PDUs that have the same QoS requirements and that may, e.g. , belong to the same frame in the HDRLL application.

-	Support QoS control based on media unit granularity (e.g. video frame/tile granularity)

-	Support differentiated QoS handling considering importance of packet of media units.

-	Study architecture enhancement/QoS adjustment to support trade-off among throughput and latency, with reliability and device battery life.

NOTE: Coordination with RAN WGs may be needed.

-	Support uplink-downlink transmission coordination to meet RTT (Round-Trip Time) requirements.

-	Enhance the 5GS to minimize the media packet drop and its impact, e.g., eligible drop packet, reduce the resource wasting, reduce the useless packets.

-	Potential policy enhancements needed to support the above and to minimize the jitter.

-	Enhance the QoS to support the low round trip time for the HDRLL traffic.

Objective 4: Study potential enhancements of Mobility and power management considering traffic pattern of media services:

-	Support handover enhancement considering the traffic pattern of media services to minimizing service disruption.

-	Power saving enhancement e.g. whether and how to enhance CDRX , considering XR/media traffic pattern.

Objective 5: Whether and how to support network slice for mobile media:

-	Potentially define a network slice type supporting mobile media transmission.

The time for this study item is about 18TUs.

and that may, e.g. , belong to the same frame in the HDRLL application.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 23.xxx | Study on 5G System (5GS) Enhancement on High Data Rate and Low Latency (HDRLL) Services | TBD | TBD |  |

## 6	Work item Rapporteur(s)

TBD

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

SA3 for security aspects.

SA4 for media codec aspects.

SA5 for management and charging aspects.

RAN1/2/3 for RAN part enhancements.

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Tencent

T-Mobile

ABS

CBN

SJTU

China Telecom

OPPO

Spreadtrum

Alibaba

CAICT

Vivo

CATT

Xiaomi

ZTE

KPN

China Unicom

Oracle

Toyota

Sandvine

Qualcomm Incorporated

Facebook

Broadcom

Allot

Sony

NEC

Cisco

Samsung

Interdigital

Fudan University

LG Electronics

Apple

BT

Nokia

Nokia Shanghai Bell