3GPP TSG-SA2 Meeting #147-e	S2-2107609

# Online, , 18th-22nd Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 036 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new clauses for local MBS considering activation and idle/inactive mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Communications Co. LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. For local MBS, current specification does not specify how activation is performed considering the MBS service area, if not, UEs outside the area will be notified during activation, which is not necessary. 2. For local MBS, if UE moves out the area, the UE will not receive MBS data and may transit into IDLE/INACTIVE state. The UE may not know whether the MBS session is deactivated or not, the specification does not specify how UE and NW will do when moving again into the area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. SMF sends the area info to AMF for paging area consideration and UE selection. 2. UE transit into RRC-CONNECTED state when moving into the area, SMF provides information to MBS related information to RAN and RAN configure the UE with MBS resources. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4.X (new), 7.2.4.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (new text) * * * *

7.2.4.X	MBS session activation procedure

The MBS session activation procedure is performed as defined in clause 7.2.5.2 with the following additions:

-	In step 3, the SMF sends the MBS service area to the AMF.

-	In step 4, the AMF responds the list of the UE resides in the MBS service area.

-	In step 5, the AMF figures out the paging area considering the MBS service area.

-	In step 7, the AMF identifies and notifies the related SMF of the UE(s) that resident in the MBS service area.

-	In step 8, the SMF includes the MBS service area, and may include Area Session ID, in the N2 SM information.

# * * * * Second change (new text) * * * *

7.2.4.Y	Idle/inactive mobility procedure

If a UE that has joined into a multicast MBS session moves into the MBS service area from outside:

-	If the UE is in RRC-IDLE state, the UE initiates Service Request procedure to activate the associated PDU Session. The AMF triggers the notification of UE location to the SMF.

-	If the UE is in CM-CONNECTED with RRC-INACTIVE state, when the associated PDU Session is activated, the UE resumes RRC connection with the NG-RAN, when the associated PDU Session is inactive, the UE initiates Service Request procedure to activate the associated PDU Session. The AMF triggers the notification of UE location to the SMF.

-	If the MBS service area has been changed and has not been provided to the UE, the SMF sends the MBS service area to the UE.

-	If the SMF determines that the UE camps on a target cell inside the MBS service area based on the notification of UE location, the SMF triggers the modification of the associated PDU Session and sends the multicast MBS session information to the NG-RAN node, which contains MBS service area and may contain Area Session ID, if the multicast MBS session is in "inactive" state, the multicast MBS session information also contains "inactive" indication. When the NG-RAN node supports 5G MBS, the UE is aware whether the multicast MBS session is active or not based on whether the radio resources of the multicast MBS session is configured or not. When the NG-RAN node does not support 5G MBS, the associated QoS flow(s) in the associated PDU Session is setup for transmitting the multicast MBS session data.

# * * * * End of changes * * * *