3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107504

Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

Title:	[DRAFT] LS Reply on rejected NSSAI for the maximum number of UE reached when TAIs belonging to different PLMNs

Response to:	LS (C1-214895/S2-2107020) on rejected NSSAI for the maximum number of UE reached when TAIs belonging to different PLMNs from CT1

Release:	Rel-17

Work Item:	eNS_Ph2

Source:	[Huawei to be] SA2

To:	CT1

Cc:

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

Thanks CT1 for the LS on rejected NSSAI for the maximum number of UE reached when TAIs belonging to different PLMNs. Please find SA2’s answer as following:

Question: CT1 is discussing the UE handling on the received rejected NSSAI for the maximum number of UEs reached when the registration area contains TAIs belonging to different PLMNs. There are two options observed:

Option 1: the rejected NSSAI for the maximum number of UEs reached is applicable to all of these PLMNs in the registration area;

Option 2: the rejected NSSAI for the maximum number of UEs reached is applicable to the registered PLMN only.

SA2’s Answer: It is assumed in this scenario, the different PLMNs share the same resource (i.e. RPLMN and EPLMN), which means the NSAC should share one unified maximum UE number among these PLMNs. So SA2 agrees that Option 1 is correct in this scenario.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks CT1 group to

Take the above information into account and update their specifications if needed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021			Elbonia