SA WG2 Meeting #S2-152e(e-meeting)	S2-2205655

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	KI#6, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR/Rel-18

Abstract of the contribution: Introduction evaluations and conclusion for KI#6.

# 1. Introduction

This paper introduces the evaluation for solutions related to KI#6 and interims conclusion for KI#6

# 2. Proposal

This paper proposes the following conclusion points to TR 23.700-68 clause 8.

* Start of change *

# 7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

## 7.x	Evaluation on Key Issue #6

### 7.x.1	General

There is one solution, solution #9, documented for KI#6 in the current study.

### 7.x.2	Evaluation on solutions

Solution #9: It proposes that gNB (i.e. IAB-donor gNB) provides additional ULI, besides the normal ULI for the UE connected via IAB-node. The normal ULI (TAI/NR CGI) is the cell UE selected and covered by the IAB-node. The additional ULI is the cell selected by the IAB-node during restriction and managed by the IAB-donor gNB.

Considering the TAI/NR CGI in additional ULI is managed by gNB, it’s static and related to a specific geographic area if the TAI/NR CGI in normal ULI does not reflect a specific geographic area.

It shall also be realized that the geographic area covered by the mobile IAB-node is normally covered by the IAB-donor gNB. Using the additional ULI, together with the normal ULI, to manage mobility related restriction is reasonable.

* Next change *

# 8	Conclusions

Editor's note:	This clause provides the conclusions for the study.

## 8.x	Conclusions on Key Issue #6

The following interim conclusions are agreed:

- IAB-donor gNB provides additional ULI (e.g. TAI/NR CGI) in NGAP messages to AMF for UE accessing the network via IAB-node, together with the existing ULI. The additional ULI includes the TAI/NR CGI of selected by the IAB-Node when it’s registered to the network.

- AMF may use the additional ULI, together with the existing ULI, apply the mobility management (e.g. Mobility restriction).

NOTE: providing additional ULI needs further coordination with RAN WG and further updates may apply.

* End of changes *