3GPP TSG-SA2 Meeting #152E	S2-2205984

Aug 17 – 26, 2022

Source:	Rakuten Mobile

Title:	Update Solution#3 to consider the case of Macro base station in LTE

Document for:	Approval

Agenda Item: 	9.9

Work Item / Release:	FS_VMR

1	Discussion

For Solution #3: Reusing existing procedures for UE mobility it can be consider the case of Macro base station is in LTE i.e. eNB. Between 5G and LTE there are specific mobility procedure e.g. Dual connectivity, Emergency fallback. It should also clarified whether such inter RAT mobility can also supported between mobile base station relay and macro base station.

2	Proposal

It is proposed to adopt the following changes into TS23.700-05

* * * * Start of Change * * * *

6.3.3.2	UE mobility between mobile base station relay and macro base station

In case of UE mobility between mobile base station relay and macro base station, e.g. due to mobile base station relay out of service, the following applies:

-	If this mobile base station relay's IAB-donor node is this macro base station, the legacy procedure of Inter-gNB-DU Mobility as defined in the TS 38.401 [6] is used.

-	If this mobile base station relay's IAB-donor node is another macro base station including the case the macro base station is in the other RAT , the legacy handover procedure using the Xn or N2 reference points as defined in the TS 23.502 [5] is used.

-	If this mobile base station relay’s IAB-donor node is another macro base station in LTE, It is also clarified that the ENDC/dual connectivity or EPS/Emergency fallback can be applicable.

Editor's note:	The details of UE mobility between mobile base station relay and macro base station will be coordinated with RAN WGs.

* * * * End of Changes * * * *