SA WG2 Meeting #152E (e-meeting)	S2-2206021

Aug 17 – 26, 2022, Elbonia

Source:	LG Electronics

Title:	KI#2&1, Sol#11: Update to resolve ENs

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract of the contribution: This paper proposes to update solution #11 to resolve ENs.

Discussion

There are two ENs in Solution#11 as below.

Editor's note:	Whether MICO/PSM mode or a different indication of unreachability in the AMF/CN is FFS.

Editor's note:	It is FFS whether enhancements are required based on whether MICO or a different mode is used while the UE is out of coverage.

For the first EN, when the network (RAN or AMF) detects that the UE in CM-CONNECTED is about to be out of network coverage based on the coverage information, the network may trigger the AN release procedure to move UE into CM-IDLE. However, the UE moved to CM-IDLE state may initiate MO data or signalling although it is going to be out of coverage soon because the coverage information between the NW and UE may be different and UE may determine itself is in coverage. In this case, UE may fail to communicate because the UE will be out of coverage after a while. To prevent this case, NW can send an indication of unreachability to the UE and UE can decide not to send any UL data/signalling.

Regarding the second EN, existing MICO/PSM modes do not prevent the UE's Mobile Originated data and signalling because these modes are for UE power saving and it is not assumed that the UE will be out of coverage for a long time. A different mode to restrict the UE not to send MO data and signalling when the UE is out of coverage is required for power saving enhancement.

To resolve the both ENs, it is proposed that the NW can send an indication of unreachability with NTN MO Backoff time to the UE and when the UE receives the indication, the UE enables a new mode called NTN MO Backoff Mode, to withhold the UL MO data and signalling for a given time.

Proposal

It is proposed to agree the following changes into TR 23.700-28v0.3.0.

* * * * Start of 1st Change * * * *

6.11	Solution #11: Combined UE Management Architecture

6.11.1	Description

This is a solution for Key Issue #1: Mobility Management enhancement with discontinuous satellite coverage and Key Issue #2: Power saving enhancement for UE in discontinuous coverage.

When the UE is out of coverage, a UE is unreachable and any paging from the network will not succeed. The UE mobility patterns may be known to either the UE, network, both the UE and network or neither.

This solution combines proposed solutions for different cases of mobility and a solution for when the mobility pattern is not known therefore providing a complete solution for UE power saving and reachability management.

The solution has 2 phases. 1st phase when the UE is released, either at the end of a coverage window by the CN or the requested by the UE, or during a coverage period when the UE is not expected to return to CONNECTED during a coverage window. The 2nd phase is when the UE returns to coverage and the actions the UE takes when it returns to coverage.

Throughout the solution it is expected that the UE is aware of the coverage available from satellites and the UE, if it knows there is no coverage at a time, chooses not to attempt to activate its AS and initiate a connection or attempt to receive paging from the network.

If at any point between coverage periods, if the UE needs to connect to the network (e.g. for MO traffic or signalling) can detect coverage using a TN or NTN it can choose to initiate a connection, otherwise it is expected the UE will disable its AS procedures. Whether and how the UE choose to deactivate its AS when it is not required to receive paging or initiate a connection to the network is up to the UE implementation or network can send an indication to the UE to withhold UL MO data traffic/signalling or maybe addressed by other solutions.

The UE maybe in RRC_CONNECTED at the end of a coverage period, or at any point during a coverage period and transitioning from RRC_CONNECTED to RRC_IDLE. If the UE is in RRC_CONNCETED the UE may send a Registration Request to the network to transfer its understanding of its mobility pattern before the UE is released. If the UE is in RRC_IDLE or RRC_CONNECTED at the end of a coverage period, the UE may send a Registration Request to the to notifying the network it is leaving network coverage. In both cases, after the Registration procedure the UE is released.

The UE sends a Registration Request to inform the network about 4 different cases related to discontinuous coverage:

1.	The UE believes itself to be stationary or moving on a predictable trajectory.

2.	The UE understands it mobility and can predict a location it will be in the future and/or the time it will return to coverage at a location.

3.	The UE does not have an understanding of its mobility because it is not predictable.

4.	The UE is about to leave network coverage or has returned to network coverage and the UE has been requested to notify the network about its leaving network and returning to coverage in an earlier Registration procedure.

The network may use additional information to help determine the UE mobility in addition to the indications from the UE. This can include interactions with the NWDAF or other mobility information, for example mobility patterns provisioned by an AF to the network or other sources of information.

Taking into account the UEs indication about its mobility state, the network can combine it with information it holds and respond to the UE.

The response to the UE can be to:

1.	Provide the UE with an eDRX cycle the UE should follow, if there is no satellite coverage available to the UE at a specific location

2.	Enable MICO/PSM mode for the UE, providing a periodic registration timer to the UE that coincides with coverage in the future so that the UE performs Periodic Registration or TAU when it returns to coverage.

3.	Provide an indication that the UE should inform the CN when it leaves and subsequently returns to coverage in the future at an unpredictable time and use MICO/PSM mode until the UE returns to coverage. The CN should provide a periodic registration timer to the UE that covers the longest expected period out of coverage.

4.	If the UE had provided its capability for supporting NTN MO Backoff Mode and AMF had accepted the NTN MO Backoff Mode during earlier Registration, provide an indication that the UE to enable NTN MO Backoff Mode with NTN MO Backoff time to withhold UL MO data traffic/signalling.

If the UE is provided with an eDRX cycle, the CN may take into account the coverage at the predicted position when paging the UE. This may mean that some PTWs are skipped and this should be taken into account when responding to requests or providing a reachability time for the UE.

6.11.2	Procedures

6.11.2.1	5GS UE Leaving Coverage Procedure

This procedure is used when the UE is leaving coverage.

Figure 6.11.2.1-1: UE Leaving Coverage Procedure

1a.	RAN detects the UE is leaving coverage and initiates N2 UE Context Release Request if the UE is in RRC_CONNECTED.

1b.	The UE sends a Registration Request including:

-	[Optional] a time when it expects to return to coverage, and/or

-	[Optional] where the UE believes it will be when it returns to coverage, and/or

-	[Optional] an indication that it does not know when or where it will be back in coverage.

-	[Optional] an indication that the UE is leaving coverage if the UE has previously received a Leaving Notification Indication instructing the UE to notify the network when it is leaving the network coverage.

The UE may be in RRC_IDLE or RRC_CONNECTED when it determines it needs to send the Registration Request.

1c.	If the AMF detects that the UE is about to leave the current network coverage based on the coverage information, the AMF may trigger this procedure to move UE into CM-IDLE state when the UE is still within the network coverage.

2.	[Optional] The AMF make request predicated mobility and coverage information from the NWDAF.

3.	If the AMF had provided the UE with a Leaving Notification Indication and the UE is indicating that it is leaving coverage then a Registration Accept is sent, otherwise the AMF takes into account the UEs indications in Registration Request about mobility to determine whether to:

-	Provide the UE with an eDRX configuration the UE should follow, if there is no satellite coverage available to the UE at a specific location.

-	Enable MICO/PSM mode for the UE, providing a periodic registration timer to the UE that coincides with coverage in the future so that the UE performs Periodic Registration or TAU when it returns to coverage.

-	Provide the UE with a Leaving Notification Indication so the UE informs the CN when leaves or returns to coverage in the future and use MICO/PSM mode until the UE is out of coverage. The CN should provide a periodic registration timer to the UE that covers the longest expected period out of coverage.

-	If the UE and the AMF had negotiated NTN MO Backoff Mode, the AMF may include an indication to enable NTN MO Backoff Mode with NTN MO Backoff time.

The AMF uses the UEs indications in the Registration Request, its knowledge of mobility and coverage, if available, and information from the NWDAF, if available, to determine what to indicate to the UE for handling the out of coverage period. The AMF always provides a Periodic Registration Timer value, taking into account the coverage and how the UE will handle the out of coverage period.

If Registration Request was received in Step 1b, then the AMF responds with Registration Accept (step 3b) otherwise the AMF uses the UE Configuration Update procedure (step 3a).

The AMF may include an indication to enable NTN MO Backoff Mode with NTN MO Backoff Time during UE Configuration Update procedure.

4.	The AMF sends an N2 UE Context Release Command to the (R)AN.

5.	Steps 2 onwards of the AN Release Procedure as defined in clause 4.2.6 of TS 23.502 [3].

6.11.2.2	EPS UE Leaving Coverage Procedure

For the procedure for a UE leaving coverage in EPS is the same as the 5GS procedure in clause 6.11.2.1. with the following modifications:

-	Throughout: The TAU procedure instead of the Registration Procedure is used.

-	Steps 1b & 3b: The ATTACH procedure may additionally be used.

-	Steps 2 and 3a: The NWDAF and UCU procedure are not available in EPS, so these steps don't apply.

-	Step 5: S1 UE Context Release Command and the S1 release procedure, see clause 5.3.5 of TS 23.401 [5], instead of the N1 UE Context Release Command and the AN Release Procedure is used.

6.11.2.3	EPS and 5GS UE Returning to Coverage Procedure

This procedure is used when the UE returns to coverage if it was instructed to inform the network during the leaving coverage procedure that it has returned to coverage or e.g. if the time the UE returns to coverage is different from the expected time as determined in the leaving procedure.

If 5GS is used then the Registration procedure as defined in clause 4.2.2.2 of TS 23.502 [3] is reused and if EPS is used then Tracking Area Update procedure is used as defined in TS 23.401 [5].

The triggers the UE uses to detect coverage can be UE implementation dependent or defined by other solutions. The UE may take into account any previously received satellite ephemeris/coverage information from RAN, its expected or known position and time to determine whether to look for coverage.

6.11.3	Impacts on existing nodes and functionalities

UE:

-	Provide the AMF/MME with indications about its mobility predictability, including extrapolated future location and/or time in coverage, whether it is stationary/predictable mobility or not predictable.

-	Send indications to the AMF/MME when it is leaving coverage/returning to coverage if instructed to do so.

-	Support NTN MO Backoff Mode when it is about to leaving coverage.

RAN:

-	Trigger AN Release for a UE if it is about to leave coverage.

MME/AMF:

-	Obtain information on future coverage given satellites motion.

-	Trigger AN Release if the UE is about to leave coverage (5GS only).

-	Configure the power saving parameters or Leaving Notification Indication based on the coverage information for the UE, indications from UE, information from the NWDAF (5GS only) and other information the MME/AMF may hold/have been provisioned with.

-	Consider the UE unreachable while out of coverage.

-	Send an indication to UE to enable NTN MO Backoff Mode with NTN MO Backoff time. (5GS only)

* * * * End of Changes * * * *