SA WG2 Meeting #152e	S2-2206251

# August 17th – 26th, 2022; Elbonia               			(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-85: Conclusions for KI#1

Document for: 	Approval

Agenda Item: 	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This pCR proposes interim conclusions for KI#1, URSP in VPLMN

# 1 Discussion

With solutions that propose VPLMN generating the URSP rules, the HPLMN has no control on the actual URSP rule provided by the VPLMN and may lead to unexpected behaviour. This may risk user complaints to HPLMN leading to no usage of the feature.

For solutions with proposal that the V-PCF provides URSP related information to H-PCF during UE policy association, although the HPLMN has total control on the URSP rules generation, this would require the V-PCF to provide such information for each individual UEs. In practice it may happen that a group of UEs (e.g. all UEs from HPLMN using a specific DNN, S-NSSAI) the same rule shall be provided. In such case, VPLMN providing URSP guidance will be more optimal.

It is proposed to use the principles described in Solution #1 for supporting VPLMN specific URSP rules, while still keeping the control of generating the URSP rules at the HPLMN.

The "Location Criteria" in Route Selection Validity Criteria of the RSD is extended to support PLMN ID as the location. If an RSD in a URSP rule is valid throughout the VPLMN, the "Location Criteria" is set to the PLMN ID.

The UE may be configured with URSP rules with different RSD for each VPLMN, with "Location Criteria" containing corresponding VPLMN ID. This may lead to situations where the URSP is too big. In such cases it may be also possible that HPLMN dynamically update the URSP rules for a UE when the UE registers to the VPLMN i.e. update the URSP rules with the VPLMN specific RSD and Location Criteria.

# 2 Proposal

It is proposed to update 23.700-85 as follows

FIRST CHANGE

# 8	Conclusions

## 8.1	Conclusion on Key Issue #1

The following are concluded for KI#1, URSP in VPLMN

-	A VPLMN AF can provide URSP guidance to the HPLMN via the NEF in the HPLMN.

-	The UE subscription may contain the relevant VPLMN DNN and S-NSSAI information.

-	UE may re-evaluate URSP rules for a traffic when the Location Criteria becomes invalid.  The re-evaluation may cause the UE to move the traffic to a different PDU Session and release the PDU Session that was created/used for a specific location validity condition if there is no traffic is using the PDU Session after re-evaluation.

-	The "Location Criteria" in Route Selection Validity Criteria of the RSD is extended to support PLMN ID as the location. If an RSD in a URSP rule is valid throughout the VPLMN, the "Location Criteria" is set to the PLMN ID.

-	The UE may be configured with URSP(s) with different RSD for each VPLMN, with "Location Criteria" containing corresponding VPLMN ID. The HPLMN may also dynamically update the URSP rules for a UE when the UE registers to the VPLMN.

END OF CHANGES