3GPP TSG-SA2 Meeting #153E	S2-2208236r01

# E-Meeting, 10th – 17th October 2022

Source:	Ericsson, Orange

Title:	KI#1: Updates to Overall Evaluation for KI#1

Agenda item:	  9.23

Document for:	  Approval

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract:	This contribution updates the overall evaluation of KI#1 by adding evaluation for the solutions #61, #62, and #63. One editorial change is also applied.

1. Discussion

Overall evaluation is updated by adding information about solutions #61, #62, and #63. One minor editorial change is applied to make the titles consistent with the category names.

2. Proposal

Following changes in 3GPP TR 23.700-81 v1.0.0 is proposed.

* * * Start of Changes* * * *

## 7.1	Key Issue #1: How to improve correctness of NWDAF analytics

According to the Table 6.0-1, solutions #1-#7, solutions #28-#36, and solutions #61-#63 are proposed for Key Issue #1. All proposed solutions can be categorized in the following categories:

-	Support for using multiple ML models (4 solutions).

-	Dataset properties and ML model accuracy management (6 solutions).

-	Detection of drift of an ML model (5 solutions).

-	Receiving feedback from service consumers to NWDAF (3 solutions).

-	Accuracy report from NWDAF to the service consumer (1 solution).

In the following each category will be evaluated separately.

Support for using multiple ML models

Solutions #1, #31, #36 and #62 propose multiple ML models for improving overall accuracy of prediction generated by NWDAF. Having multiple ML models for a single analytics report helps NWDAF containing MTLF to provide more than one ML model during provisioning to NWDAF containing AnLF. When a prediction is being generated, NWDAF containing AnLF has more than one ML model available to use. This allows the NWDAF containing AnLF to choose a suitable ML model depending on, e.g. UE location, time of day, network load, or any other filter of information.

Solution #1 is about using several ML models by NWDAF(AnLF) and them voting and choosing the best prediction using an internal scoring mechanism. It also includes enhancing provisioning procedure to provide pairs of unique identifiers of the ML model and its corresponding ML model information when multiple ML models are available for an analytics report.

Solution #31 proposes providing several analytics reports to service consumer and let the service consumer to choose between delivered results for an analytics report. The service can use for instance confidence level in the analytics reports and choose the one with the highest value.

Solution #36 enhances ML model provisioning in a way that NWDAF containing AnLF includes extra information i.e. inputs about the data used for inference to NWDAF containing MTLF, when ML model provisioning process is triggered. This allows the NWDAF containing MTLF to select one out of multiple ML models, or even generate a new one. In this solution, the NWDAF containing AnLF receives a single ML model as per existing procedures. Only the NWDAF containing MTLF is assumed to have access to multiple ML Models, from which one of them is selected and provisioned to the NWDAF containing AnLF.

Solution #62 proposes a method to choose between multiple ML models when available for a single analytics ID, by introducing an additional optional parameter called "use case context" provided by the consumer. This parameter is used as a guideline to NWDAF to choose the best ML model that can be suitable for the context of the usage of the requested analytics report. NWDAF containing AnLF or NWDAF containing MTLF will use this parameter to choose between a set of already provisioned ML models or choose an ML model to provision. Since the values of the new parameter i.e., "use case context" do not need to be standardized, there is no limitation on the granularity of use cases involved.

Dataset properties and ML model accuracy management

Solutions #2, #3, #33, #35, #61, and #63 propose technics for ML model accuracy management by providing information about the datasets used to train ML models.

Solution #2 is applicable to the Service Experience analytics ID. The solution proposes that when the AF provides Service Experience Information for a group UEs, the AF also provides a "Service Experience Contribution Weight" with each UE's Service Experience value. The "Service Experience Contribution Weight" is determined by the AF and indicates the relative importance of each UE's Service Experience. This information can be used by the NWDAF containing MTLF to include more important values when predictions are generated.

Solution #3 is about calculation of ML model validation in NWDAF containing MTLF (Accuracy in Training) and then ML model accuracy measurement in NWDAF containing AnLF. Then, NWDAF containing AnLF collects the actual data and reports the measured accuracy (Accuracy in Use) as feedback to NWDAF containing MTLF. Diverging between these two values is detected by NWDAF containing AnLF and will be a sign that NWDAF containing MTLF needs to take action to improve the analysed ML model. This solution can be useful to increase the quality of ML models trained by NWDAF containing MTLF where a metric can be used to evaluate the ML model's performance dynamically. However, the usage of Accuracy in Use needs to be elaborated in a clearer way, for example, it is not clear how to detect that the actions of the NFs based on predictions modify the result of a prediction, thus, invalidating the input for the Accuracy in Use.

Solution #33 introduces a way for a service consumer to request corrections, in time, of analytics predictions NWDAF will then send corrected predictions within a time window to the service consumer.

Solution #35 discusses a way to further classify a AoI into sub-areas with different properties. A service consumer subscribes to area monitoring and NWDAF will provide information about expected environment properties. However, it is not clear what environmental statistical properties are really used to classify the sub-areas. The benefits are not clear that an AnLF should understand those properties in order to request ML model for different sub-area types.

Solution #61 considers further training of an existing ML model instead of training a new ML model. The solution assumes that previously training ML models are stored in ADRF which are referred as historical ML models. Such historical ML models are then trained with extra data to increase the accuracy and extending the spatial validity of and ML model. A further trained ML model is then provisioned by NWDAF containing MTLF to NWDAF containing AnLF. A further training process is initiated if no exact match for an ML model with respect to the filter information from a service consumer is found in ADRF. For instance, if filter information limits the analytics report to TA-1,2 NWDAF containing MTLF looks for historical ML models with the same TA, and if it didn’t find a match but an ML model with TA-1,3 is found, then the model will be further trained to cover TA-1,2,3. When the training is completed, NWDAF containing MTLF will store the ML model in ADRF which can be provisioned to NWDAF containing AnLF for inference.

Solution #63 proposes a rating mechanism for the data sources, which provides performance measurements or KPIs in two ways:

Option 1: A service consumer evaluates the quality of an analytics report on behalf of NWDAF containing AnLF and reports back the result of evaluation to NWDAF containing AnLF.

Option 2: NWDAF containing MTLF performs an evaluation process on an ML model and informs NWDAF containing AnLF if any degradation is detected. Finally, NWDAF containing MTLF asks NWDAF containing AnLF for validation check of data sources.

In option 1, it is not clear whether a service consumer is in the position to evaluate the quality of analytics report. Such process might need extra information e.g., statistical data that are not necessarily available for a service consumer.

In option 2, how NWDAF containing MTLF will evaluate an ML model for correctness is questionable and needs to be elaborated. Moreover, whether NWDAF containing AnLF is the most appropriate NF to perform evaluation of data sources or if a new NF might be needed, shall be discussed in more detail.

ML model drift detection

Solutions #5, #7, #29, #30, and #32 propose solutions for ML model drift detection.

Solution #5 is about error monitoring of ML models when a model is provisioned. If any error is detected in an ML model, then proper action e.g. re-training will be triggered. Two options are presented for detecting the drift of the ML model. Option #1 is based on detection performed at an NWDAF containing AnLF. Option #2 is based on detection performed at an NWDAF containing MTLF.

Option #1 is based on a comparison, performed at an NWDAF containing AnLF, between the result of a prediction and the actual value.

Option #2 is based on the NWDAF containing MTLF collecting the data and/or events that have been previously collected by the NWDAF containing AnLF, and a subsequent computation of the differences between the set of trained data at step and the collected events/data. This option implies a duplication in the collection of data, on one side by the NWDAF containing AnLF and in another side by the NWDAF containing MTLF, which is not efficient, even in cases where ADRF is used to store the collected data.

Solution #7 introduces the notion of a new Trusted Rating Logical Function, which delivers to the analytics consumers the rating of each ML model for a given NWDAF and Analytics ID. This allows the analytics consumer to select an NWDAF with a high rating. Then NWDAF can generate a token for rating the correctness of analytics reports. The solutions seems biased to the NWDAF with low rating. Since NWDAF with low rating may never be selected by the AnLF, they can hardly be re-rated. Furthermore, it is not clear what metrics are used in the rating, who provides those metrics. The task of evaluating NWDAF performance seems to be relevant mostly to OAM and not NWDAF.

Solution #29 is about detecting ML model drift in by NWDAF containing AnLF and report it back to NWDAF containing MTLF for proper action. The solution enables the NWDAF containing MTLF to learn about NWDAF containing AnLF using already provisioned ML Models, and a subsequent subscription from the NWDAF containing MTLF to those NWDAFs containing AnLF, which send notifications when they detect the ML model degradation. The NWDAF containing MTLF marks degraded ML models and takes the proper action (e.g. ML Model retraining, indication of "degraded" ML model in ML Model provisioning service operation, choose another ML model for the same Analytics ID).

Solution #30 is about a method where either NWDAF containing MTLF or AnLF will monitor one or multiple ML models for analytics report to evaluate the accuracy of an ML model. The overall solution is appropriate, but it needs to be established in a better way. For instance, it should be clear whether it is NWDAF containing MTLF or AnLF (NWDAF containing AnLF would be preferred) who is responsible for an ML model evaluation.

Solution #32 proposes that the NWDAF containing MTLF subscribes to the NWDAFs containing AnLF in order to get the performance of previously provisioned ML Models. The NWDAF containing AnLF will then detect possible ML model degradation and will inform the NWDAF containing MTLF accordingly.

Receiving feedback from service consumers to NWDAF

Solutions #4, #6, and #28 are improving accuracy of NWDAF using feedback from the service consumers.

Solution #4 is about feedback from the service consumer to NWDAF containing AnLF which will be forwarded to NWDAF containing MTLF to decide whether the action that has been taken by the service consumer can cause ML model drift or not. The idea that a service consumer will send information about the action that has been taken seems not to be feasible since NWDAF is not supposed to know the internals of NFs, and not understand what actions taken will imply for the network, and therefore, for a prediction. All the data that are needed by NWDAF for either statistics or prediction will be gathered when needed and no more detail about the type of the action is of interest and any usage of this information seems not to be feasible to be implemented in NWDAF.

Solution #6 is about sending feedback from the service consumer to NWDAF containing AnLF. The feedback which is mentioned in this proposal is simple and does not carry any extra information about type of the action which is not relevant to what NWDAF does. The proposed solution seems to be reasonable and feasible but needs more clarification for instance about how ML model accuracy parameters are calculated to detect if an ML model is degraded.

Solution #28 proposes a method to assess accuracy of prediction by comparing predicted and real values and then from service consumer side, provide feedback about the quality of the analytics reports. The feedback has information about performance of an analytics ID by monitoring relevant KPIs. The monitoring process will provide Analytics ID Grade Information and Unstable Analytics ID information which will be used to measure the correctness of each analytics ID.

Accuracy report from NWDAF to the service consumer

Solution #34 proposes sending accuracy report from NWDAF to the service consumer which is supposed to be used to manage the subscription e.g. terminate the subscription is accuracy is low or take the reported accuracy into account when a decision is being taken. This solution seems not to have benefit on accuracy of prediction from NWDAF since the usage of accuracy report by the service consumer is not clearly elaborated. Moreover, the confidence level reported by NWDAF to service consumer can be enough to decide whether results from NWDAF are reliable or not.

* * * End of changes * * * *