3GPP TSG-SA WG2#153E	S2-2208491

Elbonia, 10 - 17 October 2022	(was S2-22xxxxx)

Source:	CATT, LG Electronics, Intel

Title:	KI#6: Overall Evaluation and Conclusions

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This contribution proposes KI#6 Overall Evaluation and Conclusions.

# Introduction

This contribution proposes KI#6 Overall Evaluation and Conclusions.

# Proposal

It is proposed to capture the following texts into TR 23.700-33.

>>>>First Change<<<<

## 7.X	Key Issue #6: Support of PC5 Service Authorization and Policy/Parameter Provisioning

For Key Issue #6: "Support of PC5 Service Authorization and Policy/Parameter Provisioning", the solutions can be summarized and evaluated as the following:

-	Sol#8 addresses the service authorization and policy/parameter provisioning for UE-to-UE Relay, and it is evaluated under clause 7.1.

-	Sol#16 addresses the service authorization and policy/parameter provisioning for direct path switching between PC5 and Uu, and it is evaluated under clause 7.3.

-	Sol#25 addresses the service authorization and policy/parameter provisioning for multi-path for UE-to-Network Relay, and it is evaluated under clause 7.5.

-	Sol#14 and Sol#15 address the service authorization and policy/parameter provisioning for path switching between two indirect network communication paths, and they are evaluated under clause 7.2.

>>>>Second Change<<<<

## 8.X	Key Issue #6: Support of PC5 Service Authorization and Policy/Parameter Provisioning

For Key Issue #6 "Support of PC5 Service Authorization and Policy/Parameter Provisioning", the followings are taken as conclusions:

-	The mechanism and procedure defined in clause 5.1 of TS 23.304 [3] is used for provisioning the Authorization and Policy/Parameters for UE-to-UE Relay, Multi-path transmission for UE-to-Network Relay, Direct communication path switching between PC5 and Uu, and Path switching between two indirect network communication paths for UE-to-Network Relaying.

NOTE:	Backward compatibility with Rel-17 shall be considered when concluding this key issue for Rel-18 ProSe.

-	For service authorization and policy/parameter provisioning to UE and NG-RAN for UE-to-UE Relay operation, Sol#8 is selected as basis for normative work.

NOTE:	Whether and which bullets (as described in clause 6.8.2.5) need to be included in the "5G ProSe authorised" information sent from AMF to NG-RAN can be determined in normative phase based on RAN WGs feedback.

-	For service authorization and policy/parameter provisioning for path switching between two indirect network communication paths, it is concluded under clause 8.2.

-	For service authorization and policy/parameter provisioning for direct path switching between PC5 and Uu, it is concluded under clause 8.3.

-	For service authorization and policy/parameter provisioning for multi-path for UE-to-Network Relay, it is concluded under clause 8.5.

>>>>End Changes<<<<