3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208646r01

Elbonia, Oct 10th – 17th, 2022	(revision of S2-220xxxx)

Source:	Samsung (Rapp), Huawei (Rapp)

Title:	Merged Conclusion for FS_GMEC KI#4

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing Conclusion for KI#4.

1. Introduction/Discussion

Based on the discussion during CC for SA2#153E preparation, it is proposed to update the conclusion for KI#4 merging PCRs from volunteers.

- Support of SMF redundancy for reliability of the 5G VN group communication

- Architectural enhancements

a)         How to manage session management when multiple SMFs are involved to serve a 5G VN group

b)         How to manage communication among the UE group members when they are served by different UPFs and different SMFs including the case of UE(s) mobility

- Rel-16 compatibility issue

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

8.4	Key Issue #4: Multiple SMFs for VN group communication

The following are way forward principles.

-	The multiple SMFs serving the same 5G VN group can belong to a single SMF Set or different SMF Sets, this is to support SMF redundancy for reliability of the 5G VN group communication

-	The associations between one or more SMF Sets and the DNN, S-NSSAI of the associated 5G VN group can be managed in NRF.

-	The service area provisioned for the 5G VN group as concluded in KI#1 can be used by the 5GC to determine whether to associate multiple SMF sets with the 5G VN group.

Editor's note:	It is FFS the 	whether a Single-SMF indicator is needed

-	For UPFs served by a single SMF Set, N19-based forwarding, N6-based forwarding and local switch as per Rel-17 can be used with the following clarifications:

-	The SMF(s) can share contextual information associated with the 5G VN group (DNN + S-NSSAI). The contextual information may e.g. relate to the N19 configuration, to the list of PDU Sessions established by 5G VN group members, etc.

-  Implementation dependent signaling can be used between SMF(s) that are part of a SMF set e.g. based on an implementation choice SMF(s) within the set can select one SMF to control the N19 configuration. Signaling between SMF(s) that are part of a SMF set is based on implementation choice, the SMF set or SMF instances in SMF set need to support functionality for 5G VN group communications across SMFs.

-	No extra standardized impacts are expected.

-	For UPFs served by different SMF Sets,

-	static connectivity between UPFs can be GTP-U tunnel or other IETF-based protocols, the type of tunnel to implement the 5G VN connectivity between UPFs is up to network implementation and deployment.

Editor's note:	It is FFS whether to use GSMF-based solution to support dynamic control of the N19-based forwarding or use new "PAS UPF" event to support dynamic control of the N6-based forwarding

# * * * * End of changes * * * *