SA WG2 Meeting #S2-153E	S2-2208774r10

10 - 17 Oct, 2022, E-Meeting

Source: 	Ericsson, LG Electronics

Title: 	KI #4: Conclusion of Key Issue #4

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes a conclusion for Key Issue #4.

Discussion

This paper based on the current evaluation for Key Issue#4 proposes conclusion to support UE discover/select/access hosting network.

The revision r01 provides a merge of S2-2208291, S2-2208293, S2-2208294, S2-2208297, S2-2208317, S2-2208444, S2-2208774, S2-2208973, S2-2209005, S2-2209167, S2-2209175.

Proposal

Add the following conclusion to TR 23.700-08.

*** BEGIN CHANGES ***

8.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

8.4.1	General

The conclusion for KI #4 is made for each component that is evaluated in clause 7.4.

When UE accesses the Hosting network using the subscription/credentials of its Home network, only two cases are considered:

-	If Home network is PLMN, the Hosting network can be PNI-NPN or SNPN.

-	If Home network is SNPN, the Hosting network can be only SNPN

If the UE accesses the Hosting network using the other credentials rather than the subscription/credentials from the UE Home network, the determination of the subscription used to access the Hosting network is by implementation specific prior to automatic network selection as described in NOTE 1 of clause 5.30.2.4.2 of TS 23.501 [3].

8.4.2	Conclusion for the content of the localized service information

The following principles based on the evaluation in clause 7.4.2 are recommended for the normative work:

1.	(Principle 1) The localized service information is provided to UE for UE to discover and select hosting network to receive desired localized service. The content of the localized service information can include the following elements:

a.	Identifier/name of the localized service.

b.	Validity conditions for the localized service, e.g. duration of remaining service operation ,time and location.

c.  Local service captive portal address which is used by User to access the service after UE registering in the network if  manual network selection is performed.

d1.	Hosting network related information, per hosting network in case of SNPN as hosting network:

i.	hosting network identifier, e.g. SNPN ID, GIN;

ii.	Validity condition, e.g. time and/or location (optional)

.

Editor's Note 2-1a:	It is FFS whether Validity condition is to be possible to be set localized service, hosting network or both.

c.	Hosting network related information, per hosting network in case of PNI-NPN as hosting network:

i.	PLMN ID of the PNI-NPN.

ii.	Optionally, if hosting network is associated with CAG IDs, a list of CAG IDs corresponding to the localized service.

Editor's Note 2-2:	It is FFS whether it is beneficial to provide CAG information within the localized service information, given that there is existing procedure to update UE with CAG information, and whether CAG ID is used to identify hosting network.

Editor's note 2-3:	It is FFS whether more hosting network related information can be sent as to better enable a user selection, e.g. cost to access the hosting network for the localized service.

2.	(Principle 2) The UE is provided with:

a.	a list of prioritized hosting networks (i.e. SNPN ID or GIN) for localized services, in case of SNPN as hosting network.

b.	allowed CAG ID list, in case of PNI-NPN as hosting network and the PNI-NPN is associated with CAG ID.

NOTE:	The home network / hosting network map localized services to hosting network ID (e.g. SNPN ID, GIN, CAG ID).

Editor's Note 2-4:	It is FFS whether the UE receives list a/list b per localized service ID, or the lists are used for localized services in general.

Editor's Note 2-5:	It is FFS whether necessary to associate validity conditions with the list in a and ba above, and what are the conditions, e.g. time and/or location.

Editor's Note 2-6:	It is FFS what localized service information needs to be provided to UE in b above, in case PNI-NPN as hosting network does not support CAG.

Editor's Note 2-7:	Principle 1 provides UE with necessary information as a first step, and then the UE based on the received information derives how to perform other activities in later steps, e.g. network selection, credential determination and provisioning, etc. Principle 2 provides UE with information directly used for network selection, but requires that UE/network knows beforehand whether the desired localized service is provided by the hosting networks in the lists. It is FFS whether both principle 1 and principle 2 above are progressed to normative work.

8.4.3	Conclusion for from where and how UE obtains the localized service information

The following principles based on the evaluation in clause 7.4.3 and principle 1 in clause 8.4.2 are recommended for the normative work:

1.	The localized service information is formulated as application data that can be stored in either home network, hosting network or serving network.

2.	The external parameter provisioning procedure in TS 23.502 [4] clause 4.15.6 is extended to support the provisioning of localized service information to home, hosting or serving network, or the OAM method is used to provision the data to home/serving network.

3.	The localized service information can be preconfigured in the UE or dynamically provisioned via signaling. For dynamic provisioning, the UE requests the desired application data (e.g. by specifying the identifier or name of the localized service) from either home network, hosting network or serving network via new UE policy.

NOTE:	How UE obtains localized service identifier or name to make the request for dynamic provisioning is out of 3GPP scope.

4.	If serving network provides the localized service information, Serving network needs to verify that the localized service for the UE is authorized by home network before provisioning UE with localized service information.

Editor’s Note 3-1:	It is FFS whether home network authorization is needed for UE to obtain localized service information, as home network authorization can be done during hosting network selection.

Editor's Note 3-2:	It is FFS whether UE can additionally fetch localized service information from hosting network e.g. via SIB, or via new UE policy.

5.	If PNI-NPN as hosting network is associated with CAG ID, separated from other localized service information, the UE obtains the Allowed CAG ID list from the home network as per clause 5.30.3.3 TS 23.501[3].

a.	The home network may send the updated Allowed CAG ID list to the UE

b.	The UE may request for the Localized service from the serving network. The home network can send the updated Allowed CAG ID list to the UE after authorizing that the UE is allowed to access the Localized service.

The following principles based on the evaluation in clause 7.4.3 and principle 2 in clause 8.4.2 are recommended for the normative work:

6.	The information for localized service and hosting network discovery, selection and access can be preconfigured in the UE or dynamically provisioned via serving(visited), hosting network or home network. The information for localized service and hosting network discovery, selection and access can also be obtained by UE at the application layer from the home network or the localized service provider via means that are outside of 3GPP scope.

7a.	In case of SNPN as hosting network, the dynamic provisioning can be done via NAS procedure by serving AMF, if there is agreement between home network, hosting network and serving network.

i.	During UE initial registration procedure, the UDM may request serving network AMF as part of subscription information to provide to the UDM a list of the available hosting network information known to serving network/AMF and/or authorize AMF to provide UE assistance information for discovery and selection of hosting networks.

ii.	When authorized by the UDM, AMF may provide UE assistance information for discovery and selection of hosting networks as part of Registration procedure (Mobility Update) or UE Configuration Update procedure.

iii.	The AMF is aware of availability of hosting network(s) that corresponds to allowed areas (such as tracking area) which represent the current UE location as indicated in the Mobility Registration Update sent by the UE. The AMF may assign Registration Area to the UE in such a way that the UE performs Mobility Registration Update when UE moves to into the Tracking Area (TA) where the hosting network is present. The AMF may be aware of the availability hosting network(s) in the registration area by local configuration or by service agreement with hosting network.

7b.	In case of SNPN as hosting network, the dynamic provisioning can be done via SoR procedure by UDM.

i.	UDM may receive list of hosting network information from SOR-AF.

NOTE:	How SOR-AF acquires hosting network information is outside the scope for 3GPP.

ii.	The home network UDM may determine to update UE with prioritized list of hosting network information using SoR procedure. Following triggers may apply

-	UE location as part of Registration procedure.

-	UE subscription data change, e.g., via external parameter provisioning.

7e. When UE has accessed with the hosting network, UE may query for further information on the localized service information hosted by this hosting network during the initial registration, AMF can provide the latest localized service information to UE as part of Registration procedure (Mobility Update) or UE Configuration Update procedure.

NOTE:	Given the potential large content of SIB information for manual selection, on demand SIB solution as already defined in TS 38.331 can be used. Details of this option are to be determined by RAN2.

Editor's Note 3-3:	It is FFS whether 7a/7b/7c/7d/7e are all progressed to normative work.

8.	In case of PNI-NPN as hosting network, the dynamic provisioning of allowed CAG ID list reuses existing procedure in clause 5.30.3.3 TS 23.501 [3].

8.4.4	Conclusion for how the localized service information is used by UE

The following principles based on the evaluation in clause 7.4.4 are recommended for the normative work:

1.	If UE uses home network credential to access hosting network:

a.	When end user intends to access localized service and the validity conditions of localized service are met, the UE initiates hosting network selection using the hosting network related information received as part of the localized service information as in principle 1 in clause 8.4.2, or using the lists as in principle 2 in clause 8.4.2.

i.	UE can switch between PLMN selection and SNPN selection as per Rel-17 specification. How the UE switches among the network selections is up to UE implementation.

ii.	For PNI-NPN as hosting network associated with CAG ID, the UE only considers an entry in the Allowed CAG list valid if and while all conditions (if there is any) for that entry are met. This may potentially initiate a new registration procedure to a PLMN.

NOTE:	Details regarding priority list for hosting network selection is up to WG CT1 to decide.

b.	Hosting network selection needs to be authorized by home network, via UE initiated SoR procedure with SoR information including certain authorized criteria e.g. time. After the home network authorization, UE is allowed to initiate hosting network selection, applicable for both automatic and manual hosting network selection.

i.	For automatic hosting network selection, UE requests home network to authorize and determine the priority list for hosting network selection. Automatic hosting network selection requires end user conset.

ii.	For manual hosting network selection, UE presents available localized service information it has received in clause 8.4.3 to the end user.

Editor's Note 4-1:	It is FFS whether the home network authorization is needed before UE performs manual selection of a hosting network, and for how the home network rejects accessing hosting network when a UE registers to a hosting network via manual selection and the UE is not authorized to access hosting network.

c.	When authorized criteria of the hosting network selection are no longer met, UE stops hosting network selection.

2.	If UE needs to obtain new set of credential/subscription to access hosting network:

a.	It is up to UE implementation to decide how to switch to the new subscription profile for accessing hosting network.

3.	UE determines SNPN access mode is activated/de-activated using implementation specific means as specified in existing Release 17, or using received localized service/hosting network assistance information as input.

8.4.5	Conclusion for what credentials are used to access hosting network and how to obtain them

The following principles based on the evaluation in clause 7.4.X are recommended for the normative work in case of SNPN as hosting network:

1.	The UE checks whether it is possible to use home network credentials to access the hosting network:

a.	If the hosting network related information indicates support of CH credentials, the UE determines home network credential can be used if the SNPN ID of the hosting network is included in the SNPN priority lists associated with home network subscription.

b.	If the UE obtains hosting network selection information from the home network, the UE uses the credentials provided by the home network to access the hosting network.

2.	If new credential for accessing hosting network is needed, the UE may use the existing credential onboarding mechanism as per Rel-17.

NOTE:	The method of delivering credentials needs to be evaluated by SA3.

The following principles based on the evaluation in clause 7.4.X are recommended for the normative work in case of PNI-NPN as hosting network:

3.	Only PLMN UEs can access a hosting network which is a PNI-NPN. When the UE requests to access the hosting network, the home PLMN credential(s) is used during authentication procedure.i

8.4.6	Conclusion for how localized service is accessed in hosting network per agreed conditions

The following principles based on the evaluation in clause 7.4.Y are recommended for the normative work:

1.	Validity conditions provided to the UE as part of the localized service information can be used to restrict the UE's access of the hosting network.

2.	Existing methods, such as network slicing, forbidden area restriction, service area restriction, CAG, LADN, URSP rules can also be utilized to restrict UE's access, i.e. no need for additional normative work for access control in hosting network.

3.	In order to restrict access to a hosting network to a specific area, a hosting network operator may deploy and broadcast multiple hosting network IDs, i.e. SNPN IDs for SNPN case and CAG IDs in case of PNI-NPN, in different areas depending on localized service area validity. Each localized service is mapped to a specific hosting network ID. Multiple localized service areas can be mapped to the same hosting network ID if their allowed service areas are the same. Validity conditions are also used by hosting network to restrict access.

*** END CHANGES ***