3GPP TSG-SA WG2 Meeting #154AHE	S2-2301168

# E-meeting, 16th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0682 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Roaming architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Roaming architecture for data analytics needs to defined to support data and analytics exchange in roaming case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add roaming architecture for network data analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data and analytics exchange in roaming case is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 4.3.0 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

4.3	Roaming architecture

4.3.0	General

As depicted in Figure 4.3.0-1, the 5G System architecture allows:

-	the H-NWDAF to request network analytics information in VPLMN from the V-NWDAF containing AnLF, and to collect data in VPLMN from the V-NWDAF.

-	theV-NWDAF to request network analytics information in HPLMN from the H-NWDAF containing AnLF, and to collect data in HPLMN from the H-NWDAF.

Figure 4.3.0-1: Network Data Analytics Exposure architecture - roaming

The Nnwdaf interface is used for the H-NWDAF (or V-NWDAF) to request subscription to network analytics for a particular context, to cancel subscription to network analytics and to request a specific report of network analytics for a particular context from the V-NWDAF (or H-NWDAF) containing AnLF.

The Nnwdaf_DataManagement services as specified in clause 7.4 is used for the H-NWDAF (or V-NWDAF) to collect data from the V-NWDAF (or H-NWDAF).

* * * End of Changes * * * *