3GPP TSG-SA WG2 Meeting #154		S2-2210256

# Toulouse, France, Nov 14 – 18, 2022		 		  											 (revison of )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0143 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Group Message Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR 23.700-47, KI#4 Group Message Delivery has been concluded. The normative work needs to update TS 23.247 for the new feature of Group Message Delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Group Message Delivery, introduce new clauses in feature description, procedures and service definition, according to the conclusion in the technical report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing group message delivery functionality, which has been concluded in TR 23.700-47 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new), 7.Y (new), 9.4.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.501 CR 3755 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_5MBS_Ph2 and the WI code for the work is not assigned yet & will replace WI code DUMMY when available. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

## 6.X	Group Message Delivery

The Group Message Delivery feature allows AFs to deliver a payload to a group of UEs located in a particular geographical area when MBS broadcast is used, which enables efficient group message delivery. The AF can request group message delivery as specified in clause 7.Y.1. The AF may recall or replace a previously submitted group message as described in clause 7.Y.2.

Group Message Delivery utilizes the Object Delivery Method in MBSTF specified in TS 26.502 [18]. The Object Delivery Method in MBSTF is equivalent to the File Delivery Method in eMBMS. The Object Delivery Method can benefit from Application Layer Forward Error Correction (AL-FEC) to achieve the reliable delivery, which is essential for group message delivery.

The NEF is responsible to handle Group Message Delivery request from the AF. It transforms the group message into a file and determine the meta data information of the file. In control plane, it performs Application Service Provisioning including MBS User Service creation and MBS User Data Ingest Session creation, which triggers the MBS session creation and MBS session start for broadcast towards 5GC and NR. In user plane, it is responsible for ingesting the file to the MBSTF, so that MBSTF can deliver the file to UE via 5GC shared traffic delivery and NR broadcast.

* * * * Next Changes * * * *

## 7.Y	MBS procedures for Group Message Delivery

Editor’s Note:	The service operation and parameters are FFS. It requires further investigation on whether the service operation should align more with T8 interface (delivery & modify) or follow the SBI design principles (CRUD).

Editor’s Note:	It is FFS whether TLTRI (T8 Long Term Transaction Reference ID) should be kept or use Group Message Correlation ID instead.

### 7.Y.1	Group Message Delivery via MBS Broadcast

Figure 7.Y.1-1: Group Message Delivery via MBS Broadcast.

NOTE 1:	In the procedures referred to the TS 26.502 [18], the NEF is the MBS Application Provider.

1.	The AF sends Group Message Delivery Request to the NEF, containing the Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time, External Group Identifier.

2.	The NEF checks authorization of AF. If geographical area information or civic address information was provided by the AF as MBS service area, the NEF translates the MBS service area to Cell ID list or TAI list. The NEF may further check the MBS capability within the MBS service area.

NOTE 2:	The NEF is mandated for group message delivery.

3.	The NEF transforms the group message payload into a file, and determines the meta data information of the file (e.g. File URL, etc.). The NEF assigns a Group Message Correlation ID that identifies this Group Message Delivery Request.

If Application Service Provisioning hasn't been performed, step 4 to step 8 needs to be executed. Otherwise, they can be skipped.

4.	The NEF performs Application Service Provisioning towards the MBSF as specified in step 1 in clause 5.2 of TS 26.502 [18], which including invoking Nmbsf_MBSUserService_Create and Nmbsf_MBSUserDataIngestSession_Create on the MBSF.

-	The Target service area is set to MBS Service Area.

-	The Distribution method is set to Object Distribution Method which is used for file delivery.

-	The Distribution Operating Mode is set to File or Carousel depends on the decision of the NEF.

-	The Object acquisition method is set to Push or Pull depends on the decision of the NEF.

5.	The MBSF performs MBS Session Creation as specified in clause 7.1.1.2 or clause 7.1.1.3.

6.	The MBSF performs Distribution Session Provisioning as specified in step 2 in clause 5.2 of TS 26.502 [18]. The MBSF invokes Nmbstf_MBSDistributionSession_Create on the MBSTF, passing the parameters of the MBS Distribution Session received in step 4 to the MBSTF.

7.	The MB-SMF initiates the MBS Session Start for Broadcast procedure as specified in step 2 to step 9 in clause 7.3.1.

8.	If the MBSF performs the service announcement, it initiates the MBS User Service Announcement as specified in step 3 in clause 5.2 of TS 26.502 [18]. The application may receive the appropriate information through the MBS-6 API from the MBS Client (see TS 26.502 [18]). The NEF may receive the service announcement information via Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation (see TS 26.502 [18]).

9.	The NEF sends Group Message Response to the AF, containing Group Message Correlation ID, Acceptance Status, Cause. The Acceptance Status indicates whether the group message delivery request is accepted or not. If not, the Cause indicates the appropriate failure reason. If the AF performs the service announcement, the service announcement information containing the file meta data can be optionally included in the Group Message Response.

10.	If the AF needs to perform the Service Announcement, the AF sends the application service announcement to the UE as specified in step 4 in clause 5.2 of TS 26.502 [18].

11.	The NEF performs the User Data Ingestion towards the MBSTF as specified in step 5 in clause 5.2 of TS 26.502 [18]. The NEF may push the file to the MBSTF or let MBSTF pull the file from the NEF.

12.	The MBSTF performs packetization and optionally FEC encoding as specified in clause 4.3.3.2 of TS 26.502 [18].

13.	The MBSTF delivers the packets to the MB-UPF to NG-RAN, and NG-RAN broadcast to the UE as specified in step 13 to step 15 in clause 7.3.1.

14.	Based on the service announcement information received in step 8 or step 10, the UE receives the packets, is required performs FEC decoding to restore the file, and gets the group message from the file, using the MBS Client as specified in clause 4.3.5 of TS 26.502 [18]. The MBS Client can expose the file towards the application in the UE using the MBS-7 API (see TS 26.502 [18]).

15.	The NEF sends a Group Message Delivery response to the AF, containing Group Message Correlation ID, Delivery Status. The Delivery Status indicates whether delivery of Group Message Payload corresponding is successful or not. The delivery status information is received by the NEF via Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation (see TS 26.502 [18]). The NEF may further include the area where MBS is not supported in the Group Message Delivery response to the AF.

### 7.Y.2	Modification of previously submitted Group message

Figure 7.Y.2-1: Modify or Cancel Group Message Delivery via MBS Broadcast

NOTE:	In the procedures referred to the TS 26.502 [18], the NEF is the MBS Application Provider.

1.	The AF sends Modify Group Message Request to the NEF, containing the Group Message Correlation ID, Request Action, Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time, External Group Identifier. The NEF identifies the associated MBS Service using the external Group Identifier. Requested Action is either set to "Modify", or "Cancel". "Modify" indicates the request is to modify the group message delivery transaction. "Cancel" indicates the request is to cancel the group message delivery transaction. The Group Message Correlation ID indicates the transaction to be modified or cancelled.

2.	The NEF checks authorization of AF. If geographical area information or civic address information was provided by the AF as MBS service area, the NEF translates the MBS service area to Cell ID list or TAI list. The NEF may further check the MBS capability within the MBS service area.

If Requested Action is "Modify" and Group Message Payload is updated, step 3 to step 6 are executed:

3.	The NEF transforms the group message payload into a file, and use the determined file meta data (e.g. File URL, etc.) in clause 7.Y.1.

4.	If Object acquisition method is set to Push, step 4 to step 5 can be skipped. If Object acquisition method is set to Pull, the NEF updates MBS User Data Ingest Session on the MBSF as specified in clause 7.2 of TS 26.502 [18]. The update service operation needs to indicate an update of the file containing the updated group message.

5.	The MBSF updates MBS Distribution Session on the MBSTF as specified in clause 7.3 of TS 26.502 [18]. The update service operation needs to indicate an update of the file containing the updated group message.

6.	If Object acquisition method is set to Push, the NEF pushes the update file to the MBSTF. If Object acquisition method is set to Pull, the MBSTF pull the updated file from the NEF. And the MBSTF delivers the updated file towards the MB-UPF in 5GC as specified in clause 4.3.3.2 of TS 26.502 [18].

If Requested Action is "Modify" and MBS service area is updated, step 7 to step 8 are executed:

7.	The NEF updates MBS User Service on the MBSF as specified in clause 7.2 of TS 26.502 [18]. The update service operation indicates MBS service area update.

8.	The MBSF performs MBS Session Update as specified in clause 7.1.1.6 or clause 7.1.1.7 to update MBS service area, which triggers MBS Session Update for Broadcast as specified in clause 7.3.3.

If Requested Action is "Cancel", step 9 to step 14 are executed: step 9 to step 11 are needed when the group message delivery has started and MBSF needs to inform the MBS Client about the cancel of the file delivery. Otherwise, they can be skipped.

9.	The NEF updates MBS User Data Ingest Session on the MBSF as specified in clause 7.2 of TS 26.502 [18]. The update service operation needs to indicate the cancel of the file delivery. The MBSF informs the MBS Client about the cancel of the file delivery if needed.

10.	If Object acquisition method is set to Pull, the MBSF updates MBS Distribution Session on the MBSTF as specified in clause 7.3 of TS 26.502 [18]. The update service operation indicates the cancel of the file delivery. The MBSTF stops the file delivery.

11.	If Object acquisition method is set to Push, the NEF may send HTTP DELETE to the MBSTF to cancel the file delivery. The MBSTF stops the file delivery.

12.	The NEF destroys MBS User Data Ingest Session on the MBSF as specified in clause 7.2 of TS 26.502 [18].

13.	The MBSF destroys MBS Distribution Session on the MBSTF as specified in clause 7.3 of TS 26.502 [18].

14.	The MBSF performs MBS Session Deletion as specified in clause 7.1.1.4 or clause 7.1.1.5, which triggers MBS Session Release for Broadcast as specified in clause 7.3.2.

15.	The NEF sends Modify Group Message Response to the AF, containing Acceptance Status, Cause. The Acceptance Status indicates whether the group message request is accepted or not. If not, the Cause indicates the appropriate failure reason. The NEF may further include the area where MBS is not supported in the Group Message Delivery response to the AF.

After the modified group message is delivered, the NEF sends a Group Message Delivery to the AF as described in step 15 in clause 7.Y.1.

* * * * Next Changes * * * *

### 9.4.Z	Nnef_MBSGroupMsg Service

Editor’s Note:	The service operation and parameters are FFS. It requires further investigation on whether the service operation should align more with T8 interface (delivery & modify) or follows the SBI design principles (CRUD).

Editor’s Note:	It is FFS whether TLTRI (T8 Long Term Transaction Reference ID) should be kept or use Group Message Correlation ID instead.

#### 9.4.Z.1	General

Service description: NF Service Consumer can use this service to request group message delivery or to modify or to cancel previously submitted group message delivery.

#### 9.4.Z.2	Nnef_MBSGroupMsg_Delivery service operation

Service operation name: Nnef_ MBSGroupMsg_Delivery

Description: This service is used by the NF Service Consumer to request group message delivery.

Inputs, Required: Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time, External Group Identifier.

Inputs, Optional: None.

Outputs, Required: Group Message Correlation ID, Delivery Status

Outputs, Optional: Cause, service area without MBS capability.

#### 9.4.Z.3	Nnef_MBSGroupMsg_Modify service operation

Service operation name: Nnef_MBSGroupMsg_Modify

Description: This service is used by the NF Service Consumer to recall or replace group message delivery.

Inputs, Required: Group Message Correlation ID, Request Action (Modify/Cancel), External Group Identifier.

Inputs, Optional: Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time.

Outputs, Required: Acceptance Status

Outputs, Optional: Cause, service area without MBS capability.

#### 9.4.Z.4	Nnef_MBSGroupMsg_StatusNotify service operation

Service operation name: Nnef_ MBSGroupMsg_StatusNotify

Description: This service is used by the NEF to notify the NF service consumers of the status of a group message delivery request.

Inputs, Required: Status Information, Group Message Correlation ID.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

* * * * End of Changes * * * *