3GPP TSG-WG SA2 Meeting #154 	S2-2210340

Toulouse, France, November 14 – 18, 2022

Source:	Huawei, HiSilicon

Title:	KI#1: conclusion update

Document for:	Approval

Agenda Item:	9.14.1

Work Item / Release:	FS_eNS_ph3 / Rel-18

Abstract: It is proposed to update the conclusion of KI#1.

1. Discussion

1.1 NSI reselection

There are two Editor's notes for bullet 2) regarding the NSI reselection in case of multiple NSIs. Some companies think the principle of Network Slice instance selection is in SA5’s scope.

2) In case of a same S-NSSAI associated with multiple NSIs and the change of the NSI is required, when an existing PDU session is decided as to be migrated to another NSI, the SMF performs PDU Session re-establishment by using the mechanism of SSC mode#2 or SSC mode#3. When a PDU session establishment request from the UE arrives at the AMF, the AMF performs NSI re-selection i.e. removes from the UE context the old NSI and the AMF selects another NSI, e.g. as described in solution #15. Editor's note: It is FFS whether this principle is in scope of SA5. It could be sufficient if the AMF selects the appropriate NSI at PDU Session establishment. Editor's note: Bullet 2) needs to be further evaluated as if the issue is C-plane overload of NSI this is not a good solution as the load of C-plane is increased by the need to handle migrations. If it is a UP overload then the question is whether the load balancing weights needs to take into account UP load also. In general, a proactive approach in assigning the PDU sessions to least loaded UP NSI would be better than this reactive approach. The need of this solution in normative phase is FFS.

Actually, the selection of Network Slice instance is discussed in SA2 since Rel-15. The existing mechanism of Network Slice instance selection since Rel-15 allows the NSSF determines the NSI ID associated with an S-NSSAI during UE’s registration procedure or PDU session establishment procedure. The AMF stores the selected NSI ID associated with an S-NSSAI in the UE context. For UE’s other PDU session associated with the same S-NSSAI, the AMF uses this selected Network Slice instance to serve PDU session based on UE context.

However, when NSSF selects a Network Slice instance corresponding to a given S-NSSAI, the NSSF only takes the current load status into account. However, the network cannot predict how many PDU sessions associated with the same S-NSSAI will be established by the UEs later. Once the Network Slice instance is selected, the Network Slice instance is assigned to the UEs. When the load status of network slice instance is changed as other UE’s PDU connections to this Network Slice instance, the AMF will not query NSSF to get the information of least loaded Network Slice instance because the SA2 mechanism don’t allow AMF to do so.

As illustrated in figure 1, the NSI#1 selected for PDU session is not overloaded when NSSF assigns the NSI#1 to S-NSSAI. But the AMF is restricted to use the NSI#1 to serve other new PDU sessions for the same S-NSSAI. As the number of PDU sessions increases, the load of NSI#1 aggravates because the current SA2 design doesn’t allow AMF to query NSSF for Network Slice instance at every PDU session establishment procedure.

Figure 1: Legacy mechanism of Network Slice instance selection defined in SA2 since Rel-15

Therefore, SA2 needs to consider how to eliminate the restriction of Network Slice resource used for UE’s PDU session. The load control of Network Slice instance is SA2’s scope instead of SA5.

Proposal#1: based on the above analysis, SA2 needs to address this issue in order to eliminate the restriction of Network Slice resource used for PDU session. The Editor's notes for bullet 2) can be removed.

1.2  UE MM configuration update

There are one Editor's note for bullet 4) regarding whether the existing Mapping Of Allowed NSSAI can be re-used when AMF provides the alternative S-NSSAI to UE.

4) In cases where the old S-NSSAI is replaced by an alternative S-NSSAI deployed on a different NSIs, the following principles apply: a) The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet. The AMF provides information that the alternative S-NSSAI is associated with a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case; and independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs. Editor's note: It is FFS whether the existing Mapping Of Allowed NSSAI can be re-used, or an alternative IE (e.g. Replaced NSSAI) has to be specified.

According to the descriptions of Mapping Of Allowed NSSAI in TS 23.501, the Mapping Of Allowed NSSAI allows the UE to associate Applications to S-NSSAIs of the HPLMN as per NSSP of the URSP rules or as per the UE Local Configuration (if available) and to the corresponding S-NSSAI from the Allowed NSSAI but this is removed in R15. The alternative S-NSSAI also allows the UE to associate Applications to the alternative S-NSSAI corresponding to the old S-NSSAI from the Allowed NSSAI. Considering the same UE behaviour, there is no need to introduce new IE (e.g. Replaced NSSAI).

Proposal#2: based on the above analysis, it is proposed to reuse existing Mapping Of Allowed NSSAI.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-41 V1.1.0.

# * * * * First change * * * *

8.1	Conclusions for KI#1

Editor's note:	These are interim conclusions.

The following principles are concluded for the normative work.

NOTE:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM).

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may interact with  with the NSSF to determine the alternative S-NSSAI. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. The Subscribed S-NSSAIs of the UE are not updated for the purpose of KI#1.

2)	In case of a same S-NSSAI associated with multiple NSIs and the change of the NSI is required, when an existing PDU session is decided as to be migrated to another NSI, the SMF performs PDU Session re-establishment by using the mechanism of SSC mode#2 or SSC mode#3. When a PDU session establishment request from the UE arrives at the AMF, the AMF performs NSI re-selection i.e. removes from the UE context the old NSI and the AMF selects another NSI, e.g. as described in solution #15.

3)	In cases where the old S-NSSAI and the alternative S-NSSAI are associated with the same NSI, and the IP address is retained at same UPF and SMF does not change, the PDU session is associated with multiple S-NSSAIs in the network (i.e. NG-RAN, AMF, SMF, UPF). During PDU session establishment or HO procedure, the AMF notifies the alternative S-NSSAI to SMF and the SMF provides the alternative S-NSSAI to the NG-RAN without UE impact, as described in solution #1.

4)	In cases where the old S-NSSAI is replaced by an alternative S-NSSAI deployed on a different NSIs, the following principles apply:

a)	The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Configured NSSAI, if not included yet. The AMF provides information that the alternative S-NSSAI is associated with a mapped value to the old S-NSSAI as Mapping of Allowed NSSAI in both roaming or non-roaming case; and independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs.

-

--

# * * * * End of changes * * * *