3GPP TSG-WG SA2 Meeting #154 	S2-2210344

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Apple

Title:	KI#5: Update of conclusion for KI#5

Document for:	Approval

Agenda Item:	9.14.1

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This contribution proposes to resolve the Editors notes and update the conclusion of KI#5 based on the LS response from RAN.

1. Introduction/Discussion

According to RAN2 feedback in R2-2210827, and RAN3 feedback in R3-226083, using partial allowed NSSAI as in solution 29 would have both RAN2 and RAN3 impacts as below.

“6. The NG-RAN receives in solution 29 (but conceivably this would be needed for similar solutions) the partially allowed S-NSSAIs in addition to the Allowed NSSAI. Can the NG-RAN in principle trigger handover procedure to a supporting TAI of the partially allowed S-NSSAIs when it is possible to do so? this can happen while in connected mode or when the UE is engaged in transition from Idle to connected mode. The reason is to enable the support of the maximum number of S-NSSAIs in the Allowed and partly allowed S-NSSAIs lists.

[RAN2 answer]:

RAN2 impact is foreseen to support NG-RAN triggering handover procedure to a supporting TAI of the partially allowed S-NSSAIs and RAN2 understand the feasibility should also be evaluated by RAN3.

[RAN3’s answer]:

Depending on the resulting functionality, it it may be possible for NG-RAN to receive the S-NSSAIs included in the partially allowed NSSAI and trigger connected mode handover procedure to a suitable target cell.

Companies in RAN3 raised the following technical comments:

- To RAN3´s understanding, the solution is assumed to trigger handover not during but just after state transition from idle to connected procedure, which increases signalling.

- Potential issue on propagation of Allowed NSSAI via Xn interface for Xn based handover needs to be discussed further.

- Handing over a UE to a target cell/frequency,because that mobility target supports the partially allowed S-NSSAIs (i.e. without any active UP connection for the partially allowed S-NSSAIs) needs to take into account the radio performance.”

Therefore, it is proposed to implement only partly rejected S-NSSAI based option in the normative phase of Rel. 18.

Meanwhile, there is one Editor’s note related to how to implement the TA information regarding a rejected S-NSSAI. This can be decided in the normative phase.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change * * * *

8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

8.5	Conclusions for KI #5

The following principle is proposed to be the conclusion for normative work:

Partly rejected S-NSSAI:

-	AMF provides information related to the supported or not supported TA(s) regarding a rejected S-NSSAI in the RA to UE in the registration procedure and UE configuration update procedure if the UE indicates that it supports this feature. The UE shall be able to request a rejected S-NSSAI, by initiating a registration update procedure, in a supported TA based on the supported/not supported TA information associated with this S-NSSAI. The current concept of allowed NSSAI and uniform support of it in UE’s RA is not impacted by this feature based on indication of where in RA certain rejected S-NSSAIs are supported/not supported. In this approach, the Allowed NSSAI is still uniformly supported in the RA.

NOTE:	Whether the list of TAIs associated with the rejected S-NSSAI includes TAs where the S-NSSAI is supported or not supported is to be determined in the normative phase.

# * * * * End of changes * * * *