SA WG2 Meeting #154	S2-2210376

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	Update of conclusion on KI#1

Document for:	Approval

Agenda Item:	9.14.1 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes to update conclusion on KI#1.

Discussion

This contribution proposes to release editor's notes in KI#1 conclusion.

1)

Editor's note:	It is FFS which PCF is used and what SBI service is used for the PCF notification. Further trigger events are FFS.

Editor's note:	It is FFS whether the PCF and which PCF can interact with AMF to determine alternative S-NSSAI.

AMF has interface with AM-PCF and UE-PCF. UE-PCF is used to provide polices to the UE and such polices are transferred by using transparent container and AMF does not look into polices. AM-PCF is more appropriate as AM-PCF already handles information related with slices e.g. Allowed NSSAI, Target NSSAI. It is proposed to use AM-PCF and existing Npcf_AMPolicyControl_UpdateNotify service operation can be reused.

2)

Editor's note:	It is FFS whether to allow the alternative S-NSSAI to be outside of the UE's Subscribed S-NSSAI.

If operator wants to use non-subscribed S-NSSAI to temporarily replace S-NSSAI, this should be possible. This is similar as DNN replacement, which allows operator to use non-subscribed DNN. It is proposed to remove this EN as conclusion already allows to use non-subscribed S-NSSAI.

3)

Editor's note:	It is FFS whether this principle is in scope of SA5. It could be sufficient if the AMF selects the appropriate NSI at PDU Session establishment.

Editor's note:	Bullet 2) needs to be further evaluated as if the issue is C-plane overload of NSI this is not a good solution as the load of C-plane is increased by the need to handle migrations. If it is a UP overload then the question is whether the load balancing weights needs to take into account UP load also. In general, a proactive approach in assigning the PDU sessions to least loaded UP NSI would be better than this reactive approach. The need of this solution in normative phase is FFS.

These two editor's notes are related with solution 15. In our view, the target scenario is valid and proposes to keep the conclusion and remove these editor's notes.

4)

Editor's note:	It is FFS whether it is suitable to have a PDU Session to be associated with multiple S-NSSAIs and to use different S-NSSAI towards the UE compared to what is used within the network. It is FFS whether the same cannot be achieved by repartitioning resources where needed hence this solution may not be needed in normative phase.

This is related with solution 1. This solution proposes to replace S-NSSAI without any impact on the UE. However, if S-NSSAI used in the UE and network is different, some features based on slices, e.g. S-NSSAI based congestion control, NSSAA, NSAG may be impacted. It is proposed to keep the solution but clarify that mapping information should be provided to the UE.

5)

Editor's note:	It is FFS whether the existing Mapping Of Allowed NSSAI can be re-used, or an alternative IE (e.g. Replaced NSSAI) has to be specified.

It is proposed to reuse existing Mapping Of Allowed NSSAI. No need to define new IE.

6)

Editor's note:	If there are existing PDU Sessions on the old S-NSSAI, it is FFS whether the AMF includes in the Allowed NSSAI the alternative S-NSSAI and temporarily the old S-NSSAI to ensure that first the PDU Session transfer to the alternative S-NSSAI is performed before releasing the PDU Session on the old S-NSSAI. Alternatively, the AMF provides information in MM message to the UE such that the UE does not locally release the old S-NSSAI as well as not establishing new PDU Sessions using the old S-NSSAI.

If AMF provides alternative S-NSSAI to the UE by using Mapping Of Allowed NSSAI, the old S-NSSAI is still exist in the UE's Allowed NSSAI so the PDU session can be kept if network does not release it. (Note that URSP evaluation is done based on S-NSSAI value of HPLMN.) There is no additional mechanism is needed. It is proposed to remove this editor's note.

7)

Editor's note:	It is FFS whether UE can be triggered to re-evaluate the URSP rules.

Mandating URSP rule re-evaluation was discussed in previous releases but it was not agreed. This should be kept as an UE implementation.

8)

Editor's note:	It is FFS whether (1) the AMF rejects the UE's request for PDU Session establishment with an indication to use an alternative S-NSSAI; or (2) the AMF proceeds with the PDU Session establishment to the SMF, the AMF indicates the alternative S-NSSAI to the SMF and the SMF proceeds with the PDU Session establishment on the alternative S-NSSAI. Alternatively, the AMF provides information in MM message to the UE such that the UE does not locally release the old S-NSSAI as well as not establishing new PDU Sessions using the old S-NSSAI i.e. the (1) is sufficient to handle the error case.

This editor's note is related with PDU session establishment procedure. However, according to bullet 4) a), AMF will provide alternative S-NSSAI by triggering UCU procedure. Therefore, normally, the UE should use alternative S-NSSAI and scenario b) will not happen. It is proposed to remove bullet b). Note that AMF can reject PDU Session Establishment request when network resource is insufficient with cause indicating "Cause #67 – Insufficient resources for specific slice and DNN". If needed, AMF can use the existing procedure.

9)

Editor's note:	It is FFS whether the AMF notifies to the UE after PDU Session has been established or before PDU Session has been established, e.g. during registration or UCU. It is FFS how to handle the home routed case.

Editor's note:	It is FFS whether the SMF sends to the UE (1) the alternative S-NSSAI or (2) only an indication that the PDU Session can be re-established on another S-NSSAI or (3) neither is needed.

As described in bullet 4) a), AMF will provide alternative S-NSSAI by triggering UCU procedure. Then AMF does not need to inform SMF of alternative S-NSSAI.

For home routed PDU session, AMF of serving PLMN does not have load information of HPLMN and HPLMN does not know allowed NSSAI information. Considering such aspects, it is proposed to re-use existing SSC mode procedure, i.e. SMF either sends PDU Session Release or PDU Session Modification Command with cause PDU Session re-establishment to the same DN is required. There is no need to provide slice replacement information but provides S-NSSAI based back-off timer to the UE. Then UE cannot establish PDU Session using the original S-NSSAI. Based on existing URSP rule, the UE will select other network slice and establish PDU Session.

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change * * * *

8.1	Conclusions for KI#1

The following principles are concluded for the normative work.

NOTE:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM) or based on (AM-)PCF notification.

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may interact with the (AM-)PCF to determine the alternative S-NSSAI. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. The Subscribed S-NSSAIs of the UE are not updated for the purpose of KI#1.

2)	In case of a same S-NSSAI associated with multiple NSIs and the change of the NSI is required, when an existing PDU session is decided as to be migrated to another NSI, the SMF performs PDU Session re-establishment by using the mechanism of SSC mode#2 or SSC mode#3. When a PDU session establishment request from the UE arrives at the AMF, the AMF performs NSI re-selection i.e. removes from the UE context the old NSI and the AMF selects another NSI, e.g. as described in solution #15.

3)	In cases where the old S-NSSAI and the alternative S-NSSAI are associated with the same NSI, and the IP address is retained at same UPF and SMF does not change, the PDU session is associated with multiple S-NSSAIs in the network (i.e. NG-RAN, AMF, SMF, UPF). During PDU session establishment or HO procedure, the AMF notifies the alternative S-NSSAI to SMF and the SMF provides the alternative S-NSSAI to the NG-RAN, as described in solution #1. The AMF provides alternative S-NSSAI information by providing mapped values by updating Allowed NSSAI.

4)	In cases where the old S-NSSAI is replaced by an alternative S-NSSAI deployed on a different NSIs, the following principles apply:

a)	The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet. The AMF provides information that the alternative S-NSSAI is associated with a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case; and independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs.

-

c)	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI:

-	The AMF notifies the current SMF, e.g. by triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be relocated to an alternative S-NSSAI.

-	The SMF sends to the UE either PDU Session Modification Command (if the PDU Session is of SSC mode 3) or PDU Session Release/Modification Command (if the PDU Session is of SSC mode 1 or 2) and indicates to the UE that the PDU Session can be re-established.

-	For home routed PDU Session, the SMF sends to the UE either PDU Session Modification Command (if the PDU Session is of SSC mode 3) or PDU Session Release/Modification Command (if the PDU Session is of SSC mode 1 or 2) and indicates to the UE that the PDU Session can be re-established. In addition, the SMF provides back-off timer for original S-NSSAI to the UE so that UE does not use original S-NSSAI.

--

* * * * End of Changes * * * *