3GPP TSG-WG SA2 Meeting #154	S2-2210644

# Toulouse, France, 14 – 18 November 2022		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0078 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF obtaining DNAI associated to EAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the agreed conclusion, AF obtaining DNAI associated to EAS shall be investigated in normative phase. This CR introduces solution according to agreed conclution of TR 23.700-48. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedure of solution to AF obtaining DNAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed conclusion on solution to KI#7 of TR 23.700-48 will not be included in normative work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 3799, TS 23.502 CR 3631 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new text) * * * *

## 6.x	AF obtaining DNAI associated to EAS

### 6.x.1	General

AF might need to obtain DNAI(s) associated to EAS(s), e.g. for edge relocation. However, there exists a scenario where the AF has not been provided beforehand with DNAI information and needs to know the DNAI(s) associated with the EAS(s) in the EHE under its responsibility.

### 6.x.2	Procedure

Figure 6.x.2-1: AF obtaining DNAI(s) of EAS(s)

It is assumed that NEF/UDR is configured by OAM with the mapping information between EAS IP/IP range/FQDN and DNAI.

1.	AF sends Nnef_EASExposure Request message to subscribe to DNAI associated with the required EAS(s), including EAS IP(s)/IP range(s) and/or FQDN(s), and optionally DNN, S-NSSAI and geographical area for each EAS. The message also includes report type (one-time report or continuous report) and Event Reporting Information (defined in Table 4.15.1-1 of TS 23.502 [3]) If AF unsubscribes the DNAI(s), it sends Nnef_EASExposure_Unsubscribe Request with Subscription Correlation ID.

NEF checks whether the AF is authorized to perform the subscription. If NEF does not know the mapping between the required EAS IP/IP range/FQDN and DNAI (e.g. not configured by OAM), steps 2-3 shall be performed. Otherwise steps 2-3 are skipped.

2.	NEF sends Nudr_DM_Subscribe Request to subscribe the DNAI(s), including information received from AF. If AF requests to unsubscribe the DNAI(s), NEF sends Nndr_DM_Unsubscribe Request with Subscription Correlation ID.

3.	UDR responds to the NEF with Nudr_DM_Subscribe Response message, optionally including the DNAI(s) if immediate reporting flag is included in step 2, or responds Nndr_DM_Unsubscribe Response with result indication.

4.	NEF responds to the AF with Nnef_EASExposure_Subscribe Response message, optionally including the DNAI(s) if immediate reporting flag is included in step 1, or responds Nnef_EASExposure_Unsubscribe Response with result indication.

5.	Once the mapping information between EAS IP/IP range/FQDN and DNAI is changed or removed, and report type is continuous report,

-	If UDR is subscribed the DNAI(s) by NEF, UDR sends the updated DNAI(s) to NEF with Nudr_DM_Notify, and NEF sends the updated DNAI(s) to AF with Nnef_EASExposure Notify message.

-	If NEF is subscribed the DNAI(s) by AF, and UDR is not subscribed the DNAI(s) by NEF, NEF sends the updated DNAI(s) to AF with Nnef_EASExposure Notify message.

# * * * * End of changes * * * *