3GPP TSG-WG SA2 Meeting #154 	S2-2210765

Toulouse, France, November 14 – 18, 2022	(revision of S2-2209298)

Source:	NEC

Title:	FS_eNS_Ph3 KI#1 conclusion update

Document for:	Approval

Agenda Item:	9.14.1

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This contribution addresses the open issues in the K#1 conclusion.

# 1. Introduction/Discussion

The following considerations and updates are applied to the conclusion:

1). The Editor's note: It is FFS which PCF is used and what SBI service is used for the PCF notification. Further trigger events are FFS. in bullet point 0 is deleted as the interaction between the AMF and the PCF, which may be based on an existing or new service, is to be resolved at the normative phase.

2). Which entity has a knowledge about a replacement i.e. alternative S-NSSAI? The most obvious source for an alternative S-NSSAI is the URSP rules for the UE within the PCF. The URSP rules have the S-NSSAI selection descriptor which defines which Apps what S-NSSAI can use for services. Bullet point 1 is updated in this respect and the Editor's note: It is FFS whether the PCF and which PCF can interact with AMF to determine alternative S-NSSAI. is deleted

3). Load balancing between NSIs in bullet point 2 – We believe we do not need this alternative as NSI is created, managed and terminated in the SA#5 domain and any attempt to do the same within the SA2 domain is likely to create conflicts, That is why the bullet point 2 is removed.

4).The Editor's note: It is FFS whether it is suitable to have a PDU Session to be associated with multiple S-NSSAIs and to use different S-NSSAI towards the UE compared to what is used within the network. It is FFS whether the same cannot be achieved by repartitioning resources where needed hence this solution may not be needed in normative phase. in bullet point 3 is deleted along with the bullet point itself. We believe the resource repartitioning is much simpler solution compared with the PDU session on multiple S-NSSAI management.

5). If the AMF notified the alternative S-NSSAI to the UE, the UE should not establish new PDU Sessions using the old S-NSSAI. Otherwise, the AMF indicates the alternative S-NSSAI to the SMF and the SMF proceeds with the PDU Session establishment on the alternative S-NSSAI.

6) The AMF notifies the alternative S-NSSAI to the UE before it notifies it to the SMF.

# 2. Text Proposal

Based on the above considerations, it is proposed to agree the following updates to the TR 23.700-41

# * * * * First change * * * *

# 8	Conclusions

.

## 8.1	Conclusions for KI#1

The following principles are concluded for the normative work.

NOTE:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM) or based on PCF notification. The interaction between the AMF and the PCF may be based on an existing or on a new service which is to be decided in the normative phase.

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may interact with the PCF to make use of the UE’s URSP rules in order to determine the alternative S-NSSAI. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. The Subscribed S-NSSAIs of the UE are not updated for the purpose of KI#1.

2)	In cases where the old S-NSSAI is replaced by an alternative S-NSSAI deployed on a different NSIs, the following principles apply:

a)	The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet. The AMF provides information that the alternative S-NSSAI is associated with a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case; and independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs.

-

b)	For a new PDU Session establishment request from the UE on the old S-NSSAI:

-	If the AMF notified the alternative S-NSSAI to the UE, the UE shall not establish new PDU Sessions using the old S-NSSAI. Otherwise, the AMF indicates the alternative S-NSSAI to the SMF and the SMF proceeds with the PDU Session establishment on the alternative S-NSSAI.

c)	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI:

-	The AMF notifies the current SMF, e.g. by triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be relocated to an alternative S-NSSAI and optionally indicates the alternative S-NSSAI. For the home routed case the old and the alternative S-NSSAI mapping for the HPLMN is provided to the hSMF.

-	The AMF notifies the alternative S-NSSAI to the UE before it notifies it to the SMF.

-	The SMF sends to the UE either PDU Session Modification Command (if the PDU Session is of SSC mode 3) or PDU Session Release/Modification Command (if the PDU Session is of SSC mode 1 or 2) and indicates to the UE that the PDU Session can be re-established on another S-NSSAI. If AMF indicated the alternative S-NSSAI the SMF provides the alternative S-NSSAI to the UE.

-	the UE triggers a new PDU Session Establishment procedure to establish a PDU Session on the alternative S-NSSAI.

-

--

# * * * * End of changes * * * *