3GPP TSG-SA2 Meeting #154	S2-2210926

# Toulouse, France, November 14 – 18, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF-assisted application detection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusion for KI#2 NWDAF-assisted application detection in TR23.700-81 agreed to consider the solution #9 as basis for normative work 8.2 Key Issue #2: NWDAF-assisted application detection It is concluded to focus on the use case of PFD updates for known applications in Solution#9, which is selected as baseline for the normative work. The known application means an application for which the application ID is already known by the 5GC and can be referenced within PCC rules and for which PFD information (may not be the latest) is already available. It is concluded, that the NEF(PFDF) as a consumer can request NWDAF to get a new Analytics ID for application detection, following the detail described in solution#9. This CR is to propose text for related new anlytics in TS 23.288 for support of application detection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new analytics for application detection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Application detection feature is missing in eNA3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_eNA_ph3 and the WI code for the work is not assigned yet & will replace WI code DUMMY when available |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change (all new text) * * *

6.x	Application detection analytics

6.x.1	Description

The NWDAF can provides analytics on application traffic based on user consent. The NWDAF provides analytics including PFD information for known applications by extracting network traffic signature and inferring application id. The known application means an application for which the application ID is already known by the 5GC and can be referenced within PCC rules and for which PFD information (may not be the latest) is already available, which allows the 5GC to perform the "application detection" analytics in order to get the latest PFD information.

The NWDAF can collect measurement of an application to extract statistical characteristics, and collect payload of packets to extract payload characteristics (such as domain name contained in the payload). The analytics can be used by the NEF to store the captured application characteristics as a PFD, and the PFD is used by SMF and UPF to detect a known application defined as TS 23.502 [3].

The consumer informs the NWDAF about the target applications through the "Applications Characteristics". The "Application Characteristics" are specific well know KPIs between the traffic characteristics i.e. the relations that will identify each of the application.

In case the NWDAF receives "Application Characteristics" filters in the subscription/request of the analytics, the NWDAF only retrieve the applications matching the filters.

The service consumer may be an NF (e.g. NEF)

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Application Detection";

-	Target of Analytics Reporting: a single UE (SUPI ), or a group of UEs (an Internal Group ID that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Analytics target period is requested);

-	Analytics Filter Information:

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target of Analytics Reporting is set to "any UE", optional otherwise);

-	Optionally, a list of analytics subsets that are requested among those specified in clause 6.6.3;

-	Optionally, a preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.6.3);

-	Optionally, preferred order of results for the list of Network Performance information:

-	ordering criterion: "number of UEs", "communication performance" or "mobility performance";

-	order: ascending or descending;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics information (see clause 6.6.3) in order to be notified by the NWDAF;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested; and

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.x.3.

6.x.2	Input Data

NWDAF collects QoS flow related data from SMF for a specific S-NSSAI, DNN, and UE. The detailed data are described in Table 6.x.2-1.

Table 6.x.2-1: Input data to detect known application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF / UPF | UE ID for the UE that uses the application. |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Start/end time | UPF | Start and end time of traffic detection. |
| Data volume | UPF | Measured data traffic volume (per UL/DL). |
| Data duration | UPF | Measured average data traffic (packets) duration (per UL/DL). |
| QoS flow Bit Rate | UPF | The observed bit rate (per UL/DL). |
| Packet transmission | UPF | The observed average number of packet transmission (per UL/DL). |
| UL/DL Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| IP 3-tuple | UPF | to Identify a service flow of the UE that uses the application. |
| Packet's URL | UPF | URL is extracted from the inspected user plane packets. |
| Packet's Domain Name | SMF/UPF | The domain name is extracted from the inspected user plane packets in a QoS flow. |
| Size of packets | SMF/UPF | Average size of packets. |
| PFD Information | NEF(PFDF) | PFD Information stored in the NRF and retrieved by NEF, as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name | NEF(PFDF) | A Domain name matching criteria and information about applicable protocol(s). |

NOTE 1:	Extensive reporting of all traffic flows may conflict with requirement to avoid extra UPF load in the key issue. An NWDAF may subscribe only for reporting for some UEs to limit the load.

NOTE 2:	The current Packet delay detection and reporting in R17at UPF is only supported for URLLC.

NOTE 3:	How NWDAF collects the UL/DL Packet Delay from UPF (e.g. subscription via SMF) will be defined in FS_UPEAS.

6.x.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.x.3-1.

The output analysis can be used to provision new PFDs for known applications.

Table 6.9.3-1: Application detection analytics for known applications

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |
| URL | the significant parts of the URL to be matched, e.g. host name defined in. |
| Domain name information | a Domain name matching criteria and information about applicable protocol(s). |

6.x.4	Procedures

The procedure depicted in 6.x.4-1 shows known application detection analytics.

Figure 6.x.4-1: A procedure to extract application characteristics

1.	The NEF(PFDF) as consumer NF subscribes to the NWDAF to request analytics for application detection (or via the NEF in untrusted domain). This subscription maybe is triggered by local configuration or OAM. The Analytics Filter Information may optionally include the UE ID, S-NSSAI and/or DNN, Application ID.

2.	NWDAF performs user consent check with UDM to determine if the analytics procedure is allowed or not.

3.	The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF).

4.	The NWDAF collects session related information from the UPF about URL, Domain name part, and IP 3-tuples of packets corresponding to Application ID from the SDF not matching installed PDRs.

5.	The NWDAF derives PFD analytics.

6.	The NWDAF notifies the analytics consumer NF with observed PFD Information. Based on the observed PFD information provided by the NWDAF, the consumer determines to create/update/delete PFDs information and if needed send request to the NEF(PFDF) by invoking the Nnef_PFDManagement_Create/Update/Delete service containing Application Identifier and one or more sets of PFDs, and step 1-5 defined in clause 4.18.2 in TS 23.502 [3] are executed.

7.	The NEF (PFDF) fetches the PFD information currently in use from UDR.

8.	The NEF (PFDF) compares the PDF information from UDR with PFD information from the NWDAF to determine whether to create/update/delete PFD information corresponding to the application ID.

9.	The NEF (PFDF) invokes the Nudr_DM_Create/Update/Delete (Application Identifier, one or more sets of PFDs) to the UDR to create/update/delete PFD information corresponding to the application ID, i.e. steps 3-5 defined in clause 4.18.2 of TS 23.502 [3] are executed.

* * *End of Changes * * *