SA WG2 Meeting #155	S2-2302509

# 20-24 February 2023, Athens, Greece	(was S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0913 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN Route Selection Policy - 23.503 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-02-20 |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: |  | It has been concluded in clause 8.4 of TR 23.700-88 that a new UE policy, PIN Route Selection Policy may be generated by PCF for PDU Session selection in PEGC(s). The AF may provide necessary parameters to 5GC for the generation of PRSP. Enable route selection for PIN scenarios as per Option 1 discussed in S2-2302507. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause for PIN Route Selection Policy information is included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Traffic differentiation and route selection of the relayed PINE traffic via PEGC may no possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 6.6.X	PIN Route Selection Policy information

#### 6.6.X.1	Structure Description

The PIN Route Selection Policy (PRSP) includes a prioritized list of PRSP rules.

Table 6.6.X.1-1: PIN Route Selection Policy

| Information name | Description | Category | PCF permitted to modify in a PRSP | Scope |
| --- | --- | --- | --- | --- |
| PRSP rules | 1 or more PRSP rules as specified in table 6.6.X.1-2 | Optional | Yes | UE context |

The structure of the PRSP rules is described in Table 6.6.X.1-2 and Table 6.6.X.1-3.

Table 6.6.X.1-2: PIN Route Selection Policy Rule

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |  |
| --- | --- | --- | --- | --- | --- |
| Rule Precedence | Determines the order the PRSP rule is enforced in the PEGC. | Mandatory (NOTE 1) | Yes | UE context |  |
| PIN Traffic descriptor | This part defines the Traffic descriptor components for the PRSP rule. | Mandatory |  |  |  |
| PIN ID | Matched against a PIN ID for a specific PIN configured in the PEGC. | Mandatory | Yes | UE context |  |
| List of PIN Route Selection Descriptors | A list of Route Selection Descriptors. The components of a Route Selection Descriptor are described in table 6.6.X.1-3. | Mandatory |  |  |  |
| NOTE 1: Rules in a PRSP shall have different precedence values. |  |  |  |  |

Table 6.6.X.1-3: PIN Route Selection Descriptor

| Information name | Description | Category | PCF permitted to modify in URSP | Scope |
| --- | --- | --- | --- | --- |
| PIN Route Selection Descriptor Precedence | Determines the order in which the PIN Route Selection Descriptors are to be applied. | Mandatory (NOTE 1) | Yes | UE context |
| PIN Route selection components | This part defines the PIN route selection components | Mandatory (NOTE 2) |  |  |
| SSC Mode Selection | One single value of SSC mode. (NOTE 3) | Optional | Yes | UE context |
| Network Slice Selection | Either a single value or a list of values of S-NSSAI(s). | Optional (NOTE 4) | Yes | UE context |
| DNN Selection | Either a single value or a list of values of DNN(s). | Optional | Yes | UE context |
| PDU Session Type Selection | One single value of PDU Session Type | Optional (NOTE 5) | Yes | UE context |
| NOTE 1: Every PIN Route Selection Descriptor in the list shall have a different precedence value. NOTE 2: At least one of the PIN route selection components shall be present. NOTE 3: The SSC Mode 3 shall only be used when the PDU Session Type is IP. NOTE 4: When the Subscription Information contains only one S-NSSAI in UDR, the PCF needs not provision the PEGC with S-NSSAI in the Network Slice Selection information. NOTE 5: When the PDU Session Type is "Ethernet" or "Unstructured", this component shall be present. |  |  |  |  |

Each PRSP rule contains a PIN Traffic descriptor (containing one or more components described in Table 6.6.X.1-2) that determines when the rule is applicable. A PRSP rule is determined to be applicable when every component in the Traffic descriptor matches the corresponding information from the PINE.

NOTE:	It is recommended to avoid listing more than two components in the PIN Traffic descriptor of a PRSP rule.

If a PRSP rule is provided that contains a PIN Traffic descriptor with two or more components, it is recommended to also provide PRSP rule(s) with lower precedence and a PIN Traffic descriptor with less components, in order to increase the likelihood of PRSP rule matching for a particular application.

#### 6.6.X.2	Configuration and Provision of PRSP

For PRSP rules, the PEGC shall support the provisioning (signalling) from the PCF. In addition, the PEGC may be also pre-configured with PRSP rules (e.g. by the PEMC, AF).

Only the PRSP rules provisioned (signalled) by the PCF are used by the PEGC, if both PRSP rules provisioned (signalled) by the PCF and pre-configured PRSP rules are present.

If the PCF receives application guidance for PRSP determination that may apply to a given PEGC from UDR as specified in clause 4.15.6.7 of TS 23.502 [3], the PCF may verify the requested parameters (which are described in clause 4.15.6.10 of TS 23.502 [3]) with regards to the existing PRSP rules and (re-)compose the PRSP rules for the PEGC.

#### 6.6.X.3	PEGC procedure for associating PINE traffic to PDU Sessions

For every newly detected PINE the PEGC evaluates the PRSP rules in the order of Rule Precedence and determines if the PINE traffic is matching the Traffic descriptor of any PRSP rule. The Traffic descriptor of the PRSP contains the PIN ID. When a PRSP rule is determined to be applicable for a given PINE traffic (see clause 6.6.X.1), the PEGC shall select a PIN Route Selection Descriptor within this PRSP rule in the order of the PIN Route Selection Descriptor Precedence.

When a valid PIN Route Selection Descriptor is found, the PEGC determines if there is an existing PDU Session that matches all components in the selected PIN Route Selection Descriptor. When a matching PDU Session exists, the PEGC associates the PINE traffic to the existing PDU Session, i.e. route the PINE traffic on this PDU Session.

If none of the existing PDU Sessions matches, the PEGC tries to establish a new PDU Session using the values specified by the selected PIN Route Selection Descriptor. If the PDU Session Establishment Request is accepted, the PEGC associates the PINE traffic to this new PDU Session.

# >>>>END OF CHANGES<<<<