SA WG2 Meeting #156E (e-meeting)	S2-2304682

April 17 – 21, 2023	(revision of S2-2300xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0988 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Not reporting URSP rule enforcement other then connection capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the conclusion in TR 23.700-85, it is agreed that the UE reports the URSP rule enforcement to 5GC via NAS message, and only the connection capability that enforced should be reported to 5GC. But for some of the URSP rule, it contains a Traffic descriptor (containing one or more components) and this URSP rule is determined to be applicable when every component in the Traffic descriptor matches the corresponding information from the application. So, if a URSP rule is enforced, and the traffic descriptor in URSP rule is the combination of Connection Capabilities and other TD, that the UE can only report the Connection Capabilities to 5GC. Other components in this enforced URSP rule is not reported. Just like the example in Annex A in TS 23.503, if the traffic descriptor in a URSP rule containing “Traffic Descriptor: Application descriptor=App1” and “Connection Capabilities="internet",”, after this URSP rule is enforced by UE, the UE only reports the Connection Capabilities="internet" to 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If a URSP rule in UE is enforced, and the traffic descriptor in this URSP rule containing one or more components, that the UE can only report the Connection Capabilities to 5GC and other components in this enforced URSP rule is not reported to 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will report all of the component in enforced URSP rule to network, but except the Connection Capability, some of other kinds of TD should not be reported to network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.6.2.4	UE reporting of URSP rule enforcement

For a UE indicating capability of support to report URSP rule enforcement to network, the PCF may indicate the UE to send reporting of URSP rule enforcement.

If the PCF indicates to the UE to send reporting of URSP rule enforcement, and if a UE URSP rule includes Connection Capabilities in the TD (see clause 6.6.2.1):

-	when a UE associates a newly detected application to a new PDU Session based on URSP evaluation result (see clause 6.6.2.3), the UE shall include in the PDU Session Establishment Request (see clause 4.3.2.2.1 of TS 23.502 [3]) the Connection Capabilities contained in the Traffic descriptor of the associated URSP rule.

-	when a UE associates a newly detected application to an existing PDU Session based on URSP evaluation result (see clause 6.6.2.3), the UE shall send a PDU Session Modification Request (see clause 4.3.3.2 of TS 23.502 [3]) including the Connection Capabilities contained in the Traffic descriptor of the associated URSP rule.

If the UE enforces several URSP rules for multiple applications, and these multiple applications' traffic are all associated to this PDU session, in order reduce the number of uplink NAS messages, the UE may include more than one URSP rule enforcement report in one PDU Session Modification Request to 5GC (see clause 4.3.3.2 of TS 23.502 [3]).

NOTE:	A rule with the "match-all" traffic descriptor cannot contain Connection Capabilities in the Traffic descriptor. The format and values of the Traffic descriptor component type identifier are defined in clause 5.2 of TS 24.526 [19].

The PCF receives reporting from URSP rule enforcement for a given UE via Policy Control Request Triggers (see clause 6.1.3.5).

Policy control decisions based on awareness of URSP rule enforcement are described in clause 6.1.6.

If one URSP rule in UE is enforced, and the Traffic Descriptor in this URSP rule containing one or more components, that the UE only reports the Connection Capabilities to 5GC, and other components in this enforced URSP rule is not reported to 5GC.

* * * End of Change * * *