3GPP TSG-WG SA2 Meeting #156-e	S2-2304859r02

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0346 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF service operation and procedure for GNSS assistance data tranfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support of following agreed functionality GNSS assistance data transfer between LMFs, a new service operation and corresponding procedure is needed. LMFs collected GNSS assistance data may update its NF profile in NRF by indicating the TAs of which corresponding GNSS assistance data exists, then the LMF can be discovered and its collected GNSS assistance data can be used by other LMFs not having corresponding GNSS assistance data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new service operation and corresponding procedure for GNSS assistance data tansfer between LMFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not support of GNSS assistance data or PRU measurement data tansfer between LMFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 8.3.x (new), 6.15.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3	LMF Services

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |
| Nlmf_DataTransfer | GNSSAssistanceData | Request/Response | LMF |

# * * * * Second change * * * *

8.3.X	Nlmf_DataTransfer service

8.3.X.1	General

Service description: This service enables an NF to obtain location assistance data collected by an LMF. The following are the key functionalities of this NF service.

-	Allow the consumer NF to request location assistance data transfer from an LMF which collected GNSS assistance data.

The events to trigger assistance data transfer are defined in clause 6.15.x.

8.3.X.2	Nlmf_DataTransfer_GNSSAssistanceData service operation

Service operation name: Nlmf_ DataTransfer_GNSSAssistanceData

Description: Provides GNSS assistance data to the consumer NF.

Input, Required: GNSS assistance data transfer requirement indication, TAI(s).

Input, Optional: None.

Output, Required: GNSS assistance data with associated TAI(s).

Output, Optional: None.

See clause 6.15.x for an example of usage of this service operation.

# * * * * Third change * * * *

6.15.x	GNSS assistance data transfer between LMFs

The following procedure may be used by an LMF to get GNSS assistance data from other LMFs which collected GNSS assistance data based on procedures in clause 6.15.1 or 6.15.2.

Figure 6.15.x-1: GNSSAssistance Data Transfer between LMFs

- An LMF may invoke a Nnrf_NFDiscovery Request service operation to an NRF. The service operation includes a GNSS assistance data indication and a list of TAs.

- The NRF selects one or more LMFs which collected GNSS assistance data based on the GNSS Assistance data indication and a list of TAs received in step 1 and sends a Nnrf_NFDiscovery Response to the LMF. The service operation includes the profiles of LMFs selected by the NRF.

- The LMF send a Nlmf_DataTransfer_GNSSAssistanceData Request service operation to one or more of the LMFs indicated at step 2. The service operation for each of the LMFs which collected GNSS assistance data includes a list of TAs and an indication of GNSS assistance data transfer requirement.

- LMFs received the request in step 3 return collected GNSS assistance data to the LMF by using a Nlmf_DataTransfer_GNSSAssistanceData Response service operation, which includes GNSS assistance data and associated TAI(s).

- After the LMF gets the GNSS assistance data, it can send the assistance data to UE via the procedures described in clauses 6.14 or 6.11.1 during a positioning session.

# * * * * Fourth change * * * *

5.11	Collection of GNSS assistance data

LMF may collect GNSS assistance data by reusing Data Collection from AF as described in clause 5.20a of TS 23.501 [18] and Naf_EventExposure service described in clause 5.2.19.2 of TS 23.502 [19]. The collection may be performed on a periodic basis.

LMF generates a mapping table of GNSS assistant data and applicable area (e.g. TA lists). LMF may generate multiple sets of GNSS assistance data for the same location area, and each of them corresponds to a certain location accuracy. The accuracy of GNSS assistance data may vary for different areas.

NOTE 1:	The multiple accuracy level of GNSS assistance data can be the reason the collection method, and /or calculation method.

After the collection, LMF may send the collected GNSS assistance data to UEs via the procedures described in clause 6.14.1 (Broadcast of Assistance Data by an LMF) or clause 6.11.1 (UE Assisted and UE Based Positioning Procedure) during a positioning session.

In the case of using Broadcast of Assistance Data by an LMF, the LMF may provision one or more of GNSS assistance data to RAN. LMF may also provision one or more cyphered key(s) to UE from the AMF, and each cyphered key corresponds to a certain GNSS assistance data.

LMF(s) collected GNSS assistance data may update its NF profile in NRF by indicating the TAs of which corresponding GNSS assistance data exists, then the LMF can be discovered and its collected GNSS assistance data can be used by other LMFs not having corresponding GNSS assistance data.

NOTE 2:	The GNSS assistance data with applicable areas don’t mean all parameters of the GNSS data only applies to the area. Both paramaters applicable to global area and paramaters (e.g., RTK related parameters) applicable to specific areas can be collected by the LMF and can be transferred to other LMFs based on the clasue of 6.15.x.

# * * * * End of changes * * * *