23GPP TSG-WG Meeting #156e	S2-2305223r04

# Electronic Meeting, April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4529 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF provides the Periodicity to SMF via PCC rules for resolving the EN for KI#8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China moible |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the discussion paper in S2-2305196, the idea of AF provide periodicity as individual parameter to NEF/PCF, and PCF generate the pcc rule for N6 jitter monitoring for specific QoS flow based on application id, periodicity, and local configuration. PCC rule sent to SMF to trigger the N6 jitter monitoring for specific qos flows as well as construct the TSCAI. Complete the UPF derive periodicity and reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PCF should send the Periodicity information received from the AF to SMF via PCC rules. Remove Editor’s note in 5.37.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The problem addressed in EN cannot be solved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR XXX TS 23.503 CRxxx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.37.8	UE power saving management

5.37.8.1	General

The following traffic assistance information may be provided by the CN to NG RAN in order to configure UE power saving management scheme for connected mode DRX:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

-	Indication of End of Data Burst.

5.37.8.2	Periodicity and N6 Jitter Information associated with Periodicity

At AF Session Establishment with QoS, the 5G System may be provided with Periodicity information for NG RAN to configure UE power management for connected mode DRX. The Periodicity information is provided by the AF to the PCF via NEF or directly to the PCF when the AF is trusted.

The PCF should send the Periodicity information received from the AF/NEF to SMF via PCC rules, with includeing an indication for SMF to request UPF to perform jitter measurements within PCC Rules, in accordance with the operator's local policies.

Upon reception of PCC rule, the SMF determines the TSCAI and forwards it to the NG-RAN. If the PCC rule includes an indication to perform jitter measurements, the SMF also sends the DL Periodicity to the UPF and requests the UPF to monitor and periodically report the Jitter information associated with the DL Periodicity using the N4 Session Modification procedure, see clause 5.8.5.11. It provides the DL Periodicity in the request to the UPF.

NOTE 1:	How the UPF derives the N6 jitter is implementation dependent.

NOTE 2:	How the UPF derives periodicity when not provided by the AF is implementation dependent.

Editor's note:	The method used to report the Periodicity information from the UPF to the SMF is FFS.

At reception of Jitter measurements from the UPF in the N4 Session Level Report, the SMF includes the Jitter information together with the associated Periodicity in the TSCAI and forwards it to the NG-RAN in an NGAP message, see clause 5.27.2.

NOTE 3:	In order to prevent frequent updates from the UPF, the UPF sends the N6 Jitter Measurement Report periodically or only when the jitter is larger than a threshold.

5.37.8.3	End of Data Burst Indication

An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g. to configure UE power management schemes like connected mode DRX.

The PCF may provision the Protocol Description within PCC Rules based on the information provided by the AF and/or the local operator policies.

SMF should request the UPF to detect the last PDU of the data burst and the mark the End of Data burst in the GTP-U header of the last PDU in downlink, according to the PCC rule and/or the local operator policies.

According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the Protocol Description and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst.

Editor's note:	Support of new RTP header extension for End of Data Burst indication by the application server depends on progress in SA WG4 5G_RTP WI.

NOTE:	There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.

# * * * * End of change * * * *