3GPP TSG-WG SA2 Meeting #157	S2-2306730

# Berlin, May 22 – 26, 2023		(revision of S2-2304711)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0121 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: |  | KI#1 EAS Re-discovery: Resolve ENs |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  | [Huawei, HiSilicon], China Unicom |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  | EDGE_Ph2 |  |  |  |  |  |  |  | Date: |  | 2023-05-12 |
| Category: |  | B |  |  |  |  |  |  |  | Release: |  | Rel-18 |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  | 1. Resolve ENs 1) EN1 about AF triggered EAS rediscovery Editor's note: AF triggered EAS Re-discovery is FFS. It has been approved in SA2#155 that AF interacts with VPLMN using Application Function influence on traffic routing procedure. This paper proposes that AF can trigger EAS Re-discovery via HPLMN (per AF influence procedure defined for non-roaming case) or via VPLMN (per AF influence procedure via VPLMN) -------- Note, there are 3 cases, i.e. UE mobility, AF interacting with HPLMN, AF interacting with VPLMN covered in this procedure. For easy reading, the following table summarizes which steps are performed in each 3 cases. steps; UE Mobility case; AF interacting with HPLMN; AF interacting with VPLMN. V-SMF unchange; V-SMF insertion; V-SMF change; V-SMF remove. 1a; Y. 1b; Y. 2; Conditional; Y; Conditional. 3a; Y. 3b; Y. 4; Y; Y; Y; Y. 5; Y; Y; Y; Y; Y. 6; Y; Y; Y; Y; Y. 7; Y. ----------- Proposal 1: Remove the EN1. 2) EN2 about EASDF removal Editor’s note: whether the above EASDF removal applies to intra PLMN mobility is FFS. For simplicity, this contribution proposes that in the case of V-SMF change with or without V-EASDF change, the source V-SMF always removes the DNS context on the (source) V-EASDF after SM context is transferred from source V-SMF to target V-SMF. Proposal 2: Remove EN2 and describe that in case of V-SMF change, the source V-SMF always removes the DNS context on the selected V-EASDF after SM context is transferred from source V-SMF to target V-SMF. 3) EN3 about interaction between H-SMF and UDM during inter-PLMN HO Editor’s note: It is FFS whether H-SMF needs to contact UDM to retrieve UE subscription during inter-PLMN handover. This EN is not EC specific issue and according to the description in TS 23.502, the H-SMF invokes Nudm_SDM_Get service to get subscription data from UDM by providing serving PLMN ID to get subscription data specific to the Serving PLMN. Therefore, during inter-PLMN handover, the H-SMF should provide the target serving PLMN ID to get subscription data specific to the target serving PLMN. Proposal 3: Remove the EN3 directly. 2. Editorial changes. SA2#157: This current EAS rediscovery procedure covers the UE mobility case, AF interacting with HPLMN case, and AF interacting with VPLMN case. Therefore the descriptions of this procedure need to be clearer to differentiate the various cases. In the UE mobility case and AF interacting with VPLMN case, the V-SMF triggers the PDU session modification procedure. In the AF interacting with HPLMN case, the H-SMF initiate the PDU session modification procedure. In addition, the EAS (re-)discovery using IP replacement mechanism is added as an optional EAS rediscovery procedure corresponding to the clause 6.7.2.5. |
| Summary of change: |  |  | See the above proposals. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | ENs cannot be resolved. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 6.7.3, 6.7.3.1, 6.7.3.2 |  |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | X |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | X |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  | SA2#157 (highlight the change): - Differentiate the cases (i.e. UE mobility, AF interacting with HPLMN, AF interacting with VPLMN) with the clearer description; - Add the description about EAS re-discovery using IP replacement mechanism (optional). |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.7.3	EAS Re-discovery Procedure

6.7.3.1	General

The EAS re-discovery in VPLMN can be triggered due to UE mobility, AF interacting with HPLMN, or AF interacting with VPLMN.

6.7.3.2	EAS Re-discovery

Figure 6.7.3.2-1 shows the procedure of EAS re-discovery in VPLMN for HR roaming.

Figure 6.7.3.2-1: EAS rediscovery procedure for HR-SBO

1.	For UE mobility triggered EAS rediscovery, step 3 is skipped.

The procedure described in clause 4.23.7 (N2 based handover) or 4.23.11 (Xn based handover) of TS 23.502 [3] is performed by replacing I-SMF with V-SMF and SMF with H-SMF.

1a.	(V-SMF insertion case) The (target) V-SMF retrieves SM context from H-SMF using Nsmf_PDUSession_Context Request/Response.

1b.	(V-SMF change case) The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response.

In step 1b, the SM context includes Authorization Result for HR-SBO and VPLMN Specific Offloading Information if source and target V-SMFs belongs to same VPLMN.

The SM context also includes EAS information to be refreshed for EAS re-discovery. The EAS information corresponds to the old target DNAI if it has been inserted by the H-SMF (V-SMF insertion case) or the source V-SMF (V-SMF change case).

After the SM context is transfered to the (target) V-SMF by (source) V-SMF/H-SMF, the DNS context in the old (V-)EASDF is removed by the (source) V-SMF/H-SMF using Neasdf_DNSContext_Delete service.

In the case of V-SMF insertion or V-SMF change, when the AMF sends Nsmf_PDUSession_CreateSMContext Request to the target V-SMF, the AMF includes HR-SBO allowed indication if received from the UDM.

Step 1 is skipped if UE moves without V-SMF change.

Steps 1-6 are skipped if UE moves with V-SMF removal.

After the handover procedure, the (target) V-SMF may perform UL CL/BP and local PSA insertion/change/removal as described in clause 4.23.9 of TS 23.502 [3] by replacing I-SMF with V-SMF and SMF and H-SMF.

NOTE:	When there are other V-UPF(s) between UL CL/BP and H-UPF, the (target) V-SMF sets up user plane between this ULCL/BP and the existing V-UPF.

2.	The (target) V-SMF may (re-)select a new V-EASDF, e.g. based on the DNAI corresponding to the target UE location. The (target) V-SMF may invoke Neasdf_DNSContext_Create service to obtain the new V-EASDF address or the (target) V-SMF obtains the new V-EASDF address based on local configuration.

If a new V-EASDF is selected, the DNS context in the old (V-)EASDF is removed by the (source) V-SMF/H-SMF using Neasdf_DNSContext_Delete service.

If a new V-EASDF is selected without V-SMF change, the DNS context in the old V-EASDF is removed by the V-SMF using Neasdf_DNSContext_Delete service.

The (target) V-SMF configures the selected V-EASDF with the DNS handling rules.

3.	The (target) V-SMF invokes Nsmf_PDUSession_Update Request (EAS rediscovery indication, EAS information to be refreshed for EAS re-discovery (such as FQDN range and IP range corresponding to the old local part of DN) if available, the optional new V-EASDF address) to the H-SMF.

For the case of local PSA removal, the request message is sent to the H-SMF without including any DNAI.

The EAS information to be refreshed for EAS re-discovery may correspond to:

-	the new target DNAI in case of local PSA insertion.

-	the new target DNAI and old target DNAI in case of local PSA change.

-	the old target DNAI in case of local PSA removal or local PSA is unchanged.

3a.	For AF triggered EAS rediscovery via interacting with HPLMN, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the H-SMF via the H-PCF using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3]. In this case, step 4 is skipped.

3b.	For AF triggered EAS rediscovery via interacting with VPLMN, the AF may indicate the EAS rediscovery for the impacted applications via the V-NEF using the procedure described in clause 4.3.6 of TS 23.502 [3].

4.	(V-SMF insertion/change case or AF triggered EAS rediscovery via interacting with VPLMN) The (target) V-SMF invokes Nsmf_PDUSession_Update Request (EAS rediscovery indication, EAS information to be refreshed for EAS re-discovery (such as FQDN range and IP range corresponding to the old local part of DN) if available, the optional new V-EASDF address) to the H-SMF.

For the case of local PSA removal, the request message is sent to the H-SMF without including any DNAI.

The EAS information to be refreshed for EAS re-discovery may correspond to:

-	the new target DNAI in case of local PSA insertion.

-	the new target DNAI and old target DNAI in case of local PSA change.

-	the old target DNAI in case of local PSA removal.

5.	(V-SMF insertion/change case or AF triggered EAS rediscovery via interacting with VPLMN) The H-SMF sends the Nsmf_PDUSession_Update Resquest towards the (target) V-SMF including the PCO. The PCO includes EAS rediscovery indication (optional), impact field (optional), new V-EASDF address (optional).

NOTE:     If the PCO does not include the V-EASDF address and the V-SMF decides to perform the EAS (re-)discovery using IP replacement mechanism, the V-SMF configures the (new) UPF in VPLMN with the IP replacement information (i.e. DNS server IP address and port number of HPLMN, V-EASDF IP address and port number) as described in clause 6.7.2.5.

(AF triggered EAS rediscovery via interacting with HPLMN) The H-SMF initiates Nsmf_PDUSession_Update Request towards the (target) V-SMF including the PCO. The PCO includes EAS rediscovery indication (optional), impact field (optional).

6.	The V-SMF sends PDU Session Modification command including the PCO to the UE.

7.	(V-SMF removal case) The (H-)SMF selects an EASDF in HPLMN and creates a context for the PDU session in that EASDF and then initiates PDU Session Modification command including PCO to the UE.

The PCO includes EAS rediscovery indication (optional) and the impact field (optional).

# * * * * End of changes * * * *