3GPP TSG-WG SA2 Meeting #157	S2-2306908

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2306097)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0755 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Federated Learning among Multiple NWDAFs in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile, Lenovo, Ericsson, vivo], Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There seems no extra criteria, so the following EN can be removed: Editor's note: Additional criteria for FL server to discover FL clients from the NRF (e.g. data availability) is FFS. Particularly for maximum response time, in step 3, saying like the below: If the Client NWDAF is not able to complete the training within the maximum response time, the Client NWDAF notifies the Server NWDAF on the delay event before the maximum response time elapses. Therefore, the two ENs regarding maximum response time in step3 and 4 can be removed. The FL procedures should keep align with the lastest service definition of Nnwdaf_MLModelTraining and Nnwdaf_MLModelTrainingInfo. Rev4: 1. To align with the definition in clause 7.10.2 and clause 7.X.2, the “Available data requirement” and “Availability time requirement” should be changed to “data availability requirement” and “time availability requirement” respectively. 2. For the following EN, it seems no other parameters is needed in the preparation request, and the "Available data requirement" should be “data availability requirement”, which can be defined in clause 7.10.2 and clause 7.X.2. Editor's note: Other parameters in the preparation request and the definition of "Available data requirement" are FFS. 3. Step 7-9 are intended to check if the FL Client NWDAF(s) received in step 6 can meet the ML model training requirement. However, the FL Client NWDAF(s) received in step 6 may already meet some criteria (e.g., Analytic ID of the ML model required, FL capability Type (i.e. FL client), NF type(s) of data sources, Time Period of Interest and Interoperability Indicator, etc.), so step 7-9 are optional, and the FL Server NWDAF can select FL Client NWDAF(s) based on the information received in step 6 and other information received in step 9 (if available). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove 3 Editor’s Notes in clause 6.2C which already are resolved after SA2#155 meeting. Update the steps of Federated Learning with the latest service definition. Rev4: 1. Change the “Available data requirement” and “Availability time requirement” to “data availability requirement” and “time availability requirement” respectively. 2. Remove the following EN. Editor's note: Other parameters in the preparation request and the definition of "Available data requirement" are FFS. 3. Clarify that the FL Server NWDAF can select FL Client NWDAF(s) based on the information received in step 6 and other information received in step 9 (if available), and Figure 6.2C.2.1-1 is updated. 4. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality defined in Rel-18 eNA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2C	Federated Learning among Multiple NWDAFs

6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML model.

6.2C.2	Procedures

6.2C.2.1	Registration and Discovery procedure for Federated Learning

FL Server NWDAF

FL Client NWDAF 1

NRF

...

FL Client NWDAF N

1a. Nnrf_NFManagement_NFRegister Request (FL Server NWDAF profile)

1b. Nnrf_NFManagement_NFRegister Request (FL Client NWDAF profile)

2. Store FL Server NWDAF and FL Client NWDAF profile

3a. Nnrf_NFManagement_NFRegister Response

3b. Nnrf_NFManagement_NFRegister Response

4. Nnrf_NFDiscovery_Request Request

6. Nnrf_NFDiscovery_Request Response

(one or multiple FL Client NWDAF instances, i.e., FL Client NWDAF 1, ..., N)

5. Authorize NF Service Discovery

NWDAF Registration Discovery

7. FL preparation request

8. Decide to join or not

9. Response to FL Server NWDAF

10. Select FL Client NWDAF(s)

Figure 6.2C.2.1-1: Registration and Discovery procedure for Federated Learning

1-3.	NWDAF containing MTLF as FL Server NWDAF or FL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability type information (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

4-6.	NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as FL Server NWDAF, the MTLF first discovers and selects FL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in TS 23.288 [5], FL capability Type (i.e. FL server), check if the selected FL Server NWDAF can execute the Federated Learning procedure for the Analytics ID, Time Period of Interest, Service Area.

Once the FL Server NWDAF (the requested or the selected one) is determined, it discovers and selects other NWDAF(s) containing MTLF as FL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, NF type(s) of data sources from which the FL Client NWDAF is able to collect data for local model training, Time Period of Interest, Interoperability Indicator.

7.	FL Server NWDAF sends Federated Learning preparation request to the FL Client NWDAF(s) , using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the FL Client NWDAF(s) can meet the ML model training requirement (e.g. Analytics ID, ML Model Interoperability information, data availability requirement, time availability requirement, etc).

8.	FL Client NWDAF(s) checks if it can meet the ML model training requirement and/or can successfully download the model if the model information is provided in the request, and decides whether to join the Federated Learning process based on implementation. Example criteria used by a FL Client NWDAF may be based on its data availability and time availability, computation and communication capability, and ML Model Interoperability information.

9.	FL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify service operation or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate FL Server NWDAF if it will join the FL procedure.

10.	FL Server NWDAF selects FL Client NWDAF(s) based on the information received in step 6 and other information received in step 9 (if available).

END of CHANGES