3GPP TSG-WG SA2 Meeting #157	S2-2306927

# Berlin , May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.273 | CR |  | 0383 |  |  |  |  | rev |  |  | - |  |  | Current version: |  |  |  |  |  |  | 18.1.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  | UICC apps |  |  |  |  |  |  | ME |  |  | Radio Access Network |  |  |  |  |  |  | Core Network | X |  |
| Title: |  |  | LMF initiated User Plane Disconnection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  |  | China Telecom |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  | 5G_eLCS_Ph3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  | 2023-05-12 |  |  |  |  |  |
| Category: |  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-18 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |  |  |  |  |
| Reason for change: |  |  |  |  | In current specification, UE and LMF initiated User Plane Connection have been defined. In case the LMF decide to continue the LCS service using Control Plane the LMF may terminate the user plane connection. However, it is not clear how the LMF terminates the user plane connection. This CR is propesed to add this signalling procedure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  | It is unclear how to disconnect the user plane between the UE and LMF. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 6.18.X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  |  |  | X | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| affected: |  |  |  |  |  |  | X | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  |  |  |  | X | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.18.X  LMF initiated User Plane Disconnection

If LMF determines that the user plane positioning is not used, e.g. LMF determines to switch the position service to control plane. LMF may terminates the user plane connection. Figure 6.18.4-1 shows a procedure triggered by LMF to terminate a secure user plane connection between UE and LMF.

Figure 6.18.4-1: Terminating connection between UE and LMF initiated by LMF

- LMF decides to terminate the user plane between the LMF and UE and continue the position procedure via a control plane between the UE and LMF as described in clause 6.1.

- The LMF sends an Nlmf_Location_UPNotify includes indicates the LMF terminates the user plane connection.

- LMF sends an DL NAS TRANSPORT towards the UE and notify the secure position connection should be terminated.

- UE acknowledges the user plane disconnection by the transmission of DL NAS transport message to the AMF.

- AMF sends an Nlmf_Location_UPNotify toward the LMF to acknowledge the disconnection user plane. The AMF deletes the LCS-UP connection context if the AMF store the LCS-UP connection context.

- UE initiated the release the secure user plane connection.

# * * * * End of changes * * * *