3GPP TSG-WG SA2 Meeting #157	S2-2307154

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4661 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update overload control and resolve EN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Since the priority users and priority service may be different for the different UEs, the UEs can determine a random value based on the priority users and priority service for discontinuous coverage waiting to avoid the overload. So, the discontinuous coverage waiting time can be the latest maximum time offset for this PLMN and RAT type and a random value based on the priority users and priority service. The following EN has been resolved and can be removed. Editor's note: It is for CT WG1 to decide whether the maximum wait time will require a new IE or reuse existing IE used for MINT i.e. disaster return wait range information. SA WG2 specifications will be aligned based on CT WG1 decision. From the approved C1-232739 T-doc, the Registration wait range IE which is used for MINT contains a RANGE which starts from a MIMIMUM value to a MAXIMUM value and cannot be reused. A new Maximum NAS signalling wait time IE has been defined and is provided to the UE in the Configuration Update Command message or the Registration Accept message |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update overload control - resolve EN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.13.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.13.5	Overload control

The AMF and UE may only use the procedure defined in this clause if both the UE and AMF indicate "Discontinuous Coverage Supported", see clause 5.4.13.1.

In order to avoid a large number of UEs causing excessive signalling load on the network when re-gaining coverage after being out of coverage, the AMF may determine a maximum waiting time before UEs are allowed to initiate NAS signalling with the network, as described in this clause.

In this case, the AMF determines this maximum waiting time offset based on network configuration, priority users and priority service as specified in TS 23.122 [17] and TS 24.501 [47]. The AMF sends this maximum waiting time offset to individual UEs during the Registration procedure or UE Configuration Update procedure. The maximum waiting time offset is referred to the Discontinuous coverage maximum NAS signalling wait time IE in the in the REGISTRATION ACCEPT or CONFIGURATION UPDATE COMMAND message.

If the UE receives a maximum waiting time offset from the network in a Registration Accept or UE Configuration Update Command message, the UE shall replace any previously received maximum waiting time offset on the same RAT type and PLMN with this one.

When the UE knows a later time at which coverage will be lost and when the UE does not send Mobility Registration Update to the AMF in advance (see clause 5.4.13.1), the UE determines that discontinuous coverage waiting time is the latest maximum time offset for this PLMN and RAT type and a random value based on the priority users and priority service.

Upon returning in coverage after being out of coverage due to discontinuous coverage, the UE sets the discontinuous coverage wait timer value to a random value up to and including the latest maximum waiting time for this PLMN and RAT type, and starts this timer. The UE shall not initiate any NAS signalling on that RAT Type and PLMN while the discontinuous coverage wait timer is running.

The UE shall stop the discontinuous coverage wait timer and initiate NAS signalling if the UE receives paging message, has pending emergency services or when UE enters a TAI outside the registration area.

# * * * * End of changes * * * *