SA WG2 Meeting #157	S2-2307235

# Berlin, Germany, May 22nd – 26th, 2023           	  			  			       		 (revision of S2-230)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.216 | CR | 0376 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cancelling AMF location at 5G-SRVCC from NG-RAN to 3GPP UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SRVCC, TEI17 |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | After 5G-SRVCC from NG-RAN to 3GPP UTRAN the UE is no more served by the AMF that can no more deliver services to the UE including MT service. Thus 5G-SRVCC from NG-RAN to 3GPP UTRAN should trigger UDM to trigger deregistration notification to AMF for 3GPP access BUT this is not explicitly written in 23.216 Keeping the AMF registration is useless: AMF/5GC cannot provide any service, even MT -SMS or MT-Loc attempts via AMF/5GS would fail if they are sent to the AMF (that actually no more serves the UE). if the UDM deregistration after 5G-SRVCC would indeed be removed, there should be additional definitions related to the AMF behaviour after completing 5G-SRVCC – now it is “clearly” stated that PDUsessions shall be removed, but if there is additionally no UDM deregistration expected, then also the AMF actions would need to be additionally specified e.g. on 29.518, otherwise the AMF may e.g. page the UE on MT SMS, and such paging would be doomed to fail, causing possibly extra traffic on radio interface due to absolutely unnecessary paging attempts |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G-SRVCC from NG-RAN to 3GPP UTRAN triggers the UDM to initiate a Nudm_UECM_DeregistrationNotification (see clause 23.502 clause 5.2.3.2.2) to the AMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications that may lead to failed MT LCS procedures and useless attempts for MT-SMS, |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.5.4	5G-SRVCC from NG-RAN to 3GPP UTRAN procedure

Depicted in Figure 6.5.4-1 is a call flow for 5G-SRVCC from NG-RAN to 3GPP UTRAN.

Figure 6.5.4-1: 5G-SRVCC from NG-RAN to 3GPP UTRAN call flow

1.	UE establishes the PDU session for IMS.

2.	5G-SRVCC HO is triggered by NG-RAN.

3.	NG-RAN sends a Handover Required (Target ID, generic Source to Target Transparent Container, 5G-SRVCC HO indication) message to the source AMF. The Target ID is the UTRAN RNC-ID. 5G-SRVCC HO Indication indicates to AMF that this if for 5G-SRVCC. The Generic Source to Target Transparent Container is the Source RNC to Target RNC Transparent container.

4.	AMF determines the HO is used for 5G-SRVCC by the 5G-SRVCC HO Indication. AMF selects an MME_SRVCC that can have Sv connection to the MSC SERVER/MSC according to the target RNC ID which is included in the Target ID.

NOTE 1:	The MME_SRVCC selection can be realised through operator's configuration.

5.	AMF sends the forward relocation request (IMSI, Target ID, STN-SR, C-MSISDN, MM Context, Generic Source to Target Transparent Container, 5G-SRVCC HO Indication, Supported Codec IE, MS ClassMark 2, Emergency Indication, Equipment Identifier) to MME_SRVCC. The Emergency Indication and the equipment identifier are included if the ongoing session is emergency session. Authenticated IMSI and C MSISDN are also included in this case, if available.

6.	MME_SRVCC initiates the PS-CS handover procedure towards MSC Server. Steps 5 to 13 as specified in Figure 6.2.2.1-1 (referenced by clause 6.2.2.1A SRVCC from E-UTRAN to UTRAN without PS HO) are performed for the PS-CS handover procedure. This triggers the UDM to initiate a Nudm_UECM_DeregistrationNotification (see clause 23.502 clause 5.2.3.2.2) to the AMF.

7.	MME_SRVCC receives the response message from MSC server after HO preparation is completed. MME_SRVCC sends the Forward Relocation Response message (Target to Source Transparent Container, MSC Server Identity) to AMF.

8.	AMF sends the HO command to NG-RAN.

9.	NG-RAN sends a HO command to the UE. UE detects the 5G-SRVCC HO.

10.	UE tunes to the target UTRAN cell.

11.	Handover Detection at the target RNS occurs, then the target RNS sends Handover Detection message to the target MSC SERVER/MSC.

12.	The UE sends a Handover Complete message via the target RNS to the target MSC SERVER. Steps 19 to 21as specified in Figure 6.2.2.1-1 (referenced by clause 6.2.2.1A SRVCC from E-UTRAN to UTRAN without PS HO) are performed for this handover complete procedure. At this stage, the target MSC SERVER/MSC can send/receive voice data.

13.	MSC SERVER sends MME_SRVCC the SRVCC PS to CS completion.

14.	MME_SRVCC forwards the Forward Relocation completion message which includes the information received in step 13 to AMF.

15.	AMF forwards the Forward Relocation Complete ACK message to MME_SRVCC. AMF releases the UE context related to MME_SRVCC.

16.	MME_SRVCC forwards the PS to CS Complete ACK message to MSC server. MME_SRVCC removes stored UE context. After receives the message, MSC server removes the UE context related to the MME_SRVCC.

17.	AMF performs the PDU session release procedure for all the PDU session(s) which is described in TS 23.502 [45] indicating that the release is due to PS to CS handover for 5G SRVCC.

18.	For an emergency services session after handover is complete, the source AMF or the MSC Server may respectively invoke the Namf_Location_EventNotify service operation or send a Subscriber Location Report, towards a GMLC associated with the source or target side, respectively, carrying the identity of the MSC Server, as defined respectively in TS 23.273 [50], TS 23.502 [45] or TS 23.271 [29], to support location continuity.

NOTE 2:	Some configuration of the choice between a source AMF versus MSC Server update to GMLC needs to ensure that a single update occurs from one of these entities.

19.	The MSC server executes steps 17f and 17 g of Figure 6.2.2.2-1

The IMS service control is described in TS 23.292 [13] when UE accesses in UTRAN cell due to 5G-SRVCC.

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES