3GPP TSG-WG SA2 Meeting #157	S2-2307338

# Berlin, May 22-26, 2023		(revision of S2-2304487)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4326 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solve the EN about handling of the PEMC in 5GC in relation with PIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei], [HiSilicon], Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is Editor’s Note about handling of PEMC in 5GC in relation with PIN. SA2 has concluded that there is no PEMC specific subscription in 5GC. Therefore, 5GC is not aware of PEMC role. In this case, the traffic between PEMC and PEGC will be transmitted via application layer and user plane over N6. When PEMC or PINE is a UE, it is not clear how to handle the PIN traffic. It is benefit to add description in Annex |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the Editor’s Note without any changes in the TS. Add description in the Annex P1 that When PINE or PEMC is a UE, it can establish the PDU session as normal UE with the URSP rules (no PIN ID in the traffic descriptor) provisioned by PCF as specified in TS 23.502 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Whether and how to handle PEMC in 5GC is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.1, Annex P1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.44.3	Session management enhancement for PIN service support

5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC. One PIN may be served by more than one PDU sessions in the PEGC.

Editor's note:	How and whether to handle the case where PINs share a PDU session and local switching is FFS.

Editor's note:	One PIN served by more than one PDU sessions in PEGC is FFS.

# * * * * Second change * * * *

Annex P (informative):
Personal IoT Networks

P.1	PIN Reference Architecture

Figure P.1-1 shows the logical PIN reference architecture.

Figure P.1-1: PIN reference architecture

A Personal IoT Network (PIN) in 5GC consists of one or more devices providing gateway/routing functionality known as the PIN Element with Gateway Capability (PEGC), and one or more devices providing PIN management functionality known as the PIN Element with Management Capability (PEMC) to manage the Personal IoT Network; and device(s) called the PIN Elements (PINE). A PINE can be a non-3GPP device.

The PIN can also have a PIN Application Server that includes an AF functionality. The AF can be deployed by mobile operator or by an authorized third party. When the AF is deployed by third party, the interworking with 5GS is performed via the NEF.

The PEMC and PEGC communicates with the PIN Application Server at the application layer over the user plane. The PEGC and PEMC can communicate with each other via direct communication using 3GPP access (e.g. PC5) or non-3GPP access (e.g. WiFi, BT) or via a PDU Session in the 5GS.

When PINE or PEMC is a UE, it can establish the PDU session as normal UE with the URSP rules (no PIN ID in the traffic descriptor) provisioned by PCF as specified in TS 23.502[3].

# * * * * End of changes * * * *