3GPP TSG-WG SA2 Meeting #158	S2-2308677

# Gothenburg, Sweden, 21-25 August 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4314 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Meaning of availability for 3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The (UE generated) PMF reports are used by the UPF in ATSSS to understand whether a certain access of the MA PDU Session is available or unvailable. It is also described that the UE reports availability/unavailability of accesses based on implementation. While for a non-3GPP access the meaning of available is clear (since there is no concept of UE in CM-idle mode), for a 3GPP acces it is unclear if it is available only when the UE is in CM-connected mode or also when the UE is in CM-idle mode. This could lead to interoperability issues and/or unnecessary signaling (due to paging) in scenarios in which the UE is in CM-idle mode over the 3GPP access but still reports that the 3GPP access is available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that for a 3GPP access the PMF reports it is available if the UE is in CM-connected mode over that access, while it reports the access is unavailable if the UE is in CM-idle mode over that access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification which could lead to interoperability issues and unnecessary signaling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 4.22.5	Reporting of Access Availability

After the MA PDU session is established, if Reporting of Access Availability is required by network, the UE performs detection of the unavailability and availability of an access as described in TS 23.501 [2], clause 5.32.5.3. To report the availability/unavailability of the access, the UE sends the PMF-Access Report to the UPF via the user plane of any available access of the MA PDU session. The UPF shall use this report to decide which access can be used to deliver the downlink packets.

# >>>>END OF CHANGES<<<<