3GPP TSG-SA WG2 Meeting #158	 S2-2308710

# Goteborg, SE, August 21 – 25, 2023															(rev of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0285 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment with stage 3 on Parameter Provisioning of MBS Assistance Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Stage 3 CR C4-232640 (TS29.503 CR1079) is agreed in CT4#116, which proposes to adopt external group ID as the data key. With the above, clarification is also needed in Table 7.2.9a-1: Table 7.2.9a-1: MBS Session Assistance information Parameters; Description. MBS Session Assistance information; A list of UEs represented by GPSIs that are preferred to be kept connected when the MBS Session represented by the indicated MBS session ID the UE joined is active.. |
| Summary of change: |  | Clarify that MBS Assistant Information applies to a list of UEs with a multicast group (represented by External group ID) and a MBS Session ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with TS23.502 and stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.9a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 7.2.9a	AF provisioning MBS Session assistance information

After the AF obtains the MBS Session ID of a multicast MBS Session as specified in clauses 7.1.1.2 and 7.1.1.3, the AF may provision the MBS Session assistance information for the UE(s) via NEF to UDM using the procedure of external parameter provisioning as specified clause 4.15.6.2 of TS 23.502 [6] with the following enhancements:

-	the AF may provision MBS Session Assistance Information, which indicates that a UE is preferred to be kept connected when the related MBS Session the UE joined is active.

Table 7.2.9a-1: MBS Session Assistance information

| Parameters | Description |
| --- | --- |
| MBS Session Assistance information | A list of UEs, represented by GPSIs within a multicast MBS group (represented by External Group ID), that are preferred to be kept connected when the MBS Session represented by the indicated MBS session ID the UE joined is active. |
|  | MBS Session ID |

NOTE 1:	MBS Session Assistance information has no effect if the function of UE receiving MBS multicast in RRC_INACTIVE state is not used. The AF does not provision the MBS Session assistance information if the AF knows that the PLMN does not support the function of UE receiving MBS multicast in RRC_INACTIVE state.

NOTE 2:	The AF can provide the MBS Session assistance information for the UEs based on observed or expected UE data transmission behaviour, e.g., a frequent talker and/or the group leader of a multicast group represented by an MBS Session ID.

NOTE 3:	If the MBS Assistance Information is already available when a UE joins a multicast MBS Session, the SMF provides the information to the NG-RAN in the UE join procedure as specified in clause 7.2.1.3. If the MBS Assistance Information becomes available in the UDM, the UDM performs procedure of Session Management Subscriber Data Update Notification to SMF as specified in clause 4.5.2 of TS 23.502 [6] per subscription data change notification requested by the SMF.

If the AF is authorised by the UDM to provision the UE list MBS Assistance information, the UDM resolves the GPSI to SUPI, and requests to create, update or delete the provisioned UE list MBS Assistance information as part of the MBS subscription data for each SUPI.

* * End of Changes * * *