SA WG2 Meeting #158	S2-2308817

Goteborg, Sweden, August 21 – 25, 2023	(revision of S2-2300xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4338 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration of ECS address in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the LS in of S2-2308344, the PLMN ID is associated with the ECS Address Configuration Information that provided by AF. But the PLMN ID as optional input is missing in the Section 5.2.6.37. Also, the ECS address can be distributed to either any UE or a group of UE as indicated in Table 4.15.6.3d-1. This should also be reflected in the Section 5.2.6.37. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN ID as optional input of ECS Address Configuration Information. Add external group ID as optional input of ECS Address Configuration Information, and add a clarification when no external group ID is provided, the ECS Address Configuration Information should be applied to any UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ECS Address Configuration Information can not be associated with PLMN ID, and the granularity of ECS Address Configuration Information is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.37.1, 5.2.6.37.2, 5.2.6.37.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

5.2.6.37.1	General

Service description: The ECS Address service in the V-NEF provides functions for ECS Address Configuration Information provision for a group of UE or any UE from AF and for subscribing and retrieving ECS Address Configuration Information and providing ECS Address Configuration Information to the V-SMF to support. For related procedures see clauses 6.5.2.6.2 and 6.5.2.6.3 of TS 23.548 [74].

This procedure is defined only for the support of HR-SBO, i.e. where the ECS Address is not associated with the subscription of an user of the PLMN operating the V-NEF and V-SMF.

If External Group Identifier are not provided, then the ECS Address Configuration Information shall be correspond to any UEs of the PLMN of the NEF using the service identified by the ECS Address service.

NOTE:	There is thus no conflict with the usage of External Parameter Provisioning procedure to configure ECS Address Configuration Information for users of the PLMN operating the NEF and SMF.

* * * Start of Change 2 * * *

5.2.6.37.2	Nnef_ECSAddress_Create service operation

Service operation name: Nnef_ECSAddress_Create

Description: The NF consumer requests to create ECS Address Configuration Information.

Inputs, Required: ECS Address Configuration Information.

Inputs, Optional: AF-service-identifier, PLMN IDs, External Group Identifier.

Outputs, Required: Result Indication.

Outputs, Optional: None.

* * * Start of Change 3 * * *

5.2.6.37.3	Nnef_ECSAddress_Update service operation

Service operation name: Nnef_ECSAddress_Update

Description: The NF consumer requests to update ECS Address Configuration Information.

Inputs, Required: ECS Address Configuration Information.

Inputs, Optional: AF-service-identifier, External Group Identifier, PLMN IDs.

Outputs, Required: Result Indication.

Outputs, Optional: None.

* * * End of Change * * *