SA WG2 Meeting #158	S2-2308820

Goteborg, Sweden, August 21 – 25, 2023	(revision of S2-2300xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0171 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DNS server reselection based on common DNAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Section 6.2.3.2.4, it only considers the PDU session establishment procedure, but if there has already one PDU session established, and the PCC rule including common DNAI is provided to SMF, the SMF should reselect the local DNS server according to the common DNAI. And the SMF should configure the UE with new local DNS server with PCO in PDU session modification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the PDU session has already been established, and the PCC rule including updated common DNAI is provided to SMF, the SMF should reselect the local DNS server according to the common DNAI. And the SMF should configure the UE with new local DNS server with PCO in PDU session modification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to the common DNAI to impact current PDU session is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.2.3.2.4	Select common DNAI with Local DNS Server/Resolver for a set of UEs

The following procedure is for selecting common DNAI with Local DNS Server/Resolver for set of UEs.

Figure 6.2.3.2.4-1: Discovery Procedure for selecting the common DNAI for a set of UEs with Local DNS Server/Resolver

1.	The UE sends a PDU Session establishment request to SMF as specified in step 0 of Figure6.2.3.2.3-1.

2.	The procedure in steps 1-4 of Figure 6.2.3.2.3-1 applies with following difference:

In step 1, the SMF determines UE belongs to a set of UEs accessing an EAS corresponds to a common DNAI based on traffic correlation indication and Traffic correlation ID in the PCC rule.

If the common DNAI is not available, SMF may determine the common DNAI based on the UE location information and selects corresponding PSA UPF serves the common DNAI, and determines DNS server(s) to ensure a set of UEs uses the common DNAI.

Once the common DNAI is available, SMF selects local DNS server corresponding to the common DNAI.

If the PDU session has been established and the PCC rule including updated common DNAI is provided to SMF, the SMF should reselect the local DNS server according to the common DNAI. And if the local DNS server is reselected by the SMF, the SMF should configure the UE with new local DNS server in PCO during PDU session modification procedure.

3.	The EAS information (e.g. EAS IP address) is resolved by Local DNS Server and sent the UE as described in steps 5-6 of Figure 6.2.3.2.3-1.

* * * End of Change * * *