3GPP TSG-WG SA2 Meeting #158	S2-2308966

# Goteborg, Sweden, August 21 – 25, 2023		(was_S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0408 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NWDAF-assisted LCS services section |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Section 5.16 includes 3 aspects: - Aspect 1: location services assisted by NWDAF - Aspect 2: NWDAF services assisted by LCS - Aspect 3: LCS continuity during UE mobility It is obvious that the above 3 aspects are different and should be seperately viewed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separate Aspect 1: Location service assisted by NWDAF Separate Aspect 2: NWDAF service assisted by eLCS Separate Aspect 3: LCS continuity during UE mobility as a separate clause Enhance the description of Aspect 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unlear specification will lead people confused about how NWDAF assists LCS continuity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16, 5.x, 5.y, 5.y.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

## 5.16	Location services assisted by NWDAF

LMF and AMF may utilize analytics from NWDAF to assist with location services as described in clause 6.21.

NWDAF may provide Location Accuracy analytics as specified in clause 6.17 of TS 23.288 [37] to LMF. When LMF receives analytics for Location Accuracy from the NWDAF, it may use the analytics to determine indoor or outdoor for a location estimate, to select or adjust appropriate positioning methods for the requested location accuracy.

NWDAF may provide UE related mobility analytics as specified in clause 6.7 of TS 23.288 [37] to AMF. The UE location information received in analytics can be used by AMF to perform the UE location verification for NR satellite access.

## 5.x	NWDAF services assisted by eLCS

NWDAF may interact with the LCS system to request location information for a target UE or a group of UEs via Ngmlc services or NL9.

NWDAF may request an aggregated report for a single UE from GMLC to include multiple UE location estimates for a period of time. NWDAF may also provide a reporting time to indicate the latest time for reporting.

NOTE:	The value of the reporting time is left for NWDAF implementation. The reporting time should be earlier than the parameter 'time when analytics information is needed' (if applicable) defined in TS 23.288 [37].

### 5.y	LCS Continuity During UE Mobility

LCS session continuity during UE mobility applies to:

-	deferred triggered/periodic MT-LR, immediate MT-LR and MO-LR procedures.

-	uplink, downlink or uplink and downlink positioning methods.

### 5.y.1	Mobility Between 5GS and EPS

LCS continuity of UE in RRC-CONNECTED state enables transfer of a location session between 5GS and EPS. The LCS QoS may then be mapped from the source RAT to the target RAT.

The Lr' interface between the EPC-GMLC and 5GC-GMLC is implementation specific - or the EPC-GMLC and 5GC-GMLC may be co-located.

# * * * * End of change* * * *