SA WG2 Meeting #S2-158	S2-2309316

21 - 25 August 2023, Gothenborg, Sweden	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4895 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for Enabling Access to Localized Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To avoid any confusions, the term Subscribed SNPN used in the Annex N.2.1, should either be replaced with the term Subscribed SNPN acting as Credential Holder (as in Annex N.6) or with a more general term Credentials Holder, which also includes the cases with AAA server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subscribed SNPN is replaced with the term Credentials Holder at 2 instances and some wording modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# N.2	Enabling access to localized services

## N.2.1	General

To enable a PNI-NPN or SNPN to provide access to localized services, the PNI-NPN or SNPN operator configures the network with information enabling the UEs to access the localized services using the PNI-NPN or SNPN according to any validity of the localized services, and the information is determined in agreement with the localized service provider e.g.:

a.	Identification of each localized service, e.g. to be used in URSP rules.

b.	validity restriction for each localized service, e.g. the validity of time and/or location.

c.	service parameters for each localized service, e.g. DNN, S-NSSAI and QoS requirements.

d.	service authorization methods, e.g. NSSAA or Secondary authentication/authorization during PDU Session establishment.

To allow the UE to access the PNI-NPN or SNPN using the credentials of the HPLMN or the Credentials Holder, the PNI-NPN or SNPN can be configured, based on localized service agreements between the PNI-NPN or SNPN and the HPLMN or Credentials Holder, to allow primary authentication towards a HPLMN, when a PNI-NPN or SNPN is providing access to the localized services, and to allow primary authentication towards Credentials Holder, when an SNPN is providing access to the localized services.

To allow the UE to access the SNPN providing access to localized services when new credential is required, the SNPN can provide UE onboarding function as specified in clause 5.30.2.10 for the UE to obtain credential and necessary information to access the SNPN, or the UE can leverage existing credential and network connection to get access to a PVS via User Plane to obtain new credential.

To allow the UE to access the PNI-NPN providing access to the localized services where NSSAA or secondary authentication/authorization during PDU session establishment is required, the UE can obtain new credential using remote provisioning functionality as defined in clause 5.39.

To allow the UE to access the HPLMN or subscribed SNPN services while being registered in the PNI-NPN or SNPN, the PNI-NPN or SNPN can establish service agreements and configure inter-connect with the HPLMN or subscribed SNPN operator. If a PNI-NPN is providing access to the localized services, the existing roaming architecture with home-routed PDU Sessions are used. If an SNPN is providing access to the localized services, then the UE can access HPLMN or subscribed SNPN as described in Annex D, clauses D.3, D.6 and D.7.

END OF CHANGES