3GPP TSG SA WG2 Meeting #158	S2-2308793

# Gothenburg, Aug 21 – 25, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4332 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R18 AIMLsys_KI4_23502 CR for clarification on expected UE behavior parameter provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA2#157 meeting, it was ageed that AF can provide NEF with QoS duration and QoS inactivity interval when it requests to set up an AF session with required QoS procedure. This info then should be forwarded by NEF to PCF with Npcf_PolicyAuthorization_Create_request. Thus the PCF API should have this info as optional inputs. With this background, this CR proposes to add two inputs (QoS duration and QoS inactivity interval) as optional inputs of Npcf_PolicyAuthorization_Create_request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add two inputs (QoS duration and QoS inactivity interval) as optional inputs of Npcf_PolicyAuthorization_Create_request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.2.5.3.2	Npcf_PolicyAuthorization_Create service operation

Service operation name: Npcf_PolicyAuthorization_Create

Description: Authorize the request and optionally determines and installs SM Policy Control Data according to the information provided by the NF Consumer or provides Port Management Information Container for ports on DS-TT or NW-TT, or User plane node Management Information Container.

Inputs, Required: UE (IP or MAC) address, identification of the application session context.

Inputs, Optional: GPSI(s) or SUPI(s) if available, Internal Group Identifier, DNN if available, S-NSSAI if available, Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description information as described in clause 6.1.3.6 of TS 23.503 [20], AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, emergency indicator, ASP Identifier, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation, EAS Correlation indication, Common EAS IP address, Traffic Correlation ID, FQDN(s) as described in clause 5.6.7 in 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], TSCTSF parameters provided by the TSCTSF to the PCF as described in clause 6.1.3.23a and clause 6.1.3.23b of TS 23.503 [20], QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), TSC Assistance Container, MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20], Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], SFC Identifier(s), Metadata, Periodicity as described clauses 6.1.3.22 and 6.3.1 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description as described in clause 5.37.5 of TS 23.501 [2], Notification Target Address for PMIC/UMIC UPF event, Correlation ID for PMIC/UMIC UPF event, QoS duration, QoS inactivity interval.

NOTE 1:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

NOTE 2:	A dedicated Notification Target Address for PMIC/UMIC UPF event and Correlation ID for PMIC/UMIC UPF event are provided by the event consumer over Npcf_PolicyAuthorization as the corresponding events are reported by the UPF and not by the PCF. Providing such information indicates that the consumer of the Npcf_PolicyAuthorization (TSN AF, TSCTSF) suppots PMIC/UMIC via Nupf event reporting.

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clauses 6.1.3.16 and clause 6.1.3.10 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

END OF CHANGES