3GPP TSG-WG SA2 Meeting #158	S2-2310880

# Goteborg, Sweden, August 21 – 25, 2023		(revision of S2-230xxxx)

# Title:	[DRAFT] LS on Support of device behind 5G-RG

# Release:	Rel-18

# Work Item:	5WWC_Ph3

Source:	[Huawei to be] SA2

To:	BBF, Cable Labs

Cc:

Contact Person:

Name:	Marco Spini

Tel. Number:

E-mail Address:	m DOT spini AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

In rel-18 specification TS 23.316 has defined new functionalities for 5G-RG and FN-RG. SA2 would like to point out some functionalities for which SA2 assumed that BBF and Cable labs will endorse the functionalities and, if needed, specifies the part under their responsibility.

Item 1) Support 5G NSWO for 3GPP UE accessing via 5G-RG

. The 5G NSWO, as defined below, is similar to the NSWO defined in 4G.

5G NSWO: The 5G NSWO is the capability provided by 5G system and by UE to enable the connection to a WLAN access network using 5GS credentials without registration to 5GS.

In order to support 5G NSWO the 5G-RG shall support the SWa' interface toward the NSWOF, but the support in 5G-RG is in the scope of BBF and/or Cable Labs specifications. From 3GPP point of view the impact in normative is to describe the architecture, and the support in 5G-RG and W-AGF, as defined in TS 23.316 clause.

Item 2) Support of AUN3 devices

We define the support for device that does not support NAS signalling, which is connected to 5GC via a RG and can be authenticated by 5GC over the RG. The specification is defined in TS 23.316 clauses 4.7.11 and 4.10c where we assumed that 5G-RG and W-AGF functionalities are extended as defined.

Item 3) Support of NAUN3 devices

We define the support for  device that does not support NAS signaling which is connected to 5GC via a RG and for which authentication with 5GC is not supported. The specification is defined in TS 23.316 clause 4.10b where we assumed that 5G-RG and W-AGF functionalities are extended as defined

Item 4) Support of MBR traffic limitation for AUN3 device

Placeholder: text to be added based on selected solution

2. Actions:

To BBF and CableLabs group.

ACTION: 	SA2 kindly asks to BBF and Cable Labs to take into account the attached specification in order to provide feedback and to consider for addition to specifications under your responsibility.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160		November 13 – 17, 2023			Chicago, USA

TSG-SA2 Meeting #161		October 09 – 13, 2023			Xiamen, CN