3GPP TSG-SA WG2 Meeting #160-Ad Hoc-e 	S2-2400173r01

E-meeting, Elbonia, January 22 - 29, 2024

# Source:	KDDI

# Title:	New WID on Network Controlled Network Slice Selection

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Network Controlled Network Slice Selection

Acronym:	TEI19_SliceSel

Unique identifier:	TBD

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X | AF |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 980017 | Stage 2 of Network Slicing Phase 3 (eNS_Ph3) | {optional free text} |

Dependency on non-3GPP (draft) specification:

3	Justification

Network Slicing was standardized since Rel-15. From operators’ perspective, it is required to improve the customer experience by providing the optimal network slice in accordance with the characteristics of the applications on the UE and/or the analysis around the user’s environment information e.g., congestion, radio coverage or radio quality. Thus, it is beneficial for operators to have an option to select the network slice triggered by a notification from the Application Function.

The URSP rules are defined and the UE can use them to determine the parameters of the PDU Session including the S-NSSAI when the UE requests a PDU Session for an application/service used by the end users. The URSP rules may be locally configured in the UE or provided from the network. The network can update the URSP rules in the UE per receiving Application guidance. Whether and how to support URSP rule is left to UE implementation. Thus, the operators are constrained in providing network slices that are most suitable for the applications and/or services.

In Rel-18 network slice replacement is defined to replace the S-NSSAI with Alternative S-NSSAI for PDU Sessions when the existing S-NSSAI becomes congested. However the Network Slice Replacement notification is not triggered from the Application Function.

This work item aims to enhance the network controlled network slice selection mechanism to achieve that the user can enjoy the application on their device via the network connection which is optimally tailored by the operator using the network slice.

4	Objective

The objective is to specify the following aspects to 5GC:

-	Define a new NEF API service allowing AF(s) to request the slice selection based on the application(s) preference.

-	Modify existing slice replacement procedure to enable the slice selection based on the request from AF(s).

No impact is expected for the following aspects:

-	UDM/UDR impacts in terms of slice subscription management, and

-	UE policy management

Note: This work will require 1 TU.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Architectural enhancements to support the stated objectives | TSG-SA#103 Mar 2024 |  |
| 23.502 | Architectural enhancements to support the stated objectives | TSG-SA#103 Mar 2024 |  |
| 23.503 | Architectural enhancements to support the stated objectives | TSG-SA#103 Mar 2024 |  |

6	Work item Rapporteur(s)

Saito, Koji, KDDI,kj-saitou_at_kddi_dot_com

7	Work item leadership

SA2

8	Aspects that involve other WGs

Security aspects covered by SA3.

OAM and charging aspects covered by SA5.

9	Supporting Individual Members

Supporting IM name

BT

China Mobile

CKH IoD UK

Deutsche Telekom

DISH Network

Ericsson

InterDigital

KDDI

Lenovo

MATRIXX Software

NEC

Nokia

Nokia Shanghai Bell

oppo

Oracle

Orange

Samsung

SHARP

Telefonica

TOYOTA MOTOR CORPORATION

Verizon?

Vodafone

ZTE