SA WG2 Meeting #S2-161	S2-2403322

26 February - 3 March, 2024, Athens, Greece	(revision of S2-2402672)

Title:	KI#8 New Sol Support Avatar communication

Source:	Huawei, HiSilicon

Document for:	Approval

Agenda Item:	19.2

Abstract: This paper proposes to add network-based solution for supporting avatar communication in FS_NG_RTC_ph2 TR.

# Introduction

This paper proposes solutions according to the WT#5 of the NG_RTC_Ph2 SID.

# 2. Text Proposal

It is proposed to capture the following changes vs. XX.

# * * * * First change* * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[6]	3GPP TS 26.114: "Media handling and interaction".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TR 26.813: "Study of Avatars in Real-Time Communication Services".

[9]	3GPP TS 22.156: "Mobile Metaverse Services".

[10]	3GPP TS 24.229: "IP Multimedia Call Control based on SIP and SDP; Stage 3".

[11]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem".

[12]	3GPP TS 29.175: "IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application Server (AS) Services Stage 3 (Release 18)".

[13]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[14]	IETF RFC 3725: "Best Current Practices for Third Party Call Control (3pcc) in the Session Initiation Protocol (SIP)".

[15]	3GPP TS 24.341: "Support of SMS over IP networks".

[16]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[17]	ATIS-1000074: "Signature-based Handling of Asserted information using Tokens (SHAKEN)".

[18]	IETF draft-ietf-stir-passport-rcd-26: "PASSporT Extension for Rich Call Data".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[19]	IETF draft-ietf-sipcore-callinfo-rcd-08: "SIP Call-Info Parameters for Rich Call Data".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[x]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

# * * * * Second change* * * *

# 3	Definition, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], in TS 23.501 [2] , in TS 23.228 [5] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1], in TS 23.501 [2] or in TS 23.228 [5].

Automatic Speech Recognition: Automatic Speech Recognition, also known as ASR, is the use of Machine Learning (ML) or Artificial Intelligence (AI) technology to process human speech into readable text.

Avatar: A digital representation specific to media that encodes facial (possibly body) position, motions and expressions of a person or some software generated entity.

NOTE 1:	This definition was taken from TS 22.156 [9].

IMS Avatar Communication: The communication between two or more users using Avatar representation associated with the user based on IMS architecture.

Gesture: A change in the pose that is considered significant, i.e. as a discriminated interaction with a mobile metaverse service.

NOTE 2:	This definition was taken from TS 22.156 [9].

Sign Language Recognition: Sign Language Recognition is a computer vision and natural language processing task that involves automatically recognizing and translating sign language gestures into written or spoken language.

Speech to Sign Language: Speech to Sign Language is to translate spoken speech into video sign language.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] or in TS 23.501 [2].

3PCC	3rd Party Call Control

API	Application Programming Interface

ASR	Automatic Speech Recognition

BAR	Base Avatar Repository

CAPIF	Common API Framework

DAC	Digital Asset Container

DC	Data Channel

DCSF	Data Channel Signalling Function

IMS	IP Multimedia Subsystem

IMSEF	IMS Exposure Function

MF	Media Function

NEF	Network Exposure Function

NF	Network Function

OIP	Originating Identification Presentation

OMA	Open Mobile Alliance

SLR	Sign Language Recognition

SSL 	Speech to Sign Language

TTS	Text To Speech

UUID	Universally Unique Identifier

# * * * * Third change* * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |
| 6 |  | X |  |  |  |  |  |  |
| 7 | X | X |  |  |  |  |  |  |
| 8 |  |  | X |  |  |  |  |  |
| 9 |  |  | X |  |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |
| 13 |  |  |  | X |  |  |  |  |
| 14 |  |  |  |  |  | X |  |  |
| 15 |  |  |  |  |  | X |  |  |
| 16 |  |  |  |  |  | X |  |  |
| 17 |  |  |  |  |  |  |  | X |
| X |  |  |  |  |  |  |  | X |

# * * * * Fourth change* * * *

## 6.X	Solution #X: Supporting network-based avatar communication by media capability invocation

### 6.X.1	Description

#### 6.X.1.1	General

This solution defines the solution for network-based avatar communication addressing the following aspects that defined in the KI#8:

-	Study how to enable transcoding between speech and gesture (or text) in an IMS Avatar communication.

-	Study how to enable UE based and network-based rendering in case of IMS Avatar communication.

#### 6.X.1.2	Architecture

Figure 6.x.1.2-1: Architecture to support network-based avatar communication

To support IMS avatar communication, the architecture is enhanced as follows:

Base Avatar Repository (BAR):

-	The Base Avatar Repository is used to store and retrieve the avatar models. BAR can be inside the PLMN, e.g., a new network function, or outside the PLMN, e.g., a webserver of the 3rd party provider.

Note1: The BAR can be co-located with DCAR or deployed standalone.

XR Application Server (XR AS):

-	The XR Application Server is responsible for service control related to avatar communication, including avatar communication session media control and so on.

MF/MRF:

-	Support media processing, e.g. ASR/TTS/SLR/SSL, under the service control of XR AS.

-	Support avatar animation and rendering, under the service control of XR AS.

NOTE 2:	The MF/MRF media processing capabilities need to be aligned further with FS_AVATAR in SA4.

#### 6.X.1.3	Reference points

The following reference point is added to support avatar communication service in IMS:

-	MC1:	Reference point between the DCSF and the MF/MRF.

The MC1 reference point is used by DCSF to instruct MF the specific media capabilities will be used in a avatar communication.

Editor’s Note: The description of MC1 is FFS. The service and operation of MF to support MC1 is FFS.

### 6.X.2	Procedures

#### 6.X.2.1	Network-based procedure for avatar communication

Figure 6.x.2.1-1: Procedures of network-based avatar communication

Figure 6.x.2.1-1 depicts a typical call flow procedure of network-based avatar communication. The main steps in the call flow are as follows:

0. IMS session and bootstrap data channel has been established, and UE#1 has downloaded and run the avatar communication application.

1. UE#1 sends a re-INVITE with an updated SDP request to the IMS network to establish application data channel.

2. IMS AS selects and notifies the DCSF about the session event and data channel establishment request, DCSF identifies the application id contained in the request and transfers the request to XR AS.

3. XR AS instructs DCSF to send a media instruction request to IMS AS, the request includes service type (avatar call), dc media specification and possibly audio/video media specification according to the avatar call service requirement.

4. IMS AS selects a MF/MRF instance that support avatar call related media transcoding and rendering according to the service type.

5. IMS AS requests to MF/MRF to allocate ADC media resource, audio/video media resources if required.

6. IMS AS notifies the media instruction response to DCSF. DCSF sends the response to XR AS.

7. The XR AS/DCSF replies to the notification received in step 2.

8. IMS AS sends the re-INVITE to remote network and UE#2 via S-CSCF, which includes ADC media information, audio/video media information in the SDP.

9. UE#2 and terminating network returns a 200 OK response with SDP answer for ADC and audio/video.

10. IMS AS notifies the session event to DCSF and updates the media resources on MF/MRF. DCSF notifies XR AS after receiving notification from IMS AS.

11. IMS AS sends 200 OK response to UE#1 via S-CSCF, the response includes the SDP answer for ADC and audio/video.

12. Application data channel(s) are established between UE#1 and MF/MRF, UE#2 and MF/MRF.

13-14. XR AS sends service media control request to MF/MRF through DCSF via DC4 and MC1, MR/MRF will perform transcoding and rendering functions based on this request.

Take Avatar rendering as an example, the main parameters required include:

Drvien method: driving method, such as facial landmark, audio, video.

AvatarType: Base avatar type, such as 2D, 3D.

Avatar URL: Avatar URL information obtained by XR AS from BAR, which Avatar to use is based on the instruction from the user through application data channel or configured in XR AS. This is service specific and will not be specified in SA2.

Editor's Note: The parameters in the service media control request can be transparent transmission for DCSF, which can be negotiated between the XR AS and MF/MRF, the above parameters are for reference only.

Editor’s Note: How XR AS instruct a specific session is FFS.

15. MF/MRF downloads avatar model from XR AS using the Avatar URL received from step 14.

16. UE#1 sends metadata through the established application data channel, and/or audio/video stream over RTP, to the MF/MRF.

17. UE#2 sends metadata through the established application data channel, and/or audio/video stream over RTP, to the MF/MRF.

18. MF/MRF performs transcoding such as ASR/TTS/SLR/SSL, etc., and MF/MRF renders the avatar based on the transcoding result or the metadata received from application data channel.

19-20. MF/MRF sends the avatar video stream and/or generated metadata (such as text) to UE#2, and sends the avatar video stream and/or generated metadata (such as text) to UE#1 if required.

### 6.X.3	Impacts on Existing Nodes and Functionality

XR AS impact:

-	Support the subscription of avatar communication service and session control for avatar communication service.

DCSF impact:

-	Support identification the service media control request and send to MF/MRF.

IMS AS impact:

-	Support selecting a MF/MRF instance for avatar call based on the service type.

MF/MRF impact:

-	Support AI-based media processing including ASR/TTS/SLR/SSL and so on, under the service control of XR AS.

-	Support avatar animation and rendering, under the service control of XR AS.

# * * * * End of changes * * * *