SA WG2 Meeting #162	S2-2404615

Changsha, China, April 15 – 19, 2024

Source: 	Samsung

Title: 	Architectural Assumptions and Requirements for DualSteer

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes Architectural Assumptions and Requirements for DualSteer.

1 Discussion

As discussed in SA2#160AHE and SA2#161, the final version should be the baseline in SA2#162.

2 Proposal

It is proposed to update TR 23.700-54 as follows.

# * * * * First change * * * *

4.1	Architectural Assumptions

4.1.1	Architectural Assumptions for DualSteer

-	The 5GC architecture as specified in TS 23.501 [3], TS 23.502 [4], and TS 23.503 [5] is regarded as the baseline for this study.

-	It is assumed that no coordination is required between the two 3GPP RAN the DualSteer Device may be connected to.

-	Each subscription (i.e. SUPI) is used to connect to only one of the 3GPP access networks at any given point in time.

-	No impacts are assumed for network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

-	Solutions are expected to demonstrate not to impact PLMNs that do not support DualSteer functionality.

-	Traffic policies are controlled by the home network operator.

-	For sessions subject to potential traffic switching of a DualSteer device's user data when across two 3GPP access networks of VPLMN(s) and HPLMN (including two VPLMNs or a VPLMN and the HPLMN), it is assumed that the anchoring of all data traffic subject to traffic switching is in the HPLMN.

-    For any particular service, at any given time, it is assumed that the DualSteer device transmits all traffic of that service using only a single 3GPP access network.

-	Splitting functionality is not supported for DualSteer in any scenario.

# * * * * Second change * * * *

4.2	Architectural Requirements

4.2.1	Architectural Requirements for DualSteer

Service Requirements for DualSteer are specified in clause 6.50 of TS 22.261 [2].

The following scenarios shall be supported:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

The following architectural requirements are applicable to this study:

-	Scenario 5) requires the subscriber to be a subscriber of the PNI-NPN. In this scenario the PNI-NPN is hosted by a HPLMN. For sessions subject to traffic steering and/or to traffic switching, this scenario requires data anchoring in the PNI-NPN.

-	The subscriber shall have two subscriptions/SUPIs from the same operator.

-	In case the two 3GPP access networks belong to different PLMNs, or a PLMN and PNI-NPN, a proper business/roaming agreement among network operators is required.

-	The 5GS shall be able to identify and link the two SUPIs that are part of the subscription used for DualSteer services.

-	The 5GS shall be able to differentiate between the two connections of the DualSteer device.

-	The 5GS shall be able to minimize impacts to CN and OAM.

-	The 5GS shall be able to support mechanisms to minimize service interruption when switching a DualSteer device's user data, for one or multiple services, between two 3GPP access networks.

# * * * * End of change * * * *