# 3GPP TSG-SA WG2 Meeting #S2-162	S2-2404648

Changsha, China, April 15  –  19, 2024	(Revision of S2-240xxxx)

Title:	[Draft] Reply LS on XRM and TSC QoS requirements

Response to:	LS <S2-2403882/C3-241716> on XRM and TSC QoS requirements from CT3

Release:	Rel-18

Work Item:	     XRM

Source:	SA2

To:	CT3

Cc:

Contact Person:	Youngkyo Baek

E-mail Address:	youngkyo (dot) baek (at) samsung (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

SA2 thanks CT3 for the LS on XRM and TSC QoS requirements. SA2 has discussed the questions and provides answers below as follows:

##### 5.2.6.9.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create

Description: The consumer requests the network to provide a specific QoS for an AF session for a UE or a list of UEs.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address and only applicable for a single UE AF session), a list of UE addresses (as described in clause 4.15.6.13, and only applicable for a Multi-member AF session). Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

NOTE 1:	In this Release, when a list of UE addresses is provided, the Flow description information is common for all UE addresses in the list. Further details are described in clause 4.15.6.13.2.

Inputs, Optional: Time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS Monitoring parameter(s) as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available.

Only applicable for a single UE AF session: flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity as described in clause 5.27.2 or clause 5.37.8.2 of TS 23.501 [2], Time domain, Survival Time, BAT Window or Capability for BAT adaptation, Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], Periodicity Range, RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description (as described in clause 5.37.5 of TS 23.501 [2]), Indication of ECN marking for L4S as described in clause 6.1.3.22 of TS 23.503 [20], QoS duration, QoS inactivity interval, Multi-Modal Service ID together with Multi-modal Service Requirements information for each data flow as described in clause 6.1.3.27.3 of TS 23.503 [20].

Question 1: Besides the Periodicity and N6 Jitter Information, are there any other parameters included in TSCAI applicable to XR service? i.e., which are the TSCAC parameters as defined in TS 23.501 table 5.27.2-2 applicable to XR service?

SA2 reply: No. Other parameters than the Periodicity and N6 Jitter Information are not included.

Question 2: Can the bold parameters be included in one message which is used to request XR service and Time sensitive communication?

SA2 reply: Yes. There is no requirement for using one message for the above bold parameters.

2. Actions:

To CT3

ACTION: 	SA2 kindly asks CT3 to take the information above into account.

3. Date of Next SA2 Meeting:

SA2#163   		                            May 27 – 31, 2024			    Jeju, South Korea

SA2#164   	                               	  August 19 – 23, 2024		   Maastricht, Netherland