3GPP TSG-SA WG2 Meeting #162	S2-2404930

April 15 – April 19, 2024; Changsha, China

Title:	[Draft] Reply LS on Clarification related to the RAT type only change impact to AM/UE Policy association

Response to:	LS on Clarification related to the RAT type only change impact to AM/UE Policy association (S2-2403881/C3-241578)

Release:	Release 18

Work Item:	TEI18

Source:	Samsung [to be: SA2]

To:	CT3

Cc:	CT1

Contact Person:	Jicheol Lee 
jicheol.lee at samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks CT3 for the question.

Question 1: As per above requirements, there are multiple RAT types defined within one Access type, and there is possibility of UE mobility between RAT types (Inter RAT) within same Access. In which case, do we need the PCF to distinguish between different RAT types to provide AM/UE policies specific to RAT type?

SA2 has discussed and agreed to respond with the following answer.

SA2 Answer: Based on current TS 23 501, the AMF is able to differentiate different RAT types. Also, based on the current TS 23.502 clause 5.2.5.2.2 and 5.2.5.6.2 and TS 23.503 clause 6.2.1.2 for input policy, AMF can provide RAT type as an input for both AM and UE policy decisions.

SA2 acknowledged that PCRT (as in 6.1.2.5-1a) for AM policy only include Access Type change that does not include the change of RAT type.

Table 6.1.2.5-1a: Policy Control Request Triggers relevant for AMF and both 3GPP and Non 3GPP access type

| Policy Control Request Trigger | Description | Condition for reporting |
| --- | --- | --- |
| Access Type change | The Access Type has changed, added, or removed. | PCF (UE Policy Association) |
| UE-AMBR change | The subscribed UE-AMBR has changed. | PCF (AM Policy Association) |
| SMF selection management | UE request for an unsupported DNN or UE request for a DNN within the list of DNN candidates for replacement per S-NSSAI. | PCF (AM Policy Association) |
| Configured NSSAI change | The Configured NSSAI has changed. | PCF (UE Policy Association) |

However, SA2 haven’t identify the need of RAT-type change for PCRT for AM/UE policy decision.

2. Actions:

To 3GPP CT3

- Please take the above answer into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #163		27-31 May 2024		Jeju, Korea

TSG-SA2 Meeting #164		19-23 August 2024	Maastricht, Netherlands