3GPP TSG-SA2 Meeting # 162	S2-2405113

# Changsha, China,  15th  - 19th April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1395 | rev | 01 | Current version: | V18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Some correction and clarification to Annex AC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some correction and clarification to Annex AC are shown as follows: - Figures in Annex AC.2.1of 3GPP TS 23.228 miss Mb interface between UE and IMS-AGW. - Annex AC.7.6 of 3GPP TS 23.228 only mentions the scenario that the UE trigger the SDP re-negotiation to release the application data channel. But another scenario is also possible that the network trigger the SDP re-negotiation to release the application data channel, especially in P2A/A2P application case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Mb interface between UE and IMS-AGW into Figure AC.2.1-1 and Figure AC.2.1-2. - Add the scecario that the network releases the application data channel. - Some typos are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.2.1 and AC.7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AC.2.1	Architecture

Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

Figure AC.2.1-1: Architecture option of IMS supporting DC usage with MF

Figure AC.2.1-2 shows the data channel architecture when using the MRF.

Figure AC.2.1-2: Architecture option of IMS supporting DC usage with MRF

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this Release.

NOTE 5:	Enhancements to the N33 reference point for support of data channel services are not specified in this Release.

# * * * * Second change * * * *

## AC.7.6	Termination of IMS Data Channel

When a bootstrap data channel is established successfully, the bootstrap data channel shall be kept available during the IMS MMTel session, and it shall be terminated along with the release of the call.

The application data channel shall be terminated along with the release of the call or be terminated by closing this application separately. In the latter case, the UE or the network triggers an SDP re-negotiation to release the application data channel.

The termination of application data channel shall not have any impact on the audio or video within the same IMS session.

# * * * * End of changes * * * *