SA WG2 Meeting #162	S2-2405204

April 15 - 19, 2024 Changsha, China	(revision of S2-2404850)

Source:	Ericsson

Title:	New solution	KI#1-3: UID SUPI association

Document for:	Agreement

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: proposes ways to associate a UID with SUPI.

# 1		Proposal

It is proposed to agree on the following solution and to include it in TR.

************ 1st change (all new) ************

## 6.X	Solution #XX: UID <-> SUPI association

### 6.X.1	Key Issue mapping

KI#1-3

### 6.X.2	Description

#### 6.x.2.1	general

In 5GC, everything related to a UE/User is identified using a SUPI. If SUPI is not used to identify the current user, but some other identity, there are incalculable consequences to all existing features. By re-using SUPI all existing features of 5GC will work.  For example, event exposure related to a UE, charging, subscription information, IMS registrations (USIM method), Lawful intercept, etc.

It is there for advantageous to still use SUPI within the 3GPP system, i.e. to map the UID to SUPI. By this all procedures defined can be used also when the user uses UID for accessing the 3GPP network.

This solution defines 2 methods how this can be achieved.

#### 6.x.2.2	Method 1, sort of virtual SUPI

The main principle of this method is that the UID is linked to a SUPI (sort of a virtual SUPI). Thus, more or less existing data structures for subscriptions in UDM can be re-used.

UID server.

Subscription1 (UDM)

Subscription2 (UDM)

Subscr. data:

- MSISN

-…

Subscr. data:

- MSISN

-…

Subscr. data:

- MSISN

-…

UID subscription:

UID<->SUPI4

SUPI1 (SIM):

creditials

SUPI3 (SIM):

creditials

SUPI4

Figure 6.X.2.2-1: subscription association

To start with, the user needs to get a UID. This can be e.g. via an operator portal where the user administrates his/her account. When the UID is created, it is linked to a SUPI (SUPI4 in figure 6.X.2-1). If the user already has an account, this now will be treated as the user has multiple SUPIs for his/her subscription in 5GC.

When the user (A) wants to use another user's (B) UE, the user B needs to allow the use of his/her UE, which is a local UE matter how this is done. When allowed, the UE connects to the UID server via IP or NAS. .The proposal is to use secondary authentication. When UE is connected to the UID server the user A provides his/her credentials for the UID. If authentication is successful, UID server influences 5GC so that user A's UID associated SUPI (SUPI4) is associated with the SUPI of user B, and it also provides the UE with user A's SUPI (SUPI4), Then the UE de-registers SUPI of user B (SUPI1) and starts initial registration of User A using the SUPI of user A (SUPI4) but with credential of the SIM related to SUPI of user B (SUPI1)..

From now on, the user A is registered in 5GC, and all peculiars of user A can be handled as of existing 5GC procedures, e.g. slice selection, establishing of PDU session for browsing, PDU session for IMS and IMS registration, etc.

When user A logs out from user B's UE, the UE deregisters user A's SUPI and flushes it. The UE then registers with the user B's SUPI.

Exposure of the UID, can be done via existing procedures if user A's SUPI associated to the UID is mapped to a GPSI in 5GC.

NOTE: any security concerns needs to be handled by SA3.

#### 6.x.2.3	Method 2: eSIM

User Subscription (UDM)

UID server

The main principle of this method is that the UID is linked to a SUPI, and the SUPI is linked to an eSIM. Thus existing data structures for subscriptions in UDM can be re-used.

Subscription data:
- MSISDN

Subscription data:
- MSISDN

UID subscription:
UID <->eSIM

SUPI1 (SIM):

credentials

SUPI2 (eSIM):

credentials

Figure 6.X.2.3-1: eSIM structure

The solution uses the fact that multiple USIMs can exist in a UE, while only one need to be active at a time, i.e. the eSIM associated to the UID will be provisioned to any other UE that the user starts to use, except for the one with the regular USIM.

To start with, the user needs to get a UID. This can be e.g. via an operator portal where the user administrates his/her account. When the UID is created, it is linked to a SUPI and eSIM, and UDM/UDR will be provisioned via the backend system (OAM). If the user already has an account, this now will be treated as the user has multiple SIMs for his/her subscription in 5GC. GSMA has methods described see GSMA SGP.22 v3.1 [xx]. However, this is sort of static, here a dynamic method would be needed as there may be many users of a ME.

When the user (A) wants to use another user's (B) UE, the user B needs to allow the use of his/her UE, which is a local UE matter how this is done. When allowed, the UE connects to an authentication server via  NAS signalling. The proposal is to use secondary authentication. . When UE is connected to the authentication server user A provides his/her credentials for the UID. If authentication is successful, the eSIM associated with the UID is provisioned to the UE according to the principles of GSMA SGP.22 v3.1 [xx], and then the UE de-registers SUPI of user B and starts initial registration of User A using the SUPI and credential of the eSIM pushed to the UE.

From now on, the user A is registered in 5GC, and all peculiars of user A can be handled as of existing 5GC procedures,  e.g. slice selection, establishing of PDU session for browsing, PDU session for IMS and IMS registration, etc.

When user A logs out from user B's UE, the UE deregisters user A's SUPI and the downloaded eSIM can be purged. The UE then registers with the user B's SUPI.

Exposure of the UID, can be done via existing procedures if UID is mapped to GPSI in 5GC.

### 6.X.3	Procedures

#### 6.x.3.1	Method 1, sort of virtual SUPI

In the procedure below, user B is the owner of the UE, and user A is the user which will use a UID to login to 5GC. SUPI-B belongs to user B and SUPI-A belongs to user A. Before the user B can use his/her UID, the 5GC (UDR), and UID server needs to be populated with the data of user A (SUPI, subscription data) as described in clause 6.X.2.

UE

5GC

1. allow UID login

2. UID login

4. associate SUPIs

5. De-register (SUPI-B)

6. Register (SUPI-A)

Figure 6.x.3.1-1: Association of SUPIs

1.	The user B allows user A to use his/her UE.

Editor’s Note:	If this is a pure human-UE interaction or if the UE also interacts with the 5GC here is FFS.

2.	The user logs in. UDM is informed that new User is active

Editor’s Note:	the authentication and other security needs to be considered by SA3.

3.	At successful login, the UDM to associates SUPI-A with SUPI-B

4.	At successful login, the UDM pushes the SUPI-A via SMF to the UE. The SUPI-A could be provided in ePCO by SMF to the UE.

5.	UE deregisters from 5GC with SUPI-B

6.	UE registers to 5GC with SUPI-A using the credentials of SUPI-A

Editor’s Note:	the authentication and other security needs to be considered by SA3, including means to use SUPI-A credentials for the security procedures.

#### 6.x.3.2	Method 2: eSIM

In the procedure below, user B is the owner of the UE, and user A is the user which will use a UID to login to 5GC. SUPI-B belongs to user B and SUPI-A belongs to user A. Before the user B can use his/her UID, the 5GC (UDR), and UID server needs to be populated with the data of user A (SUPI, eSIM) as described in clause 6.X.2.

UID server,
other backend systems

UE

5GC

1. allow UID login

2. UID login

3. 5GC informs UID/backend system about new login

4. eSIM provisioning

5. De-register (SUPI1)

6. Register (SUPI2)

Figure 5.x.3.2-1: eSIM provisioning

1.	The user B allows user A to use his/her UE.

Editor’s Note:	If this is a pure human-UE interaction or if the UE also interacts with the 5GC here is FFS.

2.	The user logs in to his/her UID, which is authenticated via secondary authentication in 5GC and UDM/AAA server.

Editor’s Note:	the authentication and other security needs to be considered by SA3.

3.	At successful login the 5GC informs the the UID server that a the user has logged in.

4.	The UID server/backend system initiate provisioning of the eSIM associarted with the UID to the UE.. The provisioning follow the principles described in GSMA SGP.22 v3.1 [x].

Editor's Note: if the provisioning of the eSIM is via user plane or control plane is FFS

4.	UE deregisters from 5GC with SUPI-B.

5.	UE registers to 5GC with SUPI-A.

### 6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

UE:

-	support of allowing another user (than the owner) to access the ME

-	support connection to the authentication server

-	separation of data associated to different users

-	support download of SUPI of another user

SMF:

-	authenticate the UID based on secondary authentication, if NAS signalling is used

UDM for method 1:

-	support of associate a UID related SUPI with the SUPI of the UE

-	provide the credentials of the UID

UID server for method 1:

-	login of user (if IP is used)

-	influence 5GC for SUPI associations

UID server for method 2:

-	initiate eSIM provisioning due to information from 5GC

-	eSIM to UE.

************ 2nd change ************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.101: "Service aspects; Service principles".

[3]	3GPP TS 22.115: "Service aspects; Charging and billing".

[4]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[8]	3GPP TR 22.904: "Study on user centric identifiers and authentication (Release 16)".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[x]	GSMA SGP.22 v3.1: "Remote SIM Provisioning (RSP) Architecture for consumer Devices".

************ 3rd change ************

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> |
| #1 | X | X | X |  |
| #2 | X |  |  |  |
| #3 | X | X |  |  |
| #4 | X |  |  |  |
| #5 | X | X |  |  |
| #6 | X |  |  |  |
| #7 | X |  |  |  |
| #8 |  | X |  |  |
| #9 |  | X |  |  |
| #10 |  | X | X |  |
| #11 |  | X | X |  |
| #12 |  | X | X |  |
| #13 |  | X |  |  |
| #14 |  | X |  |  |
| #15 |  |  | X |  |
| #16 |  |  | X |  |
| #XX | X | X | X |  |

************ end changes ************