SA WG2 Meeting #162	S2-2405237

Changsha, China, April 15 –19, 2024	(revision of S2-2404076)

Source:	LG Electronics

Title:	Architectural Assumptions for NTZ

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes some Architectural Assumptions for NTZ.

Discussion

RAN provided Reply LS on clarification on requirements for NTZ (S2-2403897/RP-240810) and ETSI TC MSG/TFES provided Reply LS on “on No-Transmit Zones according to ECC decision 22(07)” (S2-2403920)

Based on these reply LSs, some Architectural Assumptions and Requirements for NTZ are proposed.

Proposal

It is proposed to agree the following changes into TR 23.700-59.

* * * * Start of 1st Change * * * *

4.1	Architectural Assumptions

The following architectural assumptions apply:

-	For solutions to enable network-assisted/ground-based mechanism for DAA (Detect And Avoid),

-	co-existence with and leveraging, to the extent possible, Direct DAA solutions specified in TS 23.256 [2] shall be considered.

-	sensing related information is out of scope of this study.

-	Regarding C2 communication reliability aspects in KI#1, only C2 over the Uu interface is considered in this study, and UAV using multiple-PLMN connectivity to support C2 communication reliability will not be considered in this release.

The following assumptions apply to the support of NTZ:

-	NTZ may be defined according to regional/national requirements;

-	an NTZ may map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE:	NTZ scope and mapping to cell(s) need to be coordinated with RAN WGs.

-	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

* * * * Start of Next Change * * * *

4.2	Architectural Requirements

-	The existing procedures specified in TS 23.256 [2] should be reused as much as possible for solutions.

-	Service exposure mechanisms defined in TS 23.256 [2] should be reused as much as possible.

The following architectural requirements apply to support NTZ:

-	No transmissions in the restricted frequency bands are allowed from an aerial UE located in the NTZ, regardless of the service type, according to local regulations.

-	Reception of downlink data from the network by the aerial UE located in the NTZ is allowed given the regulatory requirements are not violated.

* * * * Start of Next Change * * * *

5.3	Key Issue #3: Support of No Transmit Zones

5.3.1	Description

This key issue relates to the introduction by CEPT ECC Decision 22(07) [3] of No Transmit Zones for aerial UEs. The ECC Decision asserts that a mechanism is necessary to ensure that aerial UEs respect no-transmit zones in order to protect incumbent radio systems from potential interference from aerial UEs.

Since the ECC Decision does not identify any specific RAT, NTZs can be supported by both LTE and NR.

This key issue addresses the following aspects:

-	How to ensure an aerial UE respects no-transmit zones, including:

-	whether a mobile network cells overlapping completely or partially with the NTZ and using the restricted frequency bands of the NTZ;

-	whether mechanisms are needed to differentiate aerial UEs that support functions defined for NTZs in Rel. 19 and aerial UEs that don't;

-	what if any, specific aerial UE behaviour when the aerial UE approaches, enters, or exits the NTZ.

-	Whether and how to enable configuration of NTZ information in the aerial UE.

-	Whether to allow the enforcement of no-transmit zone(s) for both aerial UEs in connected mode and aerial UEs in idle mode and if yes then how.

NOTE 1:	Any potential solutions developed shall be coordinated with RAN WGs or progressed together with RAN WGs input.

For aerial UEs supporting NTZ regulations and being aware of the presence of NTZs, it is assumed that they are provided with NTZ information (either by pre-configuration or obtaining the NTZ information through the 3GPP network).

NOTE 2:	How to ensure that aerial UEs not supporting NTZ do not violate NTZ restrictions needs to be addressed during the study and conclusion of KI#3.

* * * * End of Changes * * * *