3GPP TSG-WG SA2 Meeting #162	S2-2405300

# Changsha, China, April 15- April 19, 2024

Source: 	China Mobile

Title: 	Update on Solution #2: Edge computing handling by local SMF

Document for: 	Approval

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: Propose a update on Solution #2: Edge computing handling by local SMF.

1	Discussion

This paper is to propose update on Solution #2.

It is propose to solve the Editor’s notes on Solution #2 in the following ways:

Editor's note:	How the central SMF receives the indication to trigger the L-SMF selection is FFS

Central SMF may retrieve indication of L-SMF requirement from the Session Management Subscription data. It is proposed to add clarification and remove the Editor’s note.

Editor's note:	How to support the DNS message handling in this solution is FFS.

The L-SMF gets the EASDF address from SMF.

2	Proposal

It is proposed to include the below changes into TR 23.700-49 v0.2.0.

- FIRST CHANGE

6.2	Solution #2: Edge computing handling by local SMF

6.2.1	Key Issue mapping

This solution is related to the KI#1.

6.2.2	Functional Description

The architecture of this solution is described as Figure 6.2.2-1.

Figure 6.2.2-1: Network Architecture for the L-SMF

In this solution, UL CL/BP controlled by local SMF (L-SMF) can offload edge computing related traffic to local DN. The L-SMF is responsible for EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion, and central SMF is not involved for the above information.

During PDU Session Establishment procedure, SMF decides to trigger the local-SMF selection based on SM subscription data. The SMF sends the indication of L-SMF selection to AMF, as well as the selected PCF ID/address.

The tunnels between AN, ULCL/BP UPF, central PSA UPF are setup by the following: UL N3 tunnel is setup by AMF provides UL CL/BP UPF CN Tunnel Info received from L-SMF to (R)AN, DL N3 tunnel is set up by AMF provides AN Tunnel Info received from (R)AN to UL CL/BP, UL N9 tunnel between UL CL/BP UPF and central PSA UPF is setup by AMF provides PSA CN Tunnel Info received from SMF to L-SMF, DL N9 tunnel between UL CL/BP UPF and central PSA UPF is setup by AMF provides UL CL/BP UPF CN Tunnel Info received from L-SMF to SMF.

6.2.3	Procedures

Figure 6.2.3-1: Procedure of edge computing handling by local SMF

1.	Steps 1-11 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following change:

-	In step 4, the central SMF retrieves indication of L-SMF selection from the Session Management Subscription data.

-	In step 11, the central SMF response with the indication of L-SMF selection, selected PCF ID or PCF address, selected EASDF address to AMF.

2.	AMF selects the L-SMF with considering the UE location.

3.	AMF triggers another PDU session establishment and sends the PCF address or PCF ID, selected EASDF address to the L-SMF by invoking Nsmf_PDUSession_CreateSMContext Request service.

4.	L-SMF obtains offloading policy from PCF via SM Policy Association Establish procedure as described in clause 4.16.4 of TS 23.502 [3].

5.	L-SMF selects UL CL/BP and optionally L-PSA as described in clause 6.3.3 of TS 23.501 [2].

6.	Steps 9-14 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following changes:

-	SMF is replaced by L-SMF.

-	CN Tunnel Info is UL CL/BP CN Tunnel Info.

-	The L-SMF configures the selected EASDF with DNS message handling rules, as described in step 2 of clause 6.2.3.2.2 of TS 23.548 [5].

-	In this step, the (R)AN obtains the CN Tunnel Info of the UL CL/BP UPF.

7.	Steps 15-17 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following change:

-	AN Tunnel Info is replaced by UL CL/BP CN Tunnel Info.

-	SMF is replaced by L-SMF.

-	In step 15, AMF also sends PSA CN Tunnel Info to the L-SMF.

-	In step 16a, L-SMF also provides PSA CN Tunnel Info to the UL CL/BP.

6.2.4	Impacts on existing services, entities and interfaces

AMF:

-	L-SMF selection indicated by DNN of the PDU Session Establishment Request.

-	Providing the PCF address and EASDF address from Central SMF to L-SMF.

-	Providing the PSA CN Tunnel Info to L-SMF.

-	Providing the UL CL/BP CN Tunnel Info to SMF.

SMF:

-	Providing PCF address and EASDF address to AMF.

PCF:

-	Providing Offloading policy to L-SMF.

-	Support the communication with L-SMF.

L-SMF:

-	Support the communication with PCF.

- End of CHANGE