3GPP SA WG2 Meeting #162	S2-2405408

15 – 19 April, Changsha, China				(was_S2-2405193_S2-2404736_S2-2402344)

Source:	NEC, China Telecom

Title:	KI#4, New Solution: Number of the active non-3gpp devices restriction

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: The contribution proposes a new solution: Number of the active non-3gpp devices restriction for Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG.

1. Introduction

This contribution proposes a new solution to Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-32 v0.2.0.

* * * Start of 1st change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| X |  |  |  | X |
| … |  |  |  |  |

* * * End of 1st change * * * *

* * * Start of 2nd change * * * *

## 6.x	Solution #X: Number of the active non-3gpp devices restriction

### 6.x.1	Key Issue mapping

The solution applies to Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG.

### 6.x.2	Functional Description

Assumptions:

- It is assumed that a UE and the UE’s subscription may be used by multiple users. Such an UE is provided with an User Profile in the UDM linked to the UE’s subscription profile. One of the attributes of the User Profile is the max number of the active non-3gpp devices.

- The number of the sumultaniously active non-3gpp devices control is oiptional and does not apply to the legacy non-3gpp devices.

- It is assumed that multiple non-3gpp devices can connect via the UE however, the number of the simultaneously active non-3gpp devices identified with their non-3gpp device identity shall not exceed the max number of non-3gpp devices allowed in the User profile. The restriction shall be controlled by the AMF via a counter for the simultaneously active non-3gpp devices.

### 6.x.3	Procedures

Figure 6.X.3: Number of the active non-3gpp devices restriction

- The User Profile is provided to the UDM and linked to the UE subscription information. One of the attributes of the User Profile is the max number of the active non-3gpp devices which is optional and configurable by the operator.

- The UE registers with the network. During the subscription information retrieval from the UDM, the AMF also retrieves the User Profile which may contain the max number of the non-3gpp devices that can connect simultaneously with the network. The AMF stores the max number of the non-3gpp devices in the UE’s context in the AMF and maintains a counter for the current number of the active non-3gpp devices.

- At some point a non-3gpp device with non-3gpp device identity connected to the UE triggers a request for a service.

Editor’s Note: It is FFS whether the non-3gpp device identity requires authentication.

- When the UE triggers the PDU Session Establishment/Release procedure if a new PDU Session is required/released or the PDU Session Modification procedure if an existing PDU Session is to be modified, the UE includes the ‘non-3gpp device-x identity’ and ‘non-3gpp device connection status’ parameters in the NAS message to the AMF to indicate for a new non-3gpp device connection or disconection.

- The AMF controls the number of the simultaneously active non-3gpp devices indicated with their non-3gpp device identities by the UE, i.e. the legacy non-3gpp devices are not controlled for backword compatibility. The AMF increases the number of the simultaneously active non-3gpp devices when a new non-3gpp device is connecting for service and the AMF decreases the number of the simultaneously active non-3gpp devices when a non-3gpp device is disconnecting. When increasing the number of the simultaneously active non-3gpp devices the AMF checks whether it exceeds the max number of the non-3gpp devices defined in the User Profile. If exceeds, the AMF, based on operator policy, may reject the request for service from the UE.

- The AMF returns NAS Message reject in which the AMF may include a reject cause (e.g. max number of the active non-3gpp devices reached).

Note: It is for CT1 to decide how the uplink NAS message carrying SM message can be rejected.

### 6.x.4	Impacts on existing services, entities and interfaces

UDM:	User Profile handling.

AMF:	User Profile handling;

Number of the active non-3gpp devices with non-3gpp device identity counting and control.

New ‘max number of active non-3gpp devices reached’ reject cause.

UE:	New parameter to indicate a service request from a new non-3gpp device with non-3gpp device identity.

New ‘max number of active non-3gpp devices reached’ reject cause.

* * End of 2nd change * * * *